| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Valencia, ES, 27 March – 4 April 2014 | Document: JCTVC-Q0081 |
| --- | --- |

| Title: | RExt/SHVC HLS (AHG5 and 9): On reuse of alpha planes in auxiliary pictures |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Matteo Naccari Marta Mrak Centre House, Block D 56 Wood Lane, W12 7SB London | Tel: Email: | +44 (0)303 0409640 matteo.naccari@bbc.co.uk marta.mrak@bbc.co.uk |
| Source: | BBC R&D |  |  |

_____________________________

# Abstract

Alpha channel signals are supported in HEVC using auxiliary pictures and their associated information is conveyed using the SEI message proposed in JCTVC-P0123. The syntax supports several functionalities such as values for transparent and opaque samples, whether pre-multiplied alpha planes are received etc. Additionally also the case when not all primary pictures have associated an alpha plane should be addressed. Therefore, this contribution proposes to extend the syntax of the alpha_channel_info SEI message by adding one flag specifying the persistence of alpha planes within the coded video sequence.

# Introduction

Alpha channel signals are supported in HEVC using auxiliary pictures transmitted as additional layers along with the primary pictures as detailed in the current SHVC specification draft. At the previous JCT-VC meeting, contribution JCTVC-P0123 proposed one SEI message to signal the information associated to alpha planes and their related processing. Beside the support included in that message, it is interesting to address scenarios when the auxiliary layer is not present for each primary picture. Examples of these scenarios are shown in Figure 1 where the auxiliary layers have lower frame rate than their primary counterpart. In Figure 1(a) the frame rate for the alpha channel is half the primary one while in Figure 1(b) depicts a limit case where only one alpha plane is sent for the whole CVS. This latter example may appear in applications as logo or sign interpreter insertion where one frame of the alpha channel is associated to several frames in the primary picture (see for example Figure 2). Therefore, in this contribution the syntax specified in the alpha_channel_info SEI message is extended by inserting a flag to specify how to handle missing alpha planes. The syntax and semantics for the proposed extensions are presented in the next section.

| (a) | (b) |
| --- | --- |

Figure 1: Examples of different arrangements for primary and auxiliary pictures. (a) The auxiliary picture frame rate is half the primary one and (b) there is only one auxiliary picture for the whole CVS.

| (a) | (b) |
| --- | --- |

Figure 2: Example of the alpha channel required to insert two sign language interpreters in a news programme. (a) The primary picture for the news programme and (b) the associated alpha plane which may be fixed for several frames along the time.

# Proposed syntax and semantics to specify alpha plane persistency

This section presents the syntax and semantics changes required to specify the reuse of alpha planes in case the auxiliary layer has a frame rate different from the primary one. All changes and additions are highlighted in yellow.

## Syntax changes

In clause F.7.3.1.2 the variable currAuxPicLayerId is defined. This variable stores the nuh_layer_id of the alpha plane for one access unit in the current CVS.

F.7.3.1.2	NAL unit header syntax

The specifications in subclause 7.3.1.2 apply. The variable currAuxPicLayerId is defined as follows:

–	currAuxPicLayerId = nuh_layer_id

The syntax of the alpha_channel_info SEI message is modified as follows.

| alpha_channel_info( payloadSize ) { | Descriptor |
| --- | --- |
| alpha_channel_cancel_flag | u(1) |
| if( !alpha_channel_cancel_flag ) { |  |
| alpha_channel_use_idc | u(3) |
| alpha_channel_bit_depth_minus8 | u(3) |
| alpha_transparent_value | u(v) |
| alpha_opaque_value | u(v) |
| alpha_channel_incr_flag | u(1) |
| alpha_channel_clip_flag | u(1) |
| if( alpha_channel_clip_flag ) |  |
| alpha_channel_clip_type_flag | u(1) |
| reuse_preceding_alpha_channel_flag | u(1) |
| } |  |
| } |  |

## Semantics changes

...

reuse_preceding_alpha_channel_flag equal to 0 indicates that when the current access unit does not contain a picture with nuh_layer_id equal to currAuxPicLayerId and a previous access unit in the coded video sequence contained a picture with nuh_layer_id equal to currAuxPicLayerId and AuxId equal to AuxId[ AUX_ALPHA ], a picture is generated with nuh_layer_id set equal to currAuxPicLayerId and all samples set equal to alpha_transparent_value. reuse_preceding_alpha_channel_flag equal to 1 indicates that the auxiliary alpha output picture is reused in output order until one or more of the following conditions are true:

–	A new CVS begins

–	The bitstream ends

–	A picture in an access unit with nuh_layer_id equal to currAuxPicLayerId is output having PicOrderCntVal greater than PicOrderCnt( CurrPic ).

...

# Patent rights declaration(s)

British Broadcasting Corporation (BBC) may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).