| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 17th Meeting: Valencia, ES, 27 March – 4 April 2014 | Document: JCTVC-Q0083 |
| --- | --- |

# Syntax and semantics description

## Sequence parameter set RBSP syntax

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| sps_extension_present_flag | u(1) |
| if( sps_extension_present_flag ) { |  |
| for( i = 0; i < 1; i++ ) |  |
| sps_extension_flag[ i ] | u(1) |
| sps_extension_7bits | u(7) |
| if( sps_extension_flag[ 0 ] ) { |  |
| transform_skip_rotation_enabled_flag | u(1) |
| transform_skip_context_enabled_flag | u(1) |
| intra_block_copy_enabled_flag | u(1) |
| implicit_rdpcm_enabled_flag | u(1) |
| explicit_rdpcm_enabled_flag | u(1) |
| extended_precision_processing_flag | u(1) |
| intra_smoothing_disabled_flag | u(1) |
| high_precision_offsets_enabled_flag | u(1) |
| fast_rice_adaptation_enabled_flag | u(1) |
| cabac_bypass_alignment_enabled_flag | u(1) |
| intra_screen_coding_enabled_flag | u(1) |
| } |  |
| if( sps_extension_7bits ) |  |
| while( more_rbsp_data( ) ) |  |
| sps_extension_data_flag | u(1) |
| } |  |
| rbsp_trailing_bits( ) |  |
| } |  |

intra_screen_coding_enabled_flag equal to 1 specifies that intra screen coding may be used for intra blocks. intra_screen_coding_enabled_flag equal to 0 specifies that intra screen coding is not applied. When not present, the value of intra_screen_coding_enabled_flag is inferred to be equal to 0.

## Coding unit syntax

| coding_unit( x0, y0, log2CbSize ) { | Descriptor |
| --- | --- |
| if( transquant_bypass_enabled_flag ) |  |
| cu_transquant_bypass_flag | ae(v) |
| if( slice_type != I ) |  |
| cu_skip_flag[ x0 ][ y0 ] | ae(v) |
| nCbS = ( 1 << log2CbSize ) |  |
| if( cu_skip_flag[ x0 ][ y0 ] ) |  |
| prediction_unit( x0, y0, nCbS, nCbS ) |  |
| else { |  |
| if( intra_block_copy_enabled_flag ) |  |
| intra_bc_flag[ x0 ][ y0 ] | ae(v) |
| if( !intra_bc_flag[ x0 ][ y0 ] && intra_screen_coding_enabled_flag ) |  |
| intra_scm_flag[ x0 ][ y0 ] | ae(v) |
| if( intra_scm_flag[ x0 ][ y0 ] ) { |  |
| scm_info_coding(x0, y0, nCbS, nCbS) |  |
| } else { // if(!intra_scm_flag) |  |
| if( slice_type != I && !intra_bc_flag[ x0 ][ y0 ]) |  |
| pred_mode_flag | ae(v) |
| if( CuPredMode[ x0 ][ y0 ] != MODE_INTRA ¦ ¦ intra_bc_flag[ x0 ][ y0 ] ¦ ¦ log2CbSize = = MinCbLog2SizeY ) |  |
| part_mode | ae(v) |
| if( CuPredMode[ x0 ][ y0 ] = = MODE_INTRA ) { |  |
| if( PartMode = = PART_2Nx2N && pcm_enabled_flag && !intra_bc_flag[ x0 ][ y0 ] && log2CbSize >= Log2MinIpcmCbSizeY && log2CbSize <= Log2MaxIpcmCbSizeY ) |  |
| pcm_flag[ x0 ][ y0 ] | ae(v) |
| if( pcm_flag[ x0 ][ y0 ] ) { |  |
| while( !byte_aligned( ) ) |  |
| pcm_alignment_zero_bit | f(1) |
| pcm_sample( x0, y0, log2CbSize ) |  |
| } else if( intra_bc_flag[ x0 ][ y0 ] ) { |  |
| mvd_coding( x0, y0, 2) |  |
| if( PartMode = = PART_2NxN ) |  |
| mvd_coding( x0, y0 + ( nCbS / 2 ), 2) |  |
| else if( PartMode = = PART_Nx2N ) |  |
| mvd_coding( x0 + ( nCbS / 2 ), y0, 2) |  |
| else if( PartMode = = PART_NxN ) { |  |
| mvd_coding( x0 + ( nCbS / 2 ), y0, 2) |  |
| mvd_coding( x0, y0 + ( nCbS / 2 ), 2) |  |
| mvd_coding( x0 + ( nCbS / 2 ), y0 + ( nCbS / 2 ), 2) |  |
| } |  |
| } else { // INTRA syntax |  |
| … |  |
| } |  |
| } else { // ! MODE_INTRA |  |
| … |  |
| } |  |
| if( !pcm_flag[ x0 ][ y0 ]) { |  |
| if( CuPredMode[ x0 ][ y0 ] != MODE_INTRA && !( PartMode = = PART_2Nx2N && merge_flag[ x0 ][ y0 ] ) ¦ ¦ CuPredMode[ x0 ][ y0 ] = = MODE_INTRA && intra_bc_flag[ x0 ][ y0 ] ) |  |
| rqt_root_cbf | ae(v) |
| … |  |
| } |  |
| } |  |
| } // if(!intra_scm_flag) |  |
| } |  |
| } |  |

intra_scm_flag[ x0 ][ y0 ] equal to 1 specifies that the current coding unit is coded in screen coding mode. intra_scm_flag[ x0 ][ y0 ] equal to 0 specifies that the current coding unit is not coded in screen coding mode. When intra_scm_flag[ x0 ][ y0 ] is not present, it is inferred to be equal to 0. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered coding block relative to the top-left luma sample of the picture.

## Palette syntax (scm_info_coding)

| scm_info_coding(x0, y0, CbWidth, CbHeight) { |  |
| --- | --- |
| triplet_index_rotation_flag[ x0 ][ y0 ] | ae(v) |
| palette_share_flag[ x0 ][ y0 ] | ae(v) |
| If(ChromaArrayType != 0) { |  |
| triplet_share_flag[ x0 ][ y0 ] | ae(v) |
| } |  |
| if(palette_share_flag[ x0 ][ y0 ]){ |  |
| for(cIdx = 0; cIdx < ( ChromaArrayType == 0 )?1:3; cIdx++) { |  |
| major_color_num[ x0 ][ y0 ][ cIdx ]= major_color_num_pred[ cIdx ] |  |
| for( n = 0; n < major_color_num[ cIdx ]; n++ ) { |  |
| major_color_value[ x0 ][ y0 ][ cIdx ][ n ]= major_color_value_pred[ cIdx ][ n ] |  |
| } |  |
| } |  |
| } else { |  |
| for(cIdx = 0; cIdx < ( ChromaArrayType == 0 )?1:3; cIdx++) { |  |
| major_color_num[ x0 ][ y0 ][ cIdx ] | ae(v) |
| for( n = 0; n < major_color_num[ cIdx ]; n++ ) { |  |
| major_color_pred_flag[ x0 ][ y0 ][ cIdx ][ n ] | ae(v) |
| if( major_color_pred_flag[ x0 ][ y0 ][ cIdx ][ n ] ) { |  |
| major_color_value[ x0 ][ y0 ][ cIdx ][ n ]= major_color_value_pred[ cIdx ][ n ] |  |
| } else { |  |
| major_color_value[ x0 ][ y0 ][ cIdx ][ n ] | ae(v) |
| major_color_value_pred[ cIdx ][ n ]= major_color_value[ x0 ][ y0 ][ cIdx ][ n ] |  |
| } |  |
| } |  |
| } |  |
| } |  |
| If(ChromaArrayType != 0) { |  |
| if(triplet_share_flag[ x0 ][ y0 ]){ |  |
| triplet_color_num[ x0 ][ y0 ]= triplet_color_num_pred |  |
| for( n = 0; n < triplet_color_num[ x0 ][ y0 ]; n++ ) { |  |
| for(cIdx = ( ChromaArrayType == 3 )?0:1; cIdx < 3; cIdx++) { |  |
| triplet_color[ x0 ][ y0 ][ cIdx ][ n ]= triplet_color_pred[ cIdx ][ n ] |  |
| } |  |
| } |  |
| } else { |  |
| triplet_color_num[ x0 ][ y0 ] | ae(v) |
| for( n = 0; n < triplet_color_num[ x0 ][ y0 ]; n++ ) { |  |
| triplet_color_pred_flag[ x0 ][ y0 ][ n ] | ae(v) |
| for(cIdx = 0; cIdx < ( ChromaArrayType == 0 )?1:3; cIdx++){ |  |
| if(triplet_color_pred_flag[ x0 ][ y0 ][ n ]) { |  |
| triplet_color_value[ x0 ][ y0 ][ cIdx ][ n ]= triplet_color_value_pred[ cIdx ][ n ] |  |
| } else { |  |
| color_index[ x0 ][ y0 ][ cIdx ] | ae(v) |
| triplet_color_value[ x0 ][ y0 ][ cIdx ][ n ] = color_index[ x0 ][ y0 ][ cIdx] |  |
| triplet_color_value_pred[ cIdx ][ n ]= triplet_color_value[ x0 ][ y0 ][ cIdx ][ n ] |  |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |
| If(ChromaArrayType != 3) { |  |
| all_ver_line_mode_flag | ae(v) |
| If(all_ver_line_mode_flag == 0){ |  |
| cIdx = 0; |  |
| for( I = 0; I < CbHeight; I ++) { |  |
| line_mode[ x0 ][ y0 ][ cIdx ][i] | ae(v) |
| if(line_mode[ x0 ][ y0 ][ cIdx ][ I ] == HOR_MODE) { |  |
| color_index[ x0 ][ y0 ][ cIdx + cSubIdx ][ I ][ 0 ] | ae(v) |
| } else if(line_mode[ x0 ][ y0 ][ cIdx ][i] == VER_MODE) { |  |
| for(j = 0; j < CbWidth; j++) { |  |
| for(cSubIdx = 0; cSubIdx < ( ChromaArrayType == 3 )?3:1; cSubIdx++ ) { |  |
| color_index[ x0 ][ y0 ][ cIdx + cSubIdx ][ I ][ j ] = color_index[ x0 ][ y0 ][ cIdx + cSubIdx ][ i-1 ][ j ] |  |
| } |  |
| } |  |
| } else { //NOR_MODE |  |
| for(j = 0; j < CbWidth; j++) { |  |
| index_pred_symbol[ x0 ][ y0 ][ cIdx ][ I ][ j ] | ae(v) |
| } |  |
| for(j = 0; j < CbWidth; j++) { |  |
| if(Indx_pred_symbol[ x0 ][ y0 ][ cIdx ][ I ][ j ] == NO_PREDICTION) { |  |
| color_index[ x0 ][ y0 ][ cIdx ][ I ][ j ] | ae(v) |
| } |  |
| } |  |
| } |  |
| for(j = 0; j < CbWidth; j++) { |  |
| if(color_index[ x0 ][ y0 ][ cIdx ][i][j] == major_color_num[ x0 ][ y0 ][ cIdx ]) { |  |
| quant_escape_value[ cIdx ][ I ][ j ] | ae(v) |
| } |  |
| } |  |
| } |  |
| } |  |
| If(ChromaArrayType != 0) { |  |
| all_ver_line_mode_flag | ae(v) |
| If(all_ver_line_mode_flag == 0){ |  |
| for(cIdx = ( ChromaArrayType == 3 )?0:1; cIdx < 1; cIdx++) { |  |
| for( i = 0; i < CbHeight; i ++) { |  |
| line_mode[ x0 ][ y0 ][ cIdx ][i] | ae(v) |
| if(line_mode[ x0 ][ y0 ][ cIdx ][ i ] == HOR_MODE) { |  |
| triplet_index[ x0 ][ y0 ][ cIdx][ i ][ 0 ] | ae(v) |
| } else if(line_mode[ x0 ][ y0 ][ cIdx ][i] == VER_MODE) { |  |
| for(j = 0; j < CbWidth; j++) { |  |
| triplet_index[ x0 ][ y0 ][ cIdx][ i ][ j ] = triplet_index[ x0 ][ y0 ][ cIdx][ i-1 ][ j ] |  |
| } |  |
| } else { //NOR_MODE |  |
| for(j = 0; j < CbWidth; j++) { |  |
| index_pred_symbol[ x0 ][ y0 ][ cIdx ][ i ][ j ] | ae(v) |
| } |  |
| for(j = 0; j < CbWidth; j++) { |  |
| if(Indx_pred_symbol[ x0 ][ y0 ][ cIdx ][ i ][ j ] == NO_PREDICTION) { |  |
| triplet_index[ x0 ][ y0 ][ cIdx ][ i ][ j ] | ae(v) |
| } |  |
| } |  |
| } |  |
| for(j = 0; j < CbWidth; j++) { |  |
| for(cSubIdx = 0; cSubIdx < ( ChromaArrayType == 3 )?3:2; cSubIdx++ ) { |  |
| color_index[ x0 ][ y0 ][ cIdx + cSubIdx ][i][j]= triplet_color_value[ x0 ][ y0 ][ cIdx + cSubIdx] [ triplet_index[ x0 ][ y0 ][ cIdx+ cSubIdx ][ i ][ j ] ] |  |
| if(color_index[ x0 ][ y0 ][ cIdx + cSubIdx ][i][j] == major_color_num[ x0 ][ y0 ][ cIdx + cSubIdx ]) { |  |
| quant_escape_value[ cIdx + cSubIdx ][ i ][ j ] | ae(v) |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |

triplet_index_rotation_flag[ x0 ][ y0 ] equal to 1 specifies that, in triplet index coding, the horizontal and vertical axis of sample position is switched.

palette_share_flag[ x0 ][ y0 ] equal to 1 specifies that major_color_num and major_color_value is direved from major_color_num_pred and major_color_value_pred directly.

triplet_share_flag[ x0 ][ y0 ] equal to 1 specifies that triplet_color_num and triplet_color_value is direved from triplet _color_num_pred and triplet _color_value_pred directly.

major_color_num[ x0 ][ y0 ][ cIdx ] specifies the number of major colors for color component cIdx. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered coding block relative to the top-left luma sample of the picture.

major_color_pred_flag[ x0 ][ y0 ][ cIdx ][ n ] equal to 1 specifies n-th major color value for color component cIdx could be predicted from the predictor. major_color_pred_flag[ x0 ][ y0 ][ cIdx ][ n ] equal to 0 specifies current major color couldn’t be predicted. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered coding block relative to the top-left luma sample of the picture.

major_color_value[ x0 ][ y0 ][ cIdx ][ n ] specifies the n-th major color value for color component cIdx. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered coding block relative to the top-left luma sample of the picture.

triplet_color_num[ x0 ][ y0 ] specifies the number of triplet colors. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered coding block relative to the top-left luma sample of the picture.

triplet_color_pred_flag[ x0 ][ y0 ][ n ] equal to 1 specifies n-th triplet color value could be predicted from the predictor. triplet_color_pred_flag[ x0 ][ y0 ][ n ] equal to 0 specifies current major color couldn’t be predicted. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered coding block relative to the top-left luma sample of the picture.

triplet_color_value[ x0 ][ y0 ][ cIdx ][ n ] specifies the n-th triplet color value for color component cIdx. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered coding block relative to the top-left luma sample of the picture.

all_ver_line_mode_flag equal to 1 specifies that all line modes in the CU are inferred to be VER_MODE. The color_index or triplet_index in the CU are inferred as 0.

line_mode[ x0 ][ y0 ][ i ] equal to HOR_MODE specifies that the line should be decoded in horizontal line mode. line_mode[ x0 ][ y0 ][i] equal to VER_MODE specifies that the line should be decoded in vertical line mode. line_mode[ x0 ][ y0 ][ i ] equal to NOR_MODE specifies that the line should be decoded in normal line mode. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered coding block relative to the top-left luma sample of the picture.

index_pred_symbol[ x0 ][ y0 ][ i ][ j ] specifies the prediction symbol of the index for the current sample in position ( i, j ). index_pred_symbol[ x0 ][ y0 ] [ i ][ j ] equal to NEW_INDEX specifies that index value of current sample cannot be derived from the coded sample.

quant_escape_value[ x0 ][ y0 ][ cIdx ][ i ][ j ] specifies the quantized escape sample value for color component cIdx in at the location ( i, j ). The variable bitEscapeQuant is derived from a pre-defined look-up table indexed for each QP value

- escape_value[ x0 ][ y0 ][ cIdx ][ i ][ j ] is set equal to

quant_escape_value[ x0 ][ y0 ][ cIdx ][ i ][ j ]<<(bitDepth – bitEscapeQuant)

color_index[ x0 ][ y0 ][ cIdx ][ i ][ j ] specifies the index value of the sample at the location( i, j ) for color component cIdx. The index values of predictors are skipped. The decoded color_index needs to be compared with the index values of predictors to reconstruct the final color index.

triplet_index[ x0 ][ y0 ][ cIdx ][ i ][ j ] specifies the triplet index value of the sample at the location( i, j ) for color component cIdx. The index values of predictors are skipped. The decoded triplet_index needs to be compared with the index values of predictors to reconstruct the final triplet index.