| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Valencia, ES, 27 March – 4 April 2014 | Document: JCTVC-Q0084r1 |
| --- | --- |

| Title: | VUI entries for YXZ and digital cinema EOTF |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chad Fogg, Jim Helman 530 Lytton Ave, Suite 300 Palo Alto, CA 94301 USA | Email: | chadfogg@gmail.com jhelman@movielabs.com |
| Source: | MovieLabs Inc. |  |  |

_____________________________

# Abstract

This contribution suggests editorial changes to address feedback received since the previous meeting in San Jose (January 2014) on the XYZ color description information in HEVC Annex E (Video Usability Information, a.k.a. VUI).  In the matrix_coeffs semantics, it is proposed that the YZX mapping from real values to integer follow convention for YCbCr and GBR signals for both video_full_range_flag=0 and video_full_range_flag=1. A transfer_characteristics code point for digital cinema EOTF (with gamma 2.6) is also requested.

# Suggested proposals

A code point, after the PQ code point [1], to signal the transfer function required by the digital cinema distribution master image characteristics (SMPTE 428-1) since no existing transfer_characteristic (Table E-4) entry expresses that formula.

HEVC and AVC have traditionally defined real to integer mapping (“integer quantization”) as part of the matrix_coeffs semantics, even if no actual matrix derivation is performed from a primaries (such as the GBR planar mode).  While the real-to-integer mapping could theoretically be a separate dimension from matrix_coeffs (in addition to the current three:  color primaries, transfer function aka. EOTF, and matrix coefficients), the element video_full_range_flag can distinguish the full range practice of digital cinema and several authoring formats separately from the traditional consumer distribution range indicated by the 216 and 224 range scale factors shared by BT.601, BT.709, 240M and BT.2020 (matrix_coeffs=1,4,5,6,7,9 or 10).

A suggested delta document (JCTVC-P1005_v1_Annex_E_changes_for_Q0084_r1.docx) for Annex E is attached to the .zip archive of this proposal.

# References

- S. Miller, M. Nezamabadi, S. Daly, “Perceptual Signal Coding for More Efficient Usage of Bit Codes”, SMPTE Journal, Vol. 122, No. 4, May-June 2013.

# Patent rights declaration(s)

MovieLabs does not have any current or pending patent rights relating to the technology described in this contribution.