| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Valencia, ES, 27 March – 4 April 2014 | Document: JCTVC-Q0085r1 |
| --- | --- |

| Title: | High Dynamic Range / Wide Color Gamut workflow |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC (AHG-18) |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Bill Mandel 100 Universal City Plaza Universal City, CA 91608 USA Chad Fogg, Jim Helman 530 Lytton Ave, Suite 300 Palo Alto, CA 94301 USA | Email: | bill.mandel@nbcuni.com chad.fogg@gmail.com jhelman@movielabs.com |
| Source: | Universal Pictures, MovieLabs |  |  |

_____________________________

# Abstract

This contribution describes the High Dynamic Range / Wide Color Gamut (HDR/WCG) workflow that utilizes the ACES process created by AMPAS to generate .yuv input for HEVC Main12 encoding, as reported in JCTVC-Q0087, JCTVC-Q0088 and JCTVC-Q0228.  The video container aspects of the encoding are intended to be conveyed in the video characteristics information of VUI (HEVC Annex E) , such as YCbCr BT.2020 and XYZ primary color space derivations, in combination with PQ Electro-Optic Transfer Functions (EOTF). This contribution attaches source code for some of the transformations, including the Y’DzDx and YCbCr color differencing and 4:2:0 resampling formulae used in the simulations.

# Introduction

The basic workflow process consist of:  (1) format conversion from source file to “.yuv” HEVC encoder input (2) signalling of metadata into HEVC VUI  (3) encoding HEVC with HDR/WCG-friendly parameters.

Nominally, the format conversion process herein consists of a series of mappings, ranging from camera-specific “scene referred” video container to a “display referred” format as described in [1,2]. Not all steps are needed for every target .yuv being created by this workflow. The total number of steps and their order ultimately depends upon the format in which sample clips are delivered, or, for example, how “RAW” material was developed into sample clip files.

The ODT – “Output Device Transform” step is where the non-linear EOTF (aka “PQ”) is applied. The RRT – “Reference Rendering Transform” from AMPAS is applied when the source material is in a scene referred state that, without the RRT, would have its sample levels otherwise undesirably clipped.  If the source clip is already display-referred, the RRT step can be skipped.

The author chose to use this particular tool chain to bring the dynamic range into some reasonable span and to control the LAD point (e.g. to around 5-10 nits display referred). This process can be considered as a workflow where “RAW” files are “developed” by first applying an IDT (Input Device Transform) that places samples into ICES (Input Color Encoding Specification) space, then finally develops encode-ready samples through an RRT+ODT process into non-linear displayed-referred .yuv source format suitable for HM encoding. The AMPAS HDR develop tools such as ACES have greatly helped in establishing an understanding how HDR material might be processed in an actual HDR/WCG studio workflow by serving as a reasonable model of the basic stages involved in such processing. See Chapter 21 in and page 314 of Chapter 24 of Giorgianni [1] for a walkthrough of the AMPAS system. Other chapters provide the core philosophy followed by ACES and this document.

IDT

RRT

ODT

.yuv

HM

(view & analyze)

OCES

ICES

A general guideline for using this process described in this document is: when un-ranged “RAW” or close-to-RAW material is used, it is important to first devise a method to simulate the IDT (Input Device Transform) to first map the material into ACES primaries (ICES) space, since the RRT operates only in that domain and outputs OCES (Output Color Encoding Specification) for use by the ODT stage.  Some specific examples applied in the related studies listed in the abstract of this document have been:

- Movielabs/DCI/Disney-Telescope provided content: (“DCI StEM”, “Disney-Telescope”) is delivered as if an ODT process has already been run.  The source file is already in non-linear,display-referred container mapped to the “PQ” EOTF as published in the May 2013 SMPTE journal [2]. The supplied 16-bit PQ RGB TIFF files are video-ranged from code levels 4096 to 60160.

- Technicolor HDR sequences (Market, Seine, Balloon, Seine, Tibul, FireEater) were provided as 16-bit half-floating point display-referred .exr files. Consequently integer settings such as “full range” (video_full_range_flag=1) or “video range” (video_full_range_flag=0) do not apply to those source files. This material did not appear to be “developed” in the traditional post-production sense, other than being in the familiar 709 color space. Further examination showed that this material was wider ranging in luminance than may be expected. The author subjectively chose to arbitrarily perform an “inverse transform” to ICES Scene-referred format by dividing the EXR values by a 179 scale factor (of the target Sim2 projector) and then matrixing that output into ACES primaries. Then, the AMPAS RRT was applied to bring the samples into “proper” range, followed by a simple ODT process where the PQ EOTF was then applied for this encoding study. See files “scaleR179.ctl” , “709-2-OCES.ctl”, and “odt_PQ10k.ctl” in the source zip file of this submission that execute these series of remapping steps.

- A variety of techniques in the spirit of the above stages have been used to view HDR content in this study on an HDR monitor. The techniques generally consist of a  series of .yuv back to PQ RGB TIFF conversion steps that take into account color space (XYZ or BT.2020 YCbCr), followed by additional matrix conversions into a TIFF form that the research display could show (e.g. like the Dolby Pulsar). 

Here is an example ctrlrender operation to create a properly ranged TIFF for Pulsar viewing from a PQ 2020 source:

ctlrender -force  -ctl INVPQ10k2020-2-XYZ.ctl -ctl XYZPQ2PulsarP3.ctl -ctl PQ10kRL-12B.ctl PQ2020Frame.tiff -format tiff16 Pulsar/PQP3_PulsarRange.tiff"

- The following steps can be utilized for viewing HDR content on a PC or, better yet, a bright screen that can be found on some mobile phone models:  inverse map PQ TIFF into linear range, then scale, such that when this linear signal is put through the AMPAS ODT tone curve that is traditionally used for 48 nit and 100 nit normal ODTs, samples at, for example, 500 nits are treated with functional maps normally applied to 100 nits and therefore placed through the top knee of that ODT curve rather than being clipped from above range.  Thus, 500 nit standard type compatible images and can be viewed on a monitor in its bright or “vivid” mode to forge a somewhat subjective opportunity to view HDR results on an SDR+ monitor that is capable of  rendering moderately greater than 100 nits brightness, such as 200 to 500 nits available on some mobile phone and desktop computer monitors. 8 or 10 bit/sample TIFS, JPG, PNG, and .yuv’s can easily be created from source or decoded HDR content for this purpose.. 

For example, starting with RGB-style (XYZ or 2020 primaries) non-linear PQ integer images that are in the TIFF container immediately converted from the TAppDeccoder .yuv output, the following steps can be used to make such SDR+ viewing files: 1.) inverse the PQ ODT used to convert from integer tiff samples into floating point precision (e.g. either INVPQ10k-2-XYZ.ctl or INVPQ10k2020-2-XYZ.ctl), 2.) then map into OCES domain with ACES primaries (e..g XYZ2ACES.ctl) and finally 3.) output via a full range “500nit” modified 709 100 nit AMPAS ODT and write TIFF files. From there, it is a straightforward conversion with ImageMagick to convert those .tiff’s  into jpg, png, etc. As an alternative to ImageMagick, the author’s tiff2ydzdx.cpp tool can convert the tiff’s into a 10bit .yuv file (e.g. by looping over all frames tiff2ydzdx $filename HD1920 B10 709 FULL) for encoding a file that can be viewed with Vooya YUV player to encoded with x264 etc. into a .mp4 that can be played on a mobile phone’s media player. The author has found that a player called “mpv” has good performance with such generated .yuv files. Here is an example command line for playing a “500 nit”.yuv on a desktop or laptop PC:

“mpv --speed 0.25 --loop 100 --demuxer=rawvideo --demuxer-rawvideo=w=1920:h=1080:fps=23.976:mp-format=yuv420p10le --colormatrix BT.709 --colormatrix-input-range=full --colormatrix-output-range=full --screenshot-format=png --screenshot-png-compression=0 500n.yuv”

The following tools described in sectios 1.1. through 1.7 are necessary and/or useful for the workflow recipes that have been summarized in the introductory text above.

## OpenEXR

Readers may find that their operating system running the experiments outlined in the abstract may already include OpenEXR version 1.7 or higher and can skip this OpenEXR install step altogether. Note that if the local copy of OpenEXR is updated from an earlier install of OpenEXR, some tools such as ImageMagick and image viewers that depend upon an earlier version of OpenEXR may need to be recompiled to utilize a newer edition of OpenEXR ---  otherwise such programs that depend upon OpenEXR may fail to open EXR files after such an OpenEXR update.

https://github.com/openexr/openexr/releases

If it is decided to install OpenEXR, then after unpacking the source build, it is recommended to test and install IlmBase, then build, test and install OpenEXR. Run the test as mentioned, but note that panic is unwarranted should some tests fail due to computation precision issue that was a known when the author last encountered this known issue to OpenEXR developers during a recent update procedure.

## ACES

Use the development branch of ctl. All that is needed for this tool to build is ctlrender tool so that .ctl scripts can be run on input files utilized in this workflow project. Appreciation is extended to AMPAS for making this project available.

https://github.com/ampas/CTL/releases

Follow the build instruction on their website:	https://github.com/ampas/CTL

The ACES scripts needed in the RRT and ODT steps are available on github. V0.7.1 was tested for this project. These .ctl scripts are useful to automatically manipulate wide dynamic / gamut range files through a standard process, if no traditional manual color grading or output conversion step (with tools such as BaseLight, Davinci Resolve, or RV64) for reference monitor viewing is available.

https://github.com/ampas/aces-dev/releases

Documentation for ctlrender is included with the CTL release itself, or may be found here:

https://github.com/ampas/CTL/blob/master/doc/CtlManual.pdf

## Tiff conversion tools

The following tools (copyright author of this document) are provided to convert display referred images into the .yuv container that the HM HEVC reference encoder can process. Though these tools were originally created to support 12-bit 1080p XYZ processing (hence the name), BT.2020, BT.709 content, HD and UHD resolutions, and 10, 12 or 14 bit image handling are also supported. LIBTIFF is required to compile.

### Tiff to YUV container (“tiff2ydzdx”)

This is the primary tool to convert an array of ODT TIFF files to a .yuv planar file of concatenated images that HEVC reference encoder HM expects as input.

This tool primarily converts the integer 16-bit 4:4:4 samples in the .tiff container into 12-bit 4:2:0 format for Main10 and Main12. The 4:2:0 <-> 4:4:4 chroma resampling stage is accomplished with a modified TM-5 (MPEG-2 reference encoder) 8-tap FIR near the final step of .yuv output.  An alternative workflow [7] would be to resample chroma in linear, floating point domain of ACES prior to the application of the OETF (PQ, BT.1886 gamma 2.2, etc.).

For historical reasons, the output file is always called “YDzDx.yuv” no matter what command-line color space target options are invoked. The tool is run once for each frame and appends the processed output frame to the YDzDx.yuv output file in the current directory. It is therefore important to make sure no pre-existingYDzDx.yuv file is present in the target output directory, since tiff2ydzdx will appended any processed frames to that file.

To compile:

g++ -O3 tiff2ydzdx.cpp -o tiff2ydzdx -ltiff

Use:

tiff2ydzdx [tiff file] [options]

Command line options: BOX, FULL, 709, 2020, HD1920, HD960, B10, B14, Y100, Y500

- Color formula (default is Y'DzDx) use Y500, 709 or 2020 respectively for others

- Bit Depth (default is 12 bits) use B10 for 10 bits

- Resolution (default is 3840x2160) options are HD1920 or HD960. Resolution is achieved by center cutting if the image is bigger than the resolution.

- (e.g. tiff2ydzdx Frame.tif 2020 B10 would subsample and append Frame.tif using a 2020 formula and 10 bits onto YDzDx.yuv)

Here is is an example .sh loop to make a Y500 (subsampled XYZ) .yuv file:

rm -v YDzDx.yuv

for filename in $EDRDATA/EXR/TIFs/* ; do

$EDRHOME/Tools/tiff2ydzdx/tiff2ydzdx $filename HD1920 Y500

done

### Yuv container back to TIFF (“ydzdx2tiff”)

This tool converts, for example, the decoded .yuv output of HEVC back to individual TIFF files for quality analysis.

To compile:

g++ -O3 ydzdx2tiff.cpp -o ydzdx2tiff –ltiff

Use:

ydzdx2tiff  [tiff file] [options]

Command line options: BOX, FULL, 709, 2020, HD1920, HD960, Y100, Y500, B10, B14

- Color formula (default is Y'DzDx) use Y500, 709 or 2020 respectively for others

- Bit Depth (default is 12 bits) use B10 for 10 bits

Resolution (default is 3840x2160) options are HD1920 or HD960. 
(e.g. ydzdx2tiff Frame.tif 2020 B10 would subsample and append Frame.tif using a 2020 formula and 10 bits onto YDzDx.yuv)

- Starting frame “-f” and number of frames to write out “-f” (e.g. –fs 0 –f 25)

Here is an example to fill a directory with TIFF files. This tool assumes that the output directory “tifXYZ” exists before it is run.

ydzdx2tiff dec.yuv HD1920 2020

## Included tools and scripts

The following source files are attached to the .zip archive (/src subdirectory) of this document.

Tools:

| tiff2ydzdx.cpp | Use in a loop to frame by frame build up a .yuv file. This code implements the Y’DzDx color differencing as optimized for a D65 white point. |
| --- | --- |
| ydzx2tiff.cpp | Use on a concatenated .yuv file to de-concatenate convert individual frames into a folder of TIFFs |

ctlrender scripts:

| INVPQ10k-2-XYZ.ctl | Removes PQ gamma for XYZ content. Used when testing XYZ content to transform back to XYZ (not quite OCES) for application of further transforms to convert material for Pulsar viewing or the 500nit jpg pseudo viewing. |
| --- | --- |
| INVPQ10k2020-2-XYZ.ctl | Removes PQ gamma for Rec. 2020 content. Used to transform back to XYZ (not quite OCES) for application of further transforms to convert material for Pulsar viewing or the 500nit jpg pseudo viewing. |
| PQ.ctl | Contains forward and reverse PQ EOTF equations. |
| XYZ2ACES.ctl: | Used to multiply XYZ content into ACES primaries. Used typically after the INV*ctl scripts above to put the content into OCES domain for further transforms. |
| encodeHD.sh: | Example script used to encode RD data set. Includes encoding and decoding command lines described in this document. |
| odt_PQ10k.ctl | Transforms OCES domain imagery into XYZ, applies PQ EOTF and ranges video to 16 bit video legal range. |
| odt_PQ10k2020.ctl | Transforms OCES domain imagery into Rec 2020, applies PQ EOTF and ranges video to 16 bit video legal range |
| odt_rec709_full_500nits.ctl | Modified AMPAS 709 Full Range ODT. Transforms OCES domain imagery into Rec 709, scales 500 nits down to 100 nits applies 1886 EOTF. Used to generate “500 nit” TIFFs for pseudo viewing on bright computer and mobile screens. |
| scale10k.ctl | Scale factor of 10,000 used to range some 0-1 material into OCES properly. Could have been included in other scripts but author ended up with this separate. |
| XYZPQ2PulsarP3.ctl | Transforms linear XYZ imagery into P3 primaries used for critical viewing on Pulsar (Dolby prototype monitor) |
| RL12B.ctl: | Scales full range video into nearly full range for Pulsar (16-4076) and keeps at MSB of 16 bit TIFF |

:

Example process scripts (users should modify these scripts for broader use)

| PQ.sh | used to process StEM clips into PQ |
| --- | --- |
| yuv.sh | used to make some .yuv files from PQ TIFFs |
| encodeHD.sh | used to run RD curve set See instead: encodeHD2020.sh and encodeHDY500.sh which were used with StEM RD curve generation. |
| 2020-2-709-500.sh | used to make 500nit jpgs See instead: “OutputConversion” scripts used to process StEM RD curve sequences. |
| OutputConversionXYZ.sh | used to make Pulsar TIFFs for critical viewing, 500 nit JPGs and MP4 for pseudo viewing Used with decoded PQ XYZ TIFFs. |
| OutputConversion2020.sh | used to make Pulsar TIFFs for critical viewing, 500 nit JPGs and MP4 for pseudo viewing Used with decoded PQ Rec. 2020 TIFFs |

## HEVC reference encoder

(via JCT) HM-13.0+RExt-6.0 was used for this project in the generation of Main12 bitstreams, which despite being the official RExt toolkit, is also the most up to date generator of HEVC version 1 (Main, Main10) bitstreams, verified by the HM software coordinators.

svn co https://hevc.hhi.fraunhofer.de/svn/svn_HEVCSoftware/tags/HM-13.0+RExt-6.0/

cd ./HM-13.0+RExt-6.0/build/linux

make all_highbitdepth

## Other tools

LIBTIFF:  needed to compile the tiff2yuv containers.

ImageMagick, GNU Plot

mpv player: https://github.com/mpv-player/mpv

## Steps for converting scene referred source to HEVC HM encoder input

The following serial procedures convert the .exr scene referred files into the BT.2020 PQ-EOTF container format passed as a .yuv planar input file into the HEVC HM reference encoder. (see files PQ.sh, technicolor.sh, yuv.sh, encodeHD*.sh and OutputConversion*.sh for some guidelines)

### Step 1: control dynamic range via RRT

The necessity of this step depends on the source material used for testing. This step has been helpful when working with wide range content like RAW files that have been transferred into OpenEXR. The AMPAS tool “ctlrender” is used to run .ctl scrips.

Ctlrender can combine several conversion steps with one command line chain, each initiated with –ctl command. For illustration purposes, these steps shall be shown serially, but the assumed practice is to execute one command per frame so that the .ctl scripts can be processed in parallel if computing resources are available.

For each .exr source frame, run the RRT to transform an ICES-space, ACES frame into OCES space. Since OCES is naturally HDR, do not write out OCES files unless the output is desired in floating point precision. Note: There is more configuration complexity with “ctlrender” that set up search paths etc. than considered here in this workflow simplification.

### Step 2: RRT through PQ ODT

Use an ODT structured to apply non-linear PQ EOTF to the OCES domain image and save the output to a 16-bit TIFF with utilizing Video Range in the color space desired (e.g. XYZ or 2020). Note AMPAS ODTs (e.g. for Digital Cinema and Rec 709) utilize a tone curve that ranges down to the display range (e.g. 48 or 100 nits) for HDR material. The author did not apply any tone curve in the material processed for a PQ ODT in this workflow project.

### Step 3: create .yuv encoder input for HM encoder

Use a looping script to queue up all the TIFF files and using tiff2ydzdx append them frame by frame into the .yuv file. As an example please see the files yuv.sh and makeYUV.sh.

For 10-bit:

tiff2ydzdx odt_output.tiff  709 B10

For 12-bit

# YDzDx

tiff2ydzdx odt_output.tiff  Y500 B12

# BT.2020 YCbCr

tiff2ydzdx odt_output.tiff  2020 B12

Note: Currently, the tiff2ydzdx program always writes a file called YDzDx.yuv regardless of whether the container is BT.2020 or YDzDx or BT.709.

# HEVC HM main12 encoder parameters

An example encoder setting:

For YDzDx:

TAppEncoderHighBitDepthStatic -c $EDRHOME/HEVC/HM/cfg/encoder_randomaccess_main10.cfg -i Confetti_JugglerYDzDx500.yuv -wdt 1920 -hgt 1080 -fr 24 -f $frames -fs $fs --InputBitDepth=12 --InternalBitDepth=12 --InputChromaFormat=420 -crqpofs -12 -vui 0 --Profile=main-RExt --Tier=main --Level=4.1  -b Encode.bin -o rec.yuv

Note: Chroma QP offset of -12 used.

For 2020:

TAppEncoderHighBitDepthStatic -c $EDRHOME/HEVC/HM/cfg/encoder_randomaccess_main10.cfg -i Confetti_Juggler2020.yuv -wdt 1920 -hgt 1080 -fr 24 -f $frames -fs $fs --InputBitDepth=12 --InternalBitDepth=12 --InputChromaFormat=420 -vui 1 --VideoFullRange=0 --ColourDescriptionPresent=1 --ColourPrimaries=9 --TransferCharateristics=15 --MatrixCoefficients=9 --Profile=main-RExt --Tier=main --Level=4.1  -b Encode.bin -o rec.yuv

Main12 is signaled with the default parameters in cfg/encoder_randomacess_main10.cfg by overriding both InputBitDepth=12 and InternalBitDepth=12.

# Decode and post-process

Use ydzdx2tiff to unpack the output .yuv file back to TIFF for further use.

ydzdx2tiff rec.yuv HD1920 Y500

ydzdx2tiff rec.yuv HD1920 2020

Note ydzdx2tiff expects a directory named “tifXYZ” to exist prior to running it.

# Conversion for Viewing

Convert decoded PQ TIFFs (whether XYZ or Rec. 2020) into files that can be critically viewed on a Pulsar (Dolby prototype monitor) or other files like 500 nit jpgs for pseudo-viewing in lieu of an actual HDR monitor.

# References

[1] E. Giorgianni, T. Madden, M. Kriss (2009) “Digital Color Management: Encoding Solutions”, Wiley-Blackwell, ISBN 978-0-470-51244-9

[2] S. Miller, M. Nezamabadi, S. Daly, “Perceptual Signal Coding for More Efficient Usage of Bit Codes”, SMPTE Journal, Vol. 122, No. 4, May-June 2013.

[3] R. Mantiuk, K. Myszkowski, H-P. Seidel, “Lossy Compression of High Dynamic Range Images and Video”

[4] R. Mantiuk, K. Myszkowski, H-P. Seidel, “High Dynamic Range Image and Video Compression – Human Visual Performance.”

[5] K. Sharman, N. Saunders, “16-bit Test Sequences for Range Extensions”,  JCTVC-O0069.

[6] S. Lasserre, F. Le Léannec, P. Lopez, Y. Oliver, D. Touzé, E. François, “High Dynamic Range video coding”, JCTVC-P0159

[7] G. Demos, “Filtering in High Dynamic Range (HDR) Context”, SMPTE 2013 Annual Technical Conference & Exhibition, Oct. 22-23, 2013.

# Patent rights declaration(s)

Universal Pictures and MovieLabs do not have any current or pending patent rights relating to the technology described in this contribution.