| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Valencia, ES, 27 March – 4 April 2014 | Document: JCTVC-Q0086r1 |
| --- | --- |

| Title: | Mastering Display Color Volume SEI |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chad Fogg, Jim Helman 530 Lytton Ave, Suite 300 Palo Alto, CA 94301 USA | Email: | chadfogg@gmail.com jhelman@movielabs.com |
| Source: | MovieLabs. |  |  |

_____________________________

# Abstract

A few editorial corrections are requested to section D.2.30 of the JCTVC-P1005_v1.doc release of HEVC RExt draft., and a change to the semantics of the elements max_display_mastering_luminance and min_display_mastering_luminance to express quantized fractional nits (candelas/meter^2) to the equivalent of 4 decimal places.

# Proposal

The following changes are requested in section D.2.30 (Mastering display colour volume SEI message syntax) of the latest HEVC draft [1] based on specifications [2]:

## Remove copy-and-paste fragment

Remove the sentence:

A picture in an access unit containing a tone mapping information SEI message with the same value of tone_map_id is output having PicOrderCntVal greater than PicOrderCnt( CurrPic ).

## Constant in quantized xy-coordinate

In a possible editorial error, a constant was substituted for the original formula constant proposed in JCTVC-P0084. It is suggested to change the scale factor from 216 back to 65535 to match the precision implied by the u(16) elements display_primaries_x, display_primaries_y, white_point_x, white_point y:

quantized_CE_xy_coordinate = Round( ( CIE_fractional_xy_coordinate * (1<<16)) − 1 )	(D-1)

Note that CIE_fractional_xy_coordinate is a real-valued number between 0.0 and 1.0

## Fractional nits

It is request to change the semantic description for max_display_mastering_luminance and min_display_mastering_luminance in section D.3.30

max_display_mastering_luminance specifies the target maximum luminance of the mastering display  in units of 1/10,000 candelas per square metre.

min_display_mastering_luminance specifies the target minimum luminance of the mastering display  in units of 1/10,000 candelas per square metre. When present, max_display_mastering_luminance shall be greater than or equal to min_display_mastering_luminance.

# References

[1] D. Flynn, M. Naccari, C. Rosewarne, J. Sole, G. Sullivan, T. Suzuki, “HEVC RExt Draft 6”, JCTVC-P1005

[2] MovieLabs Specification for Next Generation Video – v1.0

http://www.movielabs.com/ngvideo/

# Patent rights declaration(s)

MovieLabs does not have any current or pending patent rights relating to the technology described in this contribution.