| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Valencia, ES, 27 March – 4 April 2014 | Document: JCTVC-Q0088 |
| --- | --- |

| Title: | HDR/WCG StEM encoded with Y'DzDx |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Jim Helman, Chad Fogg, 530 Lytton Ave, Suite 300 Palo Alto, CA 94301 USA Bill Mandel 100 Universal City Plaza Universal City, CA 91608 USA | Email: | jhelman@movielabs.com chadfogg@gmail.com bill.mandel@nbcuni.com |
| Source: | MovieLabs, Universal Pictures |  |  |

_____________________________

# Abstract

For the investigations into high bitdepth (JCT Adhoc 18) and HDR/WGC adhoc (MPEG XYZ), results are described in this contribution from the encoding of five new High Dynamic Range / Wide Color Gamut test sequences are contributed to the Valencia meeting. The test sequences were remastered for HDR from original source material as described in [1]. The material is display-referred having been manually graded for a display with a peak luminance of 4,000 cd/m2 and a P3 color gamut.

The sequences were coded at an approximate target of 25Mbps using HM-13.0+RExt-6.0 main12 profile in random access configuration. Content input to the encoder was format as D65 white point adjusted Y’DzDx 4:2:0, 12-bit integer samples mapped to the PQ EOTF.

# Introduction

The five sequences that were taken from the Digital Cinema Initiatives HDR StEM for this test are as follows:

| No. | Brief description | Master frames |
| --- | --- | --- |
| 1. | StEM Confetti through Juggler | 1784 to 2144 |
| 2. | StEM Magic Hour: Fountain to Table | 3527 to 3887 |
| 3. | StEM Warm Night: Torch to Table (before slomo) | 6280 to 6640 |
| 4. | StEM Cool Night: Fog | 8208 to 8568 |
| 5. | StEM Rain | 9872 to 10232 |

The sequences were encoded using the HM-13.0+RExt-6.0. The software was complied with high bitdepth support (RExt__HIGH_BIT_DEPTH_SUPPORT = 1). The Main 12 profile in Random Access case was used, enabled by taking encoder_randomaccess_main10.cfg but changing profile to main-Rext and setting inputBitDepth and internalBitDepth to 12. Two scenarios were coded with different ChromaQPOffset values, 0 and -12 applied the same to both Cb and Cr in each set.

The attached Excel spreadsheet shows the bitrate, PSNR and md5sum of the results.

# Patent rights declaration(s)

MovieLabs and Universal Pictures do not have any current or pending patent rights relating to the technology described in this contribution.

# References

- J. Helman, W. Husak, “DCI HDR/WCG StEM Content Description”, JCTVC-P0228, JCTVC 17th Meeting: Valencia, ES, 27 March – 4 April 2014.