| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Valencia, ES, 27 March – 4 April 2014 | Document: JCTVC-Q0089 |
| --- | --- |

| Title: | Profile tables |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chad Fogg, Jim Helman 530 Lytton Ave, Suite 300 Palo Alto, CA 94301 USA | Email: | chadfogg@gmail.com jhelman@movielabs.com |
| Source: | Movielabs Inc. |  |  |

_____________________________

# Abstract

A table is requested in HEVC draft Annex A that summarizes the bitstream element restrictions for each profile.  It is suggested that this would clarify the definition of profiles added since the publication of HEVC version 1 that lack a unique code point.

# Proposal

A possible solution would resemble the following table:

|  | Bitstream |  |  |
| --- | --- | --- | --- |
| Element, when present | Main | Main10 | Main12 |
| general_profile_idc | 1 | 2 | 4 |
| chroma_format_idc | 1 | 1 | 1 |
| bit_depth_luma_minus8 | 0 | 0,1,2 | 0,1,2,3,4 |
| bit_depth_chroma_minus8 | 0 | 0,1,2 | 0,1,2,3,4 |
| transform_skip_rotation_enabled_flag | 0 | 0 | 0 |
| transform_skip_context_enabled_flag | 0 | 0 | 0 |
| intra_block_copy_enabled_flag | 0 | 0 | 0 |
| implicit_rdpcm_enabled_flag | 0 | 0 | 0 |
| explicit_rdpcm_enabled_flag | 0 | 0 | 0 |
| intra_smoothing_disabled_flag | 0 | 0 | 0 |
| high_precision_offsets_enabled_flag | 0 | 0 | 1 |
| fast_rice_adaptation_enabled_flag | 0 | 0 | 0 |
| log2_max_transform_skip_block_size_minus2 | 0 | 0 | 0 |
| extended_precision_processing_flag | 0 | 0 | 0 |
| cabac_bypass_alignment_enabled_flag | 0 | 0 | 0 |
| chroma_qp_adjustment_enabled_flag | 0 | 0 | 0 |
| separate_colour_plane_flag | 0,1 | 0,1 | 0 |
| sps_extension_flag[0] | 0 | 0 | 0,1 |
| sps_extension_7bits | 0,1 | 0,1 | 0,1 |
| CtbLog2SizeY | 4,5,6 | 4,5,6 | 4,5,6 |

| general_max_8bit_constraint_flag equal | 0 | 0 | 0 |
| --- | --- | --- | --- |

| general_max_10bit_constraint_flag equal | 0 | 0 | 0 |
| --- | --- | --- | --- |
| general_max_12bit_constraint_flag | 0 | 0 | 1 |
| general_intra_constraint_flag equal | 0 | 0 | 0 |
| general_max_420chroma_constraint_flag | 0 | 0 | 1 |
| general_max_422chroma_constraint_flag | 0 | 0 | 1 |
| general_max_monochrome_constraint_flag | 0 | 0 | 0 |
| general_lower_bit_rate_constraint_flag | 0 | 0 | 1 |

Similar tables would be constructed for Monochrome 12, Main 12, Main 4:2:2 10, Main 4:2:2 12, Main 4:4:4, Main 4:4:4 10, Main 4:4:4 12, Main Intra, Main 10 Intra, Main 12 Intra, Main 4:2:2 10 Intra, Main 4:2:2 12 Intra, Main 4:4:4 Intra, Main 4:4:4 10 Intra, Monochrome 16, and Main 4:4:4 12 Intra profile etc.

# Patent rights declaration(s)

Movielabs does not have any current or pending patent rights relating to the technology described in this contribution.