| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Valencia, ES, 27 March – 4 April 2014 | Document: JCTVC-Q0092 |
| --- | --- |

| Title: | Non-RCE1: Simplification of RCE1 Test2 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tzu-Der Chuang, Ching-Yeh Chen, Yu-Wen Huang, Shawmin Lei No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 | Tel: Email: | Shawmin Lei +886-3-5670766 ext. 25555 {peter.chuang, chingyeh.chen , yuwen.huang, shawmin.lei} @mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

In RCE1 Test2 (JCTVC-Q0049), a CU-level adaptive MV precision method is proposed. A CU-level flag is signaled to indicate whether all PUs in the CU have integer-precision MVs. The context formation of the CU-level adaptive MV precision flag requires the information of the coded adaptive MV precision flag of the upper CU which results in a requirement of a line buffer for this flag. In this contribution, the context formation of the adaptive MV precision flag is proposed to use only one context without any information of neighboring CU to remove the line buffer. When RCE1 Test2 is the anchor and implementation basis, the proposed method reportedly shows 0.2% and 0.1% loss for SC YUV 444 sequences under RA-Main-Tier and LB-Main-Tier, respectively. It is also found that the adaptive MV precision is less efficient for merge mode. Therefore, a constraint is proposed to disable the adaptive MV precision for merge mode PUs. With the proposed single context adaptive MV precision flag coding, when RCE1 Test2 is the anchor and implementation basis, the proposed constraint reportedly shows 0.2% and 0.6% BD-rate savings for SC YUV 444 sequences under RA-Main-Tier and LB-Main-Tier, respectively. In addition, the encoding times are reduced by 9% and 10% for RA-Main-Tier and LB-Main-Tier, respectively.

# Introduction

A CU-level adaptive MV precision method was proposed at the 16th JCT-VC meeting and is studied in RCE1 Test2 [1], where a CU-level flag is signaled to indicate whether all PUs in the CU have integer-precision MVs. If the flag is 1, the MV precision is in integer precision. The MV predictors of PUs are rounded to integer precision and MV differrences are signaled in integer precision. Otherwise, the MV precision is in qurter precision. The MV coding is the same with HEVC version 1 and the sub-pixel motion compensation is allowed. In RCE1 Test2, the adaptive MV precision is applied on all inter CU. The context formation of the CU-level flag requires the information of the coded adaptive MV precision flag of the upper CU, which results in a requirement of a line buffer for this flag. The adaptive MV precision is also found to be less efficient for merge mode.

# Proposed methods

## Single context adaptive MV precision flag coding

To reduce the line buffer for the adaptive MV precision flag, it is proposed to use only one context during the context formation of the adaptive MV precision flag. Therefore, no information of neighboring CUs is required to be stored.

## Disabling adaptive MV precision for merge mode

The adaptive MV precision is also found to be less efficient for merge mode. Therefore, we propose to disable the adaptive MV precision for all merge mode PUs. In RCE1 Test2, the adaptive MV precision flag is signaled before the syntax of the partitioning mode. In the proposed method, the adaptive MV precision flag is moved after all the PU syntax signaling. The adaptive MV precision flag is only transmitted when the current CU contains at least one merge mode PU. Otherwise, the adaptive MV precision flag is inferred as false.

# Simulation results

Experiments were conducted based on RCE1 Test2. All the results were simulated according to the common test conditions defined in JCTVC-P1121 [2].

Lossy coding results and lossless coding results of the proposed single context adaptive MV precision flag coding using RCE1 Test2 as the anchor and implementation basis are shown in Table 1. The results reportedly show 0.2% and 0.1% loss for SC YUV 444 sequences under RA-Main-Tier and LB-Main-Tier, respectively. for SC YUV 444 sequences under RA-Main-Tier and LB-Main-Tier.

Lossy coding results and lossless coding results of the proposed single context adaptive MV precision flag coding and disabling adaptive MV precision for merge mode using RCE1 Test2 as the anchor and implementation basis are shown in Table 2, where 0.2% and 0.6%BD-rate savings are shown for SC YUV 444 sequences under RA-Main-Tier and LB-Main-Tier, respectively. The encoding times are reduced by 9% and 10% for RA-Main-Tier and LB-Main-Tier, respectively.

Table 3 shows the lossy coding results and lossless coding results of the proposed methods using HM-13.0+RExt-6.0 as the anchor. It is reported to achieve 0.9% and 1.3% BD-rate savings for SC YUV 444 sequences under RA-Main-Tier and LB-Main-Tier, respectively.

# Conclusions

In this contribution, two methods of RCE1 Test2 are proposed. To reduce the line buffer, a single context adaptive MV precision flag coding is proposed. The results reportedly show 0.2% and 0.1% loss for SC YUV 444 sequences under RA-Main-Tier and LB-Main-Tier, respectively.. To improve the coding efficiency, the adaptive MV precision is disabled for merge mode PUs. The BD-rate savings of combining the two proposed methods on top of RCE2 Test1 are 0.2% and 0.6% for SC YUV 444 sequences under RA-Main-Tier and LB-Main-Tier, respectively. The encoding times for RA-Main-Tier and LB-Main-Tier are reduced by 9% and 10%, respectively.

# References

- X. Li, J. Sole, M. Karczewicz, “RCE1: Adaptive MV Precision,” Document of Joint Collaborative Team on Video Coding, JCTVC-Q0049, Mar. 2014.

- X. Li, J. Xu, Y. Zhou, “HEVC Range Extensions Core Experiment 1(RCE1): Adaptive MV precision,” Document of Joint Collaborative Team on Video Coding, JCTVC-P1121, Jan. 2014.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Table 1.  Results of the proposed single context adaptive MV precision flag coding; anchor: RCE1 Test2

Table 2.  Results of the proposed single context adaptive MV precision flag coding and disabling adaptive MV precision for merge mode; anchor: RCE1 Test2

Table 3.  Results of the proposed single context adaptive MV precision flag coding and disabling adaptive MV precision for merge mode; anchor: HM-13.0+RExt-6.0