| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Valencia, ES, 27 March – 4 April 2014 | Document: JCTVC-Q0093 |
| --- | --- |

| Title: | Single color intra mode for screen content coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yi-Wen Chen, Yu-Chen Sun, Yu-Wen Huang, Shawmin Lei No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 | Tel: Email: | Shawmin Lei +886-3-5670766 ext. 25555 {yiwen.chen, yc.sun, yuwen.huang, shawmin.lei} @mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

In this contribution, an intra coding mode termed as single color mode is proposed to code the smooth area within a picture for screen content coding (SCC). A CU-level flag is signaled to indicate whether a CU is coded as single color mode. When a CU is coded as single color mode, it is reconstructed by filling this CU with one single value. Additionally, an index is transmitted to select one out of several available color sample candidates derived from the spatial neighboring pixels to fill in the CU. Compared to the anchor HM-13.0_RExt-6.0, the proposed method reportedly shows 0.8% BD-rate savings for the average of difference classes of SCC call for proposal (CfP) sequences under All-Intra conditions. The results also show the proposed method could achieve 1.3%, 1.7%, 1.0% and 1.6% BD-rate savings under All-Intra conditions for the classes of “RGB, text & graphics with motion,1080p”, “RGB, text & graphics with motion,720p”, “YUV, text & graphics with motion,1080p” and “YUV, text & graphics with motion,720p”, respectively.

# Introduction

Because there is evidence that significant improvements in coding efficiency can be obtained by exploiting the characteristics of screen content, a Call for Proposals is being issued for developing possible screen context extensions of the High Efficiency Video Coding (HEVC) standard [1].

It is found that screen content video data show several signal characteristics different from natural video data. The most intuitive one is that screen content video content contains a lot of smooth area with similar sample values. For most of the cases, the samples within the smooth area even share single sample value.

In this contribution, a coding mode termed as “Single Color Mode” is proposed to efficiently code the smooth area within a picture. The concept of single color mode is to simply reconstruct the current coding unit (CU) as a smooth area with single sample value. With the help of this new coding mode, the smooth area within a picture could be coded more efficiently through more concise syntax.

# Proposed method

In the proposed scheme, one flag is signaled in the slice header to flexibly enable/disable the single color mode for the entire slice. When the slice-level flag is enabled, one CU-level flag is further signaled to indicate the enabling/disabling of the single color mode for coding each CU.

To reconstruct a CU coded as single color mode, a sample color candidate list is first constructed by inserting the sample candidates derived from the neighboring samples of current CU in a predefined order into the candidate list. As shown in Figure 1 (each square represents a sample), the spatial neighboring samples are those five reconstructed samples around the current CU and are inserted into the candidate list in the order: (Am, Bn, B0, A0 and D). To achieve higher coding efficiency, a redundancy pruning process is applied to the construction of sample candidate list. A sample candidate will not be inserted into the candidate list if it is identical to any of the previously derived sample candidates. In the proposed scheme, the size of the sample candidate list is fixed to 2. If empty entry still exists in the candidate list after the derivation of candidates from the spatial neighboring samples, a default candidate with value equal to 1<<(g_bitDepth-1) will be used to fill in those empty entries. After the sample candidate list is constructed, a sample candidate index is signaled in the bitstream to indicate which sample candidate is used to fill the current CU.

Figure 1. The spatial sample candidates used to construct the sample candidate list.

# Simulation results

Experiments were conducted based on HM-13.0_RExt-6.0 [1]. All the results were simulated according to the common test conditions for SCC CfP defined in [2]. The anchor is HM-13.0_RExt-6.0.

For the SCC CfP test conditions, the lossy coding results of the proposed method are shown in Table 1, where 0.0-1.7% BD-rate savings are shown for different classes of screen context sequences under All-Intra conditions. The lossless coding results are shown in Table 2, where 0.0-0.4% BD-rate savings are shown for different classes of screen context sequences under All-Intra conditions.

Additional experiments are also conducted under AhG8 test conditions [3]. For the AhG8 test conditions, the lossy coding results of the proposed method are shown in Table 3, where 0.0-1.3% BD-rate savings are shown for different classes of screen context sequences under All-Intra conditions. The lossless coding results are shown in Table 4, where 0.0-0.4% BD-rate savings are shown for different classes of screen context sequences under All-Intra conditions.

As shown in Table 1 to Table 4, the proposed single color mode consistently provides coding performance while introducing limited complexity increase to the encoder and decoder. The decoding time is even reduced by adopting the proposed single color mode since the decoding and reconstruction of the single color mode is very simple.

# Conclusions

In this contribution, an intra coding mode termed as single color mode is proposed to efficiently code the smooth area within a picture. The simulation results reportedly show that the proposed method introduces up to 1.7% and 0.4% under All-Intra lossy and lossless conditions for SCC CfP sequences, respectively. The simulation results also show that the proposed method introduces up to 1.4% and 0.4% under All-Intra lossy and lossless conditions for Rext AhG8 sequences, respectively.

# References

- M. Naccari, C. Rosewarne, K. Sharman, and G. J. Sullivan, “HEVC Range Extensions Test Model 6 Encoder Description,” Document of Joint Collaborative Team on Video Coding, JCTVC-P1013, Jan. 2014

- ISO/IEC JTC1/SC29/WG11 (MPEG) and ITU-T SG16 WP3 (VCEG) document N14090, “Joint Call for Proposals for coding of screen content,” Jan. 2014

- H. Yu, R. Cohen, A. Duenas, D.-K. Kwon, T. Lin, J. Xu, “JCT-VC AHG report: Screen content coding (AHG8),” Document of Joint Collaborative Team on Video Coding, JCTVC-P0008, Jan. 2014

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Table 1.  Results of the proposed method for lossy testing conditions; anchor: HM-13.0_RExt-6.0

Table 2.  Results of the proposed method for lossless testing conditions; anchor: HM-13.0_RExt-6.0

Table 3.  Results of the proposed method for lossy testing conditions; anchor: HM-13.0_RExt-6.0

Table 4.  Results of the proposed method for lossless testing conditions; anchor: HM-13.0_RExt-6.0