| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 17th Meeting: Valencia, ES, 27 March – 4 April 2014 | Document: JCTVC-Q0094 |
| --- | --- |

| Title: | AHG10: Suggested Software for Palette Coding based on RExt6.0 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Wei Pu Xun Guo Patrice Onno PoLin Lai Jun Xu | Tel: Email: | +1-858-651-7749 Wei.Pu@qti.qualcomm.com Xun.Guo@microsoft.com Patrice.Onno@crf.canon.fr Polin.Lai@mediatek.com jun.xu@am.sony.com |
| Source: | Qualcomm Incorporated Microsoft Corp. Canon Research Centre France Sony Electronics Inc. MediaTek Inc. |  |  |

_____________________________

# Abstract

During San Jose meeting, AHG10 was established to study palette based coding. Suggested software based on RExt5.1 was released in JCTVC-P0303. This contribution presents the results and source code of porting JCTVC-P0303 to RExt6.0.

# Introduction

Run-length based palette mode coding was studied in [1]. The general procedure is summarized as follows:

- Transmission of the palette: An entry-wise prediction scheme is used to encode the current palette based on the palette of the previous CU. After that, no predicted entries are transmitted.

- Transmission of pixel values: the pixels in the CU are encoded in a raster scan order using 3 modes.

  - “Copy Left Run” mode: A palette index is first signaled followed by value M, which means the following M palette indexes are the same as the one signaled.

  - “Copy Above Run” mode: A value N is transmitted to indicate that for the following N palette indexes are the same as their above neighbors, respectively.

  - “Escape” mode: The quantized pixel value is transmitted directly.

# Algorithm description

This software upgrade did not change any palette coding related algorithm compared with [1]. Specifically, the following MACROs are defined in Lib/TLibCommon/Typedef.h. RExt6.0 Anchor results can be exactly matched [2] if PALETTE_MODE = 0. Note that encoder cfg parameter Level must be set to 6.2.

Experimental Results

#define PALETTE_MODE                                       1

#if PALETTE_MODE

#define PLT_FIRST_BIN_CONTEXT_CODING  1 ///< escape pixel value only the first bin is context coded to resolve CABAC throughput issue, JCTVC-P0231

#define PLT_ESCAPE_NO_PREDICTION                       1 ///< escape pixel force no prediction, JCTVC-P0231

#define PLT_ESCAPE_ADAPTIVE_BD_FLC                  1 ///< escape pixel dynamic range determined by QP, JCTVC-P0231

#define PLT_PLT_ERR_LIMIT                                          1 ///< PLT calculation based on QP, JCTVC-P0231 (encoder only)

#define PLT_PREVIOUS_CU_PALETTE_PREDICTION 1 ///< use previous palette for prediction: JCTVC-P0114

#define PLT_NO_CONSECUTIVE_COPY_ABOVE       1 ///< save syntax for consecutive "copy above": JCTVC-P0116, JCTVC-P0095

#define PLT_NO_COPY_FROM_ABOVE_ON_FIRST_LINE    1 ///< no copy above for first line: JCTVC-P0116, JCTVC-P0179

#define PLT_REMOVE_ESCAPE_FLAG                  1 ///< remove escape flag, JCTVC-P0108

#if PLT_REMOVE_ESCAPE_FLAG

#define MAX_PLT_SIZE                                            31

#else

#define MAX_PLT_SIZE                                            32

#endif

#endif

Simulations are performance under AHG 8 test condition [2] based on RExt 6.0 reference software.

Table 1. AHG10 Reference SW v.s. RExt 6.0 Anchor (Lossy)

|  | All Intra Main-tier |  |  | All Intra High-tier |  |  | All Intra Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class F | -0.6% | -2.3% | -1.5% | -0.8% | -1.7% | -1.3% | -0.9% | -1.3% | -1.1% |
| Class B | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB 4:4:4 SC | -9.0% | -8.3% | -8.6% | -10.6% | -10.3% | -10.3% | -12.5% | -12.9% | -12.7% |
| RGB 4:4:4 Animation | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YCbCr 4:4:4 SC | -6.6% | -9.2% | -10.8% | -9.3% | -11.4% | -12.3% | -11.9% | -13.9% | -14.9% |
| YCbCr 4:4:4 Animation | 0.1% | -0.1% | -0.1% | 0.1% | -0.1% | -0.1% | 0.0% | -0.1% | -0.1% |
| RangeExt | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB 4:4:4 SC (Optional) | -24.2% | -23.2% | -24.3% | -27.0% | -26.3% | -27.0% | -32.0% | -32.4% | -33.4% |
| YCbCr 4:4:4 SC (Optional) | -18.3% | -23.7% | -22.5% | -23.3% | -27.6% | -27.5% | -29.0% | -34.4% | -34.9% |
| Enc Time[%] | 105% |  |  | 105% |  |  | 105% |  |  |
| Dec Time[%] | 98% |  |  | 96% |  |  | 95% |  |  |
|  | Random Access Main-tier |  |  | Random Access High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | -0.3% | -2.7% | -1.3% | -0.4% | -1.7% | -0.9% |  |  |  |
| Class B | 0.2% | 0.0% | 0.0% | 0.0% | -0.1% | 0.0% |  |  |  |
| RGB 4:4:4 SC | -8.2% | -7.3% | -7.7% | -8.8% | -8.5% | -8.7% |  |  |  |
| RGB 4:4:4 Animation | 0.0% | -0.1% | 0.0% | 0.0% | -0.1% | 0.0% |  |  |  |
| YCbCr 4:4:4 SC | -6.2% | -8.5% | -9.7% | -8.2% | -10.1% | -11.0% |  |  |  |
| YCbCr 4:4:4 Animation | 0.0% | -0.1% | -0.2% | 0.0% | -0.1% | -0.1% |  |  |  |
| RangeExt | 0.1% | 0.0% | 0.1% | 0.1% | -0.1% | 0.0% |  |  |  |
| RGB 4:4:4 SC (Optional) | -18.2% | -17.3% | -18.3% | -20.4% | -19.6% | -20.1% |  |  |  |
| YCbCr 4:4:4 SC (Optional) | -15.8% | -22.3% | -22.0% | -19.5% | -24.4% | -25.0% |  |  |  |
| Enc Time[%] | 104% |  |  | 103% |  |  |  |  |  |
| Dec Time[%] | 103% |  |  | 103% |  |  |  |  |  |
|  | Low delay B Main-tier |  |  | Low delay B High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | -0.3% | -2.1% | -1.0% | -0.2% | -1.2% | -0.5% |  |  |  |
| Class B | 0.1% | -0.4% | -0.5% | 0.1% | -0.3% | -0.3% |  |  |  |
| RGB 4:4:4 SC | -3.3% | -3.2% | -3.4% | -3.8% | -3.9% | -3.9% |  |  |  |
| RGB 4:4:4 Animation | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| YCbCr 4:4:4 SC | -2.6% | -3.5% | -4.1% | -3.5% | -4.1% | -4.8% |  |  |  |
| YCbCr 4:4:4 Animation | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% |  |  |  |
| RangeExt | 0.1% | 0.0% | 0.2% | 0.0% | -0.1% | 0.1% |  |  |  |
| RGB 4:4:4 SC (Optional) | -9.6% | -9.1% | -9.3% | -13.1% | -12.9% | -13.7% |  |  |  |
| YCbCr 4:4:4 SC (Optional) | -9.0% | -12.7% | -12.9% | -12.1% | -15.5% | -15.7% |  |  |  |
| Enc Time[%] | 98% |  |  | 99% |  |  |  |  |  |
| Dec Time[%] | 105% |  |  | 105% |  |  |  |  |  |

Table 2. AHG10 Reference SW v.s. RExt 6.0 Anchor (Lossless)

|  | Average bit-rate increase |  |  |
| --- | --- | --- | --- |
|  | AI | RA | LB |
| Class F | -0.3% | 0.0% | 0.0% |
| Class B | 0.0% | 0.0% | 0.0% |
| RGB 4:4:4 SC | -10.8% | -7.4% | -4.6% |
| RGB 4:4:4 Animation | 0.0% | 0.0% | 0.0% |
| YCbCr 4:4:4 SC | -11.3% | -8.3% | -5.3% |
| YCbCr 4:4:4 Animation | -0.6% | -0.1% | 0.0% |
| RangeExt | 0.0% | 0.0% | 0.0% |
| RGB 4:4:4 SC (Optional) | -27.0% | -19.6% | -10.6% |
| YCbCr 4:4:4 SC (Optional) | -29.4% | -23.3% | -15.1% |
| Enc Time[%] | 110% | 104% | 99% |
| Dec Time[%] | 95% | 105% | 108% |

# References

- W. Pu, X. Guo, P. Onno, P. Lai, and J. Xu, “Suggested software for the AHG on investigation of palette mode coding tools,” JCTVC-P0303, San Jose, USA, Jan. 2014.

- C. Rosewarne, K. Sharman, and D. Flynn, “Common test conditions and software reference configurations for HEVC range extensions,” JCTVC-P1006, Sna Jose, USA, Jan. 2014.

# Patent rights declaration(s)

Qualcomm may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Microsoft Corp. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Canon Research Centre France may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Sony Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).