The proposed modifications are based on HEVC Range Extensions text specification: Draft 6 (JCTVC-P1005). The inserted parts are marked in yellow. The deleted parts are marked in pink.

#### 7.3.8.x Block vector difference syntax

| bvd_coding( x0, y0) { | Descriptor |
| --- | --- |
| abs_bvd_msb_idx_p1[ 0] | ae(v) |
| abs_ bvd_msb_idx_p1[1] | ae(v) |
| if(abs_bvd_msb_idx_p1 [0] > 0 ) { |  |
| if(abs_bvd_msb_idx_p1 [0] > 1) |  |
| abs_bvd_refinement_bits [0] | ae(v) |
| bvd_sign_flag [0] | ae(v) |
| } |  |
| if(abs_bvd_msb_idx_p1 [1]>0 ) { |  |
| if(abs_bvd_msb_idx_p1 [1 ] > 1) |  |
| abs_bvd_refinement_bits [1] | ae(v) |
| bvd_sign_flag [ 1] | ae(v) |
| } |  |
| } |  |

#### Motion vector difference semantics

abs_mvd_greater0_flag[ compIdx ] specifies whether the absolute value of a motion vector component difference is greater than 0.

abs_mvd_greater1_flag[ compIdx ] specifies whether the absolute value of a motion vector component difference is greater than 1.

When abs_mvd_greater1_flag[ compIdx ] is not present, it is inferred to be equal to 0.

abs_mvd_minus2[ compIdx ] plus 2 specifies the absolute value of a motion vector component difference.

When abs_mvd_minus2[ compIdx ] is not present, it is inferred to be equal to −1.

mvd_sign_flag[ compIdx ] specifies the sign of a motion vector component difference as follows:

- If mvd_sign_flag[ compIdx ] is equal to 0, the corresponding motion vector component difference has a positive value.

- Otherwise (mvd_sign_flag[ compIdx ] is equal to 1), the corresponding motion vector component difference has a negative value.

When mvd_sign_flag[ compIdx ] is not present, it is inferred to be equal to 0.

The motion vector difference lMvd[ compIdx ] for compIdx = 0..1 is derived as follows:

lMvd[ compIdx ] = abs_mvd_greater0_flag[ compIdx ] *
		( abs_mvd_minus2[ compIdx ] + 2 ) * 	( 1 − 2 * mvd_sign_flag[ compIdx ] ) 	(7-1)

The variable MvdLX[ x0 ][ y0 ][ compIdx ], with X being 0 or 1, specifies the difference between a list X vector component to be used and its prediction. The value of MvdLX[ x0 ][ y0 ][ compIdx ] shall be in the range of −215 to 215 − 1, inclusive. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered prediction block relative to the top-left luma sample of the picture. The horizontal motion vector component difference is assigned compIdx = 0 and the vertical motion vector component is assigned compIdx = 1.

The variable BvdIntra[ x0 ][ y0 ][ compIdx ] specifies the difference between a vector component to be used for the intra block copying prediction mode and its prediction. The value of BvdIntra[ x0 ][ y0 ][ compIdx ] shall be in the range of −128 to 128, inclusive. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered prediction block relative to the top-left luma sample of the picture. The horizontal block vector component is assigned compIdx = 0 and the vertical block vector component is assigned compIdx = 1.

- If refList is equal to 0, MvdL0[ x0 ][ y0 ][ compIdx ] is set equal to lMvd[ compIdx ] for compIdx = 0..1.

- Otherwise (refList is equal to 1), MvdL1[ x0 ][ y0 ][ compIdx ] is set equal to lMvd[ compIdx ] for compIdx = 0..1.

- Otherwise (refList is equal to 2), BvdIntra[ x0 ][ y0 ][ compIdx ] is set equal to lMvd[ compIdx ] for compIdx = 0..1.

#### 7.4.9.x Block vector difference semantics

abs_bvd_msb_idx_p1[ compIdx] minus 1 specifies the most significant bit index of the absolute value of a block vector component difference in a binary representation. abs_bvd_msb_idx_p1[ compIdx] equal to 0 indicates the absolute value of a block vector component difference is equal to 0.

abs_bvd_refinement_bits [compIdx] specifies the refinement bits of the absolute value of a block vector component difference in a binary representation. When abs_bvd_refinement_bits [compIdx] is not present, it is inferred to be equal to 0.

bvd_sign_flag [compIdx] specifies the sign of a block vector component difference as follows:

- If bvd_sign_flag[ compIdx ] is equal to 0, the corresponding block vector component difference has a positive value.

- Otherwise (bvd_sign_flag[ compIdx ] is equal to 1), the corresponding block vector component difference has a negative value.

The block vector difference Bvd[ compIdx ] for compIdx = 0..1 is derived as follows:

- If abs_bvd_msb_idx_p1[ compIdx] is equal to 0,

Bvd[ compIdx ] = 0.

- Otherwise (abs_bvd_msb_idx_p1[ compIdx] is greater than 0),

Bvd[ compIdx ] =
		(  ( 1 << ( abs_bvd_msb_idx_p1[compIdx] – 1 ) )   +  abs_bvd_refinement_bits[compIdx] ) * 	( 1 − 2 * bvd_sign_flag[ compIdx ] )

The variable BvdIntra[ x0 ][ y0 ][ compIdx ] specifies the difference between a vector component to be used for the intra block copying prediction mode and its prediction. The value of BvdIntra[ x0 ][ y0 ][ compIdx ] shall be in the range of −127 to 127, inclusive. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered prediction block relative to the top-left luma sample of the picture. The horizontal block vector component is assigned compIdx = 0 and the vertical block vector component is assigned compIdx = 1. BvdIntra[ x0 ][ y0 ][ compIdx ] is set equal to Bvd[ compIdx ] for compIdx = 0..1.