The proposed modifications are based on HEVC Range Extensions text specification: Draft 6 (JCTVC-P1005). The inserted parts are marked in yellow.

#### 7.3.8.x Block vector difference syntax

| bvd_coding( x0, y0) { | Descriptor |
| --- | --- |
| abs_bvd_msb_idx_p1[ 0] | ae(v) |
| abs_ bvd_msb_idx_p1[1] | ae(v) |
| if(abs_bvd_msb_idx_p1 [0] > 0 ) { |  |
| if(abs_bvd_msb_idx_p1 [0] > 1) |  |
| abs_bvd_refinement_bits [0] | ae(v) |
| bvd_sign_flag [0] | ae(v) |
| } |  |
| if(abs_bvd_msb_idx_p1 [1]>0 ) { |  |
| if(abs_bvd_msb_idx_p1 [1 ] > 1) |  |
| abs_bvd_refinement_bits [1] | ae(v) |
| bvd_sign_flag [ 1] | ae(v) |
| } |  |
| } |  |

#### 7.4.9.x Block vector difference semantics

abs_bvd_msb_idx_p1[ compIdx] minus 1 specifies the most significant bit index of the absolute value of a block vector component difference in a binary representation. abs_bvd_msb_idx_p1[ compIdx] equal to 0 indicates the absolute value of a block vector component difference is equal to 0.

abs_bvd_refinement_bits [compIdx] specifies the refinement bits of the absolute value of a block vector component difference in a binary representation. When abs_bvd_refinement_bits [compIdx] is not present, it is inferred to be equal to 0.

bvd_sign_flag [compIdx] specifies the sign of a block vector component difference as follows:

- If bvd_sign_flag[ compIdx ] is equal to 0, the corresponding block vector component difference has a positive value.

- Otherwise (bvd_sign_flag[ compIdx ] is equal to 1), the corresponding block vector component difference has a negative value.

The block vector difference Bvd[ compIdx ] for compIdx = 0..1 is derived as follows:

- If abs_bvd_msb_idx_p1[ compIdx] is equal to 0,

Bvd[ compIdx ] = 0.

- Otherwise (abs_bvd_msb_idx_p1[ compIdx] is greater than 0),

Bvd[ compIdx ] =
		(  ( 1 << ( abs_bvd_msb_idx_p1[compIdx] – 1 ) )   +  abs_bvd_refinement_bits[compIdx] ) * 	( 1 − 2 * bvd_sign_flag[ compIdx ] )