| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Valencia, ES, 27 March – 4 April 2014 | Document: JCTVC-Q0095 |
| --- | --- |

| Title: | AHG8: Coding the prediction differences of the intra BC vectors |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Shih-Ta Hsiang, Shawmin Lei No. 1, Dusing Rd. 1, Hsinchu, Taiwan 30078 | Tel: Email: | Shawmin Lei +886-3-5670766 ext. 25555 {shih-ta.hsiang, shawmin.lei} @mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

This contribution develops a new method for coding the block vector prediction difference associated with the Intra block copy (BC) coding mode. The proposed method re-uses the existing binarization scheme employed for coding the motion vector difference in the current HEVC. However, an improved context modeling scheme has been developed for entropy coding the resulting bin string. It is reported to achieve 1.2%, 1.3%, and 1.0% Luma BD-rate savings for YCbCr 4:4:4 SC sequences under AI-Main-Tier, RA-Main-Tier, and LB-Main-Tier, respectively, under the AHG8 CTCs but with short-length test videos.

# Introduction

A new intra block copy (Intra BC) coding tool, indicated by a new syntax flag intra_bc_flag, has been adopted into the HEVC range extension, as specified by HEVC Range Extensions (RExt) text specification: Draft 6. When intra_bc_flag is equal to 1, the current CU is coded in the IntraBC mode and is predicted by the reconstructed block region before deblock & SAO filtering in the current frame, followed by residual coding. The location of the reference block is indicated by a block vector, specifying the coordinate offsets in the horizontal and vertical directions, respectively, from the current block.

According to the current HEVC Text draft 6, the block vector of the current IntraBC CU is predicted by the block vector of the last IntraBC CU. The resulting prediction difference of the block vector is then coded by re-using the existing syntax for coding the motion vector difference in an Inter CU. The absolute value of each component of vectors is binarized by the truncated unary code, cascaded with the 1st order of Exp-Golomb (EG1) code when the absolute value is greater than 2. The first two bins of the resulting bin string are encoded in the CABAC mode. For the absolute value of syntax component greater than one, the absolute value minus 2 is binarized by the EG1 code and encoded in the bypass mode.

# Proposed method

This contribution develops a new method for coding the block vector difference associated with the Intra BC coding mode. The proposed method represents the absolute value, | d |, of each component of the block difference vector by its most significant bit (MSB) index followed by its refinement bits. The MSB index for syntax value 0 is assigned to -1. The MSB index plus one, referred to as msb_plus_one, is binarized by the unary code. The remaining refinement bits are represented in the fixed-length binary code with the codeword length determined by the MSB index. Incidentally, this binarization scheme leads to exactly the same bin string as that of the binarization scheme employed in the current HEVC for coding motion vector difference, as compared in Table 1.

The resulting bin stings are coded by context adaptive binary arithmetic coding (CABAC). The bins for representing msb_plus_one are entropy coded in the CABAC mode. The context selection is conditioned on bin index and vector component index. The refinement bins and sign flags are coded in the bypass mode.

Table 1: The bin strings generated by the proposed method and HEVC mvd coding

|  | Proposed Method |  | HEVC mvd_coidng() |  |  |
| --- | --- | --- | --- | --- | --- |
| ¦ d ¦ | msb_plus_one | refinement bins | TU prefix | EG1 prefix | EG suffix |
| 0 | 0 | - | 0 | - | - |
| 1 | 10 | - | 10 | - | - |
| 2 | 110 | 0 | 11 | 0 | 0 |
| 3 | 110 | 1 | 11 | 0 | 1 |
| 4 | 1110 | 00 | 11 | 10 | 00 |
| 5 | 1110 | 01 | 11 | 10 | 01 |
| 6 | 1110 | 10 | 11 | 10 | 10 |
| 7 | 1110 | 11 | 11 | 10 | 11 |

# Experimental results

The proposed method has been implemented using reference software HM-13.0+RExt-6.0 as the code base. The simulation is conducted according to the common test condition specified in [1] but with short-length test videos. The simulation results are summarized in Tables 1 and 2 for lossy and lossless coding, respectively. The full-length simulation results will be updated once they are available.

Table 2: BD-rate results versus the anchor (1 sec. video only)

Table 3: Lossless bit-rate results versus the anchor (1 sec. video only)

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- D. Flynn, et al, “Common test conditions and software reference configurations for HEVC range extensions,” Document of Joint Collaborative Team on Video Coding, JCTVC-P1006, 16th Meeting: San José, US, 9–17 Jan. 2014.