| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Valencia, ES, 27 March – 4 April 2014 | Document: JCTVC-Q0097 |
| --- | --- |

| Title: | SCE1: Crosscheck report on color gamut scalability using gain-offset models (JCTVC-0072) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xiang Li | Tel: Email: | +1 858 658 3923 lxiang@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution reports the crosschecking results for JCTVC-Q0072 test 4 on color gamut scalability using gain-offset models. The simulation results reportedly matched those provided by the proponents.

# Introduction

In JCTVC-Q0072, 4 tests were proposed for color gamut scalability. Particularly in test 4, polynomial based mapping was proposed. The non-linear relationship between two color spaces is approximated with a second order polynomial equation as follows:

YCbCr2020=Gain1(Y,Cb,Cr).Y−Y0Cb−Cb0Cr−Cr07092+Gain2(Y,Cb,Cr).YCbCr709+Offset(Y,Cb,Cr)	(1)

where Gain1(Y, Cb, Cr) and Gain2(Y, Cb, Cr) are 3x3 matrices and Offset(Y, Cb, Cr) is a 1x3 matrix. (Y0, Cb0, Cr0) is the central point of the corresponding region to reduce the dynamic range of the polynomial part. It was claimed that the benefit of using polynomial equations is that one could reduce the number of regions as non-linear characteristics can already be captured with the polynomial equation.

# Experimental results

We received the source code from the proponents, implemented in SHM-5.0, and did a code study to verify that the proposed method was implemented as described. We ran simulations for the cases of AI-1, AI-2x, RA-1x, and RA-2x with SCE1 test sequences [1].

The results matched what were provided by the proponents and are summarized as follows

# Complexity analysis

The complexity analysis provided by the proponent to SCE1 summary report JCTVC-Q0021 is also verified except that the bit-depth of multiplier for color mapping. . The weighting parameters are w restricted to [-10, 10] and then left-shift by 10 bits (macro CGS_WP_Matrix_Precision). Therefore, the bit-depth of the weighting parameters is 15-bit. For the second order polynomial, it is calculated as

w⋅(2⋅(|x−512|2>>10))

where w is 15-bit as mentioned above, x is 10-bit. Therefore, the max bit-depth of the multiplier in the part of second-order polynomial is 15-bit x 9-bit.

For the linear part, it is calculated as

w⋅x

Therefore, the related bit-depth is 15-bit x 10-bit for existing software implementation (Q0072). [-2, 2047/1024 ) is a reasonable range, and can be used to reduce the bit-depth of multiplication to 12-bit x 10bit. But the performance needs to be validated with the reduced range.

# Conclusion

In this contribution, we have presented the results of our cross-check of JCTVC-Q0072. The implemented algorithm is in line with the proponent’s description, and the simulation results also match those provided by the proponents.

# References

- A. Duenas, P. Andrivon, E. Alshina, Y. Ye, K. Ugur, X. Li, “HEVC Scalable Extensions Core Experiment SCE1 description: Colour Gamut and Bit Depth Scalability”, JCTVC-P1101, Jan. 2014, US.