| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Valencia, ES, 27 March – 4 April 2014 | Document: JCTVC-Q0098 |
| --- | --- |

| Title: | Non-RCE1: Crosscheck report on MV resolution and motion vector predictor number (JCTVC-Q0067) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xiang Li | Tel: Email: | +1 858 658 3923 lxiang@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution reports the crosschecking results for JCTVC-Q0067 test 2 on PU level adaptive MV resolution and one AMVP predictor. The simulation results reportedly matched those provided by the proponents.

# Introduction

In JCTVC-Q0067, a flag pu_imv_flag is transmitted for each Inter/AMVP mode PU. When this flag is on, the integer precision is enabled for current Inter/AMVP PU. The motion vector predictors are rounded to the integer precision before motion compensation. Except when the mvd_l1_zero_flag is set equal to 1, the motion predictors of L1 for a bi-directional Inter mode are not rounded. When the flag pu_imv_flag is equal to 1, the motion vector differences are signaled in integer precision.

In addition, when the pu_imv_flag is set to 1, the number of predictors for the current Inter/AMVP PU is set equal to 1 and no predictor index (mvp_l0_flag, mvp_l1_flag) is coded.

To improve the encoding speed, fast estimation algorithm is also proposed. After the motion estimation of the Inter/AMVP PU for the quarter-pel estimation, a second motion estimation for integer precision only is carried out and this second estimation is performed with only one predictor. In addition, to limit the number of motion estimations when the first motion estimation leads to a motion vector equal to (0, 0), the second motion estimation is not performed and pu_imv_flag is set equal to 0.

# Experimental results

We received the source code from the proponents, implemented in RExt-6.0, and did a code study to verify that the proposed method was implemented as described. We ran simulations according to the RCE 1 description [1]. The results matched what were provided by the proponents and are summarized as follows

## Test 2 of lossy case

## Test 2 of lossless case

# Conclusion

In this contribution, we have presented the results of our cross-check of JCTVC-Q0067. The implemented algorithm is in line with the proponent’s description, and the simulation results also match those provided by the proponents.

# References

- X. Li, J. Xu, Y. Zhou, “HEVC Range Extensions Core Experiment 1 (RCE1)”, JCTVC-P1121, Jan. 2014, US.