| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Valencia, ES, 27 March – 4 April 2014 | Document: JCTVC-Q0099 |
| --- | --- |

| Title: | RCE1: Crosscheck report on Motion Vector Resolution Control (JCTVC-Q0155) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xiang Li | Tel: Email: | +1 858 658 3923 lxiang@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution reports the crosschecking results for JCTVC-Q0155 on motion vector resolution control. The simulation results reportedly matched those provided by the proponents.

# Introduction

In JCTVC-Q0072, it is proposed to add a high-level indicator (at the SPS, PPS, and/or slice header level) to indicate the resolution for interpretation of the motion vectors.

Before encoding a slice, encoder may decide the motion vector resolution based on historical statistical data, knowledge of its type of application, multi-pass analysis, or some other such technique. If the encoder decides to use full pixel motion only, the fractional pixel search is skipped. If a scaled motion vector prediction has a fractional part, the prediction is rounded to an integer value.

At decoder side, if the motion vectors are indicated to be at full pixel resolution and a scaled motion vector prediction has a fractional part, the prediction is rounded to an integer value. Motion vector differences are simply interpreted as integer offsets rather than 1/4-sample offsets. All other decoding processes remain same. The parsing process (below the header level) is unchanged. When the motion vectors are coded at full-sample precision and the video uses 4:2:2 or 4:2:0 sampling, the chroma motion vectors can be derived in the usual manner, which will produce 1/2-sample chroma motion displacements. Alternatively, it may be worth considering also rounding the chroma motion.

# Experimental results

We received the source code from the proponents, implemented in RExt-6.0, and did a code study to verify that the proposed method was implemented as described. We ran simulations according to the RCE description [1]. The results matched what were provided by the proponents and are summarized as follows

## Fast method for lossy case

## Fast method for lossless case

# Conclusion

In this contribution, we have presented the results of our cross-check of JCTVC-Q0155. The implemented algorithm is in line with the proponent’s description, and the simulation results also match those provided by the proponents.

# References

- X. Li, J. Xu, Y. Zhou, “HEVC Range Extensions Core Experiment 1 (RCE1)”, JCTVC-P1121, Jan. 2014, US.