| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Valencia, ES, 27 March – 4 April 2014 | Document: JCTVC-Q0102-r1 |
| --- | --- |

| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 8th Meeting: Valencia, ES, 29 March – 4 April 2014 | Document: JCT3V-H0033-r1 |
| --- | --- |

| Title: | Comments on HEVC Extensions |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC and JCT-3V |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Sachin Deshpande 5750 Pacific Rim Blvd, Camas, WA 98607, USA. | Tel: Email: | +1 360 817 8486 sdeshpande@sharplabs.com |
| Source: | Sharp Labs of America, Inc. |  |  |

_____________________________

# Abstract

In this document following is proposed:

- Proposal 1: A modification to the derivation of number of sub-DPBs and assignment of sub-DPBs to each layer considering separate_colour_plane_vps_flag is proposed.

- Proposal 2: A semantics change is proposed for sps_max_dec_pic_buffering_minus1 as a bug-fix.

- Proposal 3: It is proposed to signal the bit rate and picture rate information only up to the maximum temporal sub-layers in the corresponding layer set.

- In r1 revision following proposal 4 is added:

- Proposal 4: A modification to the derivation of numRefLayerPics, and refLayerPicIdc[ i ] to handle discardable pictures is proposed.In r2 revision

- In r2 revision updated specification text is provided in accompanying file for proposal 3 based on feedback from HLS BoG.

# Introduction

This document proposes comments on JCTVC-P1008 [1] and JCT3V-G1004 [2]. The following is proposed:

- In proposal 1 a modification to the derivation of number of sub-DPBs and assignment of sub-DPBs to each layer considering separate_colour_plane_vps_flag is proposed. It is asserted that this modification makes the derivation and assignment process for sub-DPBs complete and consistent with representation format signalling.

- In proposal 2 a semantics change is proposed for sps_max_dec_pic_buffering_minus1 as a bug-fix.

- In proposal 3 it is proposed to signal the bit rate and picture rate information for subsets only up to the maximum temporal sub-layers in the corresponding layer set.

# Proposal

This section provides motivation and draft text changes for various proposals.

Proposal 1:

In JCTVC-P1008 [1] and JCT3V-G1004 [2], the DPB parameters are signalled in dpb_size() syntax structure in Video Parameter Set (VPS). dpb_size() signals various DPB parameters for output layer sets for number of temporal sub-layers for number of sub-DPBs and for number of layers when number of sub-DPBs is different than number of layers. In F.7.4.3.1.3 DPB Size Semantics, for each layer set specified by the VPS, derivation of the number of sub-DPBs and assigning a sub-DPB to each layer included in the layer set is defined.

A modification to the derivation of number of sub-DPBs and assignment of sub-DPBs to each layer considering separate_colour_plane_vps_flag is proposed. It is asserted that this modification makes the derivation and assignment process for sub-DPBs complete and consistent with representation format signalling.

At high-level the modified derivation process utilizes information about separate_colour_plane_vps_flag of each layer as signalled in the representation format rep_format() in VPS extension. A sub-DPB is only shared between layers which have the same spatial resolution, same bit-depth for luma and chroma, same chroma_format_vps_idc value and same separate_colour_plane_vps_flag.

Without the proposed modification a sub-DPB is also shared between layers with different values of separate_colour_plane_vps_flag.

When separate_colour_plane_vps_flag (or separate_colour_plane_flag) is equal to 1 the three colour components are coded separately and the coded picture consists of three separate components, each of which consists of coded samples of one colour plane (Y, Cb, or Cr) and uses the monochrome coding syntax. In this case, each colour plane is associated with a specific colour_plane_id value. When separate_colour_plane_flag is equal to 0 the colour components are not coded separately. It is asserted that since the decoding process invocation is different when separate_colour_plane_flag is equal to 1 compared to when it is equal to 0, it is a better design choice to not share sub-DPBs between a layer which has  separate_colour_plane_vps_flag equal to 1 and another layer which has separate_colour_plane_vps_flag equal to 0.

Proposed changes related to the proposal 1 are highlighted below compared to JCTVC-P1008 [1].

- F.7.4.3.1.3 DPB size semantics

- The variable MaxSubLayersInLayerSetMinus1[ i ] is derived as follows:

for( i = 1; i < NumOutputLayerSets; i++ ) {
	maxSLMinus1 = 0
	optLsIdx = LayerSetIdxForOutputLayerSet[ i ]
	for( k = 0; k < NumLayersInIdList[ optLsIdx ]; k++ ) {
		lId = LayerSetLayerIdList[ optLsIdx ][ k ]	(F-1)
		maxSLMinus1 =Max( maxSLMinus1, sub_layers_vps_max_minus1[ LayerIdxInVps[ lId ] ] )
	}
	MaxSubLayersInLayerSetMinus1[ i ] = maxSLMinus1
}

For each layer set specified by the VPS, the following applies for derivation of the number of sub-DPBs and assigning a sub-DPB to each layer included in the layer set:

–	Let lsIdx be equal to the layer set index of the layer set.

–	For i from 0 to NumLayersInIdList[ lsIdx ] − 1, inclusive, the arrays layerSpatRes, layerColourFormat, and layerBitDepthId are derived as follows:

–	The entries layerSpatRes[ i ][ 0 ] and layerSpatRes[ i ][ 1 ] are set equal to the values of pic_width_vps_in_luma_samples and pic_height_vps_in_luma_samples, respectively, of the vps_rep_format_idx[ LayerIdxInVps[ LayerSetLayerIdList[ lsIdx ][ i ] ] ]-th rep_format( ) syntax structure in the VPS.

- The entry layerColourFormat[ i ] is set equal to the value of chroma_format_vps_idc of the vps_rep_format_idx[ LayerIdxInVps[ LayerSetLayerIdList[ lsIdx ][ i ] ] ]-th rep_format( ) syntax structure in the VPS.

- The entry layerSepColourPlane[ i ] is set equal to the value of separate_colour_plane_vps_flag of the vps_rep_format_idx[ LayerIdxInVps[ LayerSetLayerIdList[ lsIdx ][ i ] ] ]-th rep_format( ) syntax structure in the VPS if chroma_format_vps_idc of the vps_rep_format_idx[ LayerIdxInVps[ LayerSetLayerIdList[ lsIdx ][ i ] ] ]-th rep_format( ) syntax structure in the VPS is equal to 3. Otherwise the entry layerSepColourPlane[ i ] is set equal to 0.

–	The entries layerBitDepth[ i ][ 0 ] and layerBitDepth[ i ][ 1 ] are set equal to the values of bit_depth_vps_luma_minus8 and bit_depth_vps_chroma_minus8, respectively, of the vps_rep_format_idx[ LayerIdxInVps[ LayerSetLayerIdList[ lsIdx ][ i ] ] ]-th rep_format( ) syntax structure in the VPS.

–	The following applies:

subDpbCtr = 1
SubDpbAssigned[ lsIdx ][ 0 ] = 0
subDpbSpatRes[ 0 ][ 0 ] = layerSpatRes[ 0 ][ 0 ]
subDpbSpatRes[ 0 ][ 1 ] = layerSpatRes[ 0 ][ 1 ]
subDpbColourFormat[ 0 ] = layerColourFormat[ 0 ] 
subDpbSepColourPlane[0] = layerSepColourPlane[0];
subDpbBitDepth[ 0 ][ 0 ] = layerBitDepth[ 0 ][ 0 ]
subDpbBitDepth[ 0 ][ 1 ] = layerBitDepth[ 0 ][ 1 ]

for( i = 1; i < NumLayersInIdList[ lsIdx ]; i++ ) {
	newSubDpbFlag = 1
	for( j = 0; j < subDpbCtr  &&  bNewSubDpb( newSubDpbFlag > 0 ); j++ )
		if( layerSpatRes[ i ][ 0 ]  = =  subDpbSpatRes[ j ][ 0 ]  &&
		     layerSpatRes[ i ][ 1 ]  = =  subDpbSpatRes[ j ][ 1 ]  &&
		     layerColourFormat[ i ]  = =  subDpbColourFormat[ j ]  &&
		     layerSepColourPlane[ i ]  = =  subDpbSepColourPlane[ j ]  &&
		     layerBitDepth[ i ][ 0 ]  = =  subDpbBitDepth[ j ][ 0 ]  &&
		     layerBitDepth[ i ][ 1 ]  = =  subDpbBitDepth[ j ][ 1 ] ) {
			SubDpbAssigned[ lsIdx ][ i ] = j
			newSubDpbFlag = 0
			j = subDpbCtr
		}
	if( newSubDpbFlag ) {
		subDpbSpatRes[ subDpbCtr ][ 0 ] = layerSpatRes[ i ][ 0 ]
		subDpbSpatRes[ subDpbCtr ][ 1 ] = layerSpatRes[ i ][ 1 ]
		subDpbColourFormat[ subDpbCtr ] = layerColourFormat[ i ] 
		subDpbSepColourPlane[ subDpbCtr ] = layerSepColourPlane[ i ]
		subDpbBitDepth[ subDpbCtr ][ 0 ] = layerBitDepth[ i ][ 0 ]
		subDpbBitDepth[ subDpbCtr ][ 1 ] = layerBitDepth[ i ][ 1 ]
		SubDpbAssigned[ lsIdx ][ i ] = subDpbCtr++
	}
}
NumSubDpbs[ lsIdx ] = subDpbCtr

For the lsIdx-th layer set, the number of sub-DPBs is NumSubDpbs[ lsIdx ], and for each layer with a particular value of nuh_layer_id in the layer set, the sub-DPB with index SubDpbAssigned[ lsIdx ][ layerIdx ] is assigned, where LayerSetLayerIdList[ lsIdx ][ layerIdx ] is equal to nuh_layer_id.

Proposal 2:

A semantics change is proposed for sps_max_dec_pic_buffering_minus1 as a bug-fix. max_vps_dec_pic_buffering_minus1 describe the parameter for (shared) sub-DPB where as max_vps_layer_dec_pic_buff_minus1 describe the parameter for layer. Since NumSubDpbs[ LayerSetIdxForOutputLayerSet[ i ] ]  may be less than NumLayersInIdList[ LayerSetIdxForOutputLayerSet[ i ] ] the following fix is necessary.

- sps_max_dec_pic_buffering_minus1[ i ] plus 1 specifies the maximum required size of the decoded picture buffer for the CVS in units of picture storage buffers when HighestTid is equal to i. The value of sps_max_dec_pic_buffering_minus1[ i ] shall be in the range of 0 to MaxDpbSize − 1 (as specified in subclause A.4), inclusive. When i is greater than 0, sps_max_dec_pic_buffering_minus1[ i ] shall be greater than or equal to sps_max_dec_pic_buffering_minus1[ i − 1 ]. The value of sps_max_dec_pic_buffering_minus1[ i ] shall be less than or equal to vps_max_dec_pic_buffering_minus1[ i ] for each value of i. When sps_max_dec_pic_buffering_minus1[ i ] is not present for i in the range of 0 to sps_max_sub_layers_minus1 − 1, inclusive, due to sps_sub_layer_ordering_info_present_flag being equal to 0, it is inferred to be equal to sps_max_dec_pic_buffering_minus1[ sps_max_sub_layers_minus1 ]. When sps_max_dec_pic_buffering_minus1[ i ] is not present for i in the range of 0 to sps_max_sub_layers_minus1, inclusive, due to nuh_layer_id being greater than 0, it is inferred to be equal to max_vps_dec_pic_buffering_minus1 max_vps_layer_dec_pic_buff_minus1 [ TargetOptLayerSetIdx ][ currLayerId ][ i ] of the active VPS, where currLayerId is the nuh_layer_id of the layer that refers to the SPS.

Alternatively the bug can be fixed as follows:

- sps_max_dec_pic_buffering_minus1[ i ] plus 1 specifies the maximum required size of the decoded picture buffer for the CVS in units of picture storage buffers when HighestTid is equal to i. The value of sps_max_dec_pic_buffering_minus1[ i ] shall be in the range of 0 to MaxDpbSize − 1 (as specified in subclause A.4), inclusive. When i is greater than 0, sps_max_dec_pic_buffering_minus1[ i ] shall be greater than or equal to sps_max_dec_pic_buffering_minus1[ i − 1 ]. The value of sps_max_dec_pic_buffering_minus1[ i ] shall be less than or equal to vps_max_dec_pic_buffering_minus1[ i ] for each value of i. When sps_max_dec_pic_buffering_minus1[ i ] is not present for i in the range of 0 to sps_max_sub_layers_minus1 − 1, inclusive, due to sps_sub_layer_ordering_info_present_flag being equal to 0, it is inferred to be equal to sps_max_dec_pic_buffering_minus1[ sps_max_sub_layers_minus1 ]. When sps_max_dec_pic_buffering_minus1[ i ] is not present for i in the range of 0 to sps_max_sub_layers_minus1, inclusive, due to nuh_layer_id being greater than 0, it is inferred to be equal to max_vps_dec_pic_buffering_minus1 [ TargetOptLayerSetIdx ][ SubDpbAssigned[ LayerSetIdxForOutputLayerSet[ TargetOutputLayerSetIdx ] ][ currLayerId ] ][ i ] of the active VPS, where currLayerId is the nuh_layer_id of the layer that refers to the SPS.

Proposal 3:

It is asserted that in SHVC different layers may have different frame rates. As a result a layer with a higher frame rate may have a higher value of maximum temporal sub-layers compared to a layer with a lower frame-rate. The j-th subset of a layer set is the output of the sub-bitstream extraction process when it is invoked with the layer set, j, and the layer identifier list associated with the layer set as inputs. In mixed frame rate case for certain layer sets the maximum number of temporal sub-layers in the layer set may be less than vps_max_sub_layers_minus1. In this case some of the (vps_max_sub_layers_minus1 + 1) subsets of such a layer set will be identical. It is asserted that it is wasteful to signal bitrate and picture information for these identical subsets. Information regarding maximum number of temporal sub-layers for a layer (sub_layers_vps_max_minus1) is already signalled in VPS. Thus it is proposed to signal the bit rate and picture rate information for subsets only up to the maximum temporal sub-layers in the corresponding layer set.

Proposed changes related to the proposal 3 are highlighted below compared to JCTVC-P1008 [1].

- F.7.3.2.1.4  VPS VUI Syntax

| vps_vui( ){ | Descriptor |
| --- | --- |
| cross_layer_pic_type_aligned_flag | u(1) |
| if( !cross_layer_pic_type_aligned_flag ) |  |
| cross_layer_irap_aligned_flag | u(1) |
| if( cross_layer_irap_aligned_flag ) |  |
| all_layers_idr_aligned_flag | u(1) |
| bit_rate_present_vps_flag | u(1) |
| pic_rate_present_vps_flag | u(1) |
| if( bit_rate_present_vps_flag ¦ ¦ pic_rate_present_vps_flag ) |  |
| for( i = 0; i <= vps_number_layer_sets_minus1; i++ ) |  |
| for( j = 0; j <= vps_max_sub_layers_minus1MaxSlLayerSetMinus1[ i ]; j++ ) { |  |
| if( bit_rate_present_vps_flag ) |  |
| bit_rate_present_flag[ i ][ j ] | u(1) |
| if( pic_rate_present_vps_flag ) |  |
| pic_rate_present_flag[ i ][ j ] | u(1) |
| if( bit_rate_present_flag[ i ][ j ] ) { |  |
| avg_bit_rate[ i ][ j ] | u(16) |
| max_bit_rate[ i ][ j ] | u(16) |
| } |  |
| if( pic_rate_present_flag[ i ][ j ] ) { |  |
| constant_pic_rate_idc[ i ][ j ] | u(2) |
| avg_pic_rate[ i ][ j ] | u(16) |
| } |  |
| } |  |
| video_signal_info_idx_present_flag | u(1) |
| … |  |
| } |  |

- F.7.4.3.1.4 VPS VUI semantics

…

The variable MaxSlLayersetMinus1[ i ] is derived as follows :

for( i = 0; i <= vps_number_layer_sets_minus1; i++ ) {

mSlMinus1 = 0

for( k = 0; k < NumLayersInIdList[ i ]; k++ ) {

liId = LayerSetLayerIdList[ i ][ k ]

mSlMinus1 =Max(mSlMinus1, 	sub_layers_vps_max_minus1[ LayerIdxInVps[ liId ] ]);

}

MaxSlLayersetMinus1[ i ] = mSlMinus1

}

bit_rate_present_vps_flag equal to 1 specifies that the syntax element bit_rate_present_flag[ i ][ j ] is present. bit_rate_present_vps_flag equal to 0 specifies that the syntax element bit_rate_present_flag[ i ][ j ] is not present.

- …

Proposal 4:

In certain situations when all_ref_layers_active_flag is equal to 1 discardable pictures could not be used correctly under certain prediction structures. Consider an example of a layered coding system consisting of three layers (layer 0 and layer 1 and layer 2) where layer 1 is using layer 0 as a reference layer and layer 2 is using layer 1 as a reference layer and all_ref_layers_active_flag is equal to 1. Then if an AU has a picture in layer 1 which has discardable_flag equal to 1, then according to the current derivation process for layer 2 for numRefLayerPics, and refLayerPicIdc[ i ] the discardable picture will be included in numRefLayerPics and thus its reference layer will be included in refLayerPicIdc. For example this may happen if the following condition is satisfied for reference layer which has picture with discardable_flag equal to 1: (sub_layers_vps_max_minus1[ refLayerIdx ] >= TemporalId  && max_tid_il_ref_pics_plus1[ refLayerIdx ][ LayerIdxInVps[ nuh_layer_id ] ] > TemporalId)

This will violate the constraint for layer 2 that: there shall be no picture that has discardable_flag equal to 1 in RefPicSetInterLayer0 or RefPicSetInterLayer1.  To avoid this problem the derivation of numRefLayerPics, and refLayerPicIdc[ i ] is modified to handle discardable pictures.

It is asserted that the proposal 4 is complementary to the problem reported and solved in JCTVC-Q0177 section 3 and that it is asserted that both the solutions are needed to completely solve the problem.

Proposed changes related to the proposal 4 are highlighted below compared to JCTVC-P1008 [1].

all_ref_layers_active_flag equal to 1 specifies that for each picture referring to the VPS, the reference layer pictures that belong to all direct reference layers of the layer containing the picture and that might be used for inter-layer prediction as specified by the values of sub_layers_vps_max_minus1[ i ], and max_tid_il_ref_pics_plus1[ i ][ j ] and DiscardableFlag[ i ] are present in the same access unit as the picture and are included in the inter-layer reference picture set of the picture. all_ref_layers_active_flag equal to 0 specifies that the above restriction may or may not apply.

…

discardable_flag equal to 1 specifies that the coded picture is not used as a reference picture for inter prediction and is not used as an inter-layer reference picture in the decoding process of subsequent pictures in decoding order. discardable_flag equal to 0 specifies that the coded picture may be used as a reference picture for inter prediction and may be used as an inter-layer reference picture in the decoding process of subsequent pictures in decoding order. When not present, the value of discardable_flag is inferred to be equal to 0.

When nal_unit_type is equal to TRAIL_R, TSA_R, STSA_R, RASL_R or RADL_R, the value of discardable_flag shall be equal to 0.

The variable DiscardableFlag[ nuh_layer_id ] is derived as follows:

DiscardableFlag[ nuh_layer_id ] = discardable_flag

…

num_inter_layer_ref_pics_minus1 plus 1 specifies the number of pictures that may be used in decoding of the current picture for inter-layer prediction. The length of the num_inter_layer_ref_pics_minus1 syntax element is Ceil( Log2( NumDirectRefLayers[ nuh_layer_id ] ) ) bits. The value of num_inter_layer_ref_pics_minus1 shall be in the range of 0 to NumDirectRefLayers[ nuh_layer_id ] − 1, inclusive.

The variables numRefLayerPics and refLayerPicFlag[ i ]  and refLayerPicIdc[ j ] are derived as follows:

for( i = 0, j = 0;  i < NumDirectRefLayers[ nuh_layer_id ]; i++ ) {
	refLayerIdx = LayerIdxInVps[ RefLayerId[ nuh_layer_id ][ i ] ]
	if( sub_layers_vps_max_minus1[ refLayerIdx ] >= TemporalId  && 
			max_tid_il_ref_pics_plus1[ refLayerIdx ][ LayerIdxInVps[ nuh_layer_id ] ] > TemporalId  
		 && !DiscardableFlag[ refLayerIdx ])
		refLayerPicIdc[ j++ ] = i
}

The variable NumActiveRefLayerPics is derived as follows:

if( nuh_layer_id  = =  0  | |  NumDirectRefLayers[ nuh_layer_id ]  = =  0 )
	NumActiveRefLayerPics = 0
else if( all_ref_layers_active_flag )
	NumActiveRefLayerPics = numRefLayerPics
else if( !inter_layer_pred_enabled_flag )
	NumActiveRefLayerPics = 0
else if( max_one_active_ref_layer_flag  | |  NumDirectRefLayers[ nuh_layer_id ]  = = 1 )
	NumActiveRefLayerPics = 1
else
	NumActiveRefLayerPics = num_inter_layer_ref_pics_minus1 + 1

All slices of a coded picture shall have the same value of NumActiveRefLayerPics.

# Conclusion

In this document following is proposed:

- Proposal 1: A modification to the derivation of number of sub-DPBs and assignment of sub-DPBs to each layer considering separate_colour_plane_vps_flag is proposed.

- Proposal 2: A semantics change is proposed for sps_max_dec_pic_buffering_minus1 as a bug-fix.

- Proposal 3: It is proposed to signal the bit rate and picture rate information for subsets only up to the maximum temporal sub-layers in the corresponding layer set.

- Proposal 4: A modification to the derivation of numRefLayerPics, and refLayerPicIdc[ i ] to handle discardable pictures is proposed

We propose to adopt the proposals in the draft of SHVC and MV-HEVC.

# Patent rights declaration(s)

Sharp may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

[1] J. Chen, J. Boyce, Y. Ye, M.M. Hannuksela, Y.-K. Wang, “High Efficiency Video Coding (HEVC) Scalable Extension Draft 5,” JCTVC-P1008, San Jose, January 2014.

[2] G. Tech, K. Wegner, Y. Chen, M. Hannuksela, J. Boyce, “MV-HEVC Draft Text 7,” JCT3V-G1004, San Jose, January 2014.