| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Valencia, ES, 27 March – 4 April 2014 | Document: JCTVC-Q0107 |
| --- | --- |

| Title: | SHVC HLS: On picture level resampling phase filter selection |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yan Ye, Yong He, Yuwen He 9710 Scranton Rd, #250 San Diego, CA 92121 USA | Tel: Email: | +1-858-210-4803 yan.ye@interdigital.com Yong.he@interdigital.com Yuwen.he@interdigital.com |
| Source: | InterDigital Communications LLC |  |  |

_____________________________

# Abstract

At the 16th JCTVC meeting, a few flags for the purpose of vertical resampling phase adjustment were adopted into the SHVC draft 6. In combination, these flags allow the resampling phase filters to be adjusted at the picture level. One important use case to which vertical resampling phase adjustment applies is the case of interlaced to progressive scalability. In this contribution, alternative signaling methods for resampling phase filter selection are proposed. It is asserted that the proposed signaling methods harmonize all syntax elements related to resampling phase filter selection, and that they provide more flexibility than SHVC draft 6.

# Introduction

At the 16th JCTVC meeting in San Jose, a set of flags were adopted into the SHVC draft 6 to enable interlaced to progressive scalability:

- In sps_multilayer_extension(), vert_phase_position_enable_flag is added to indicate whether picture by picture adjustment of vertical phase filters is allowed for a given reference layer in the CVS;

- In slice_segment_header(), vertical_phase_position_flag is added to toggle between two possibilities of vertical phase adjustments. This flag is signaled for a given reference layer only if vert_phase_position_enable_flag is equal to 1 for that given reference layer.

- In VPS VUI, vert_phase_position_in_use_flag is added to indicate whether any picture-level vertical phase adjustment is allowed in any layer in the scalable bitstream. This flag is placed in VPS VUI as it is for information only and does not mandate non-normative behavior.

Together these flags enable good scalable coding efficiency in the case of interlaced to progressive scalability. However, in the current SHVC draft, syntax elements related to resampling phase filter selection are not harmonized:

- Different syntax elements related to resampling phase selection are placed at different levels. Specifically, cross_layer_phase_alignment_flag, which is used to select resampling phase filters based on whether the downsampling filters are top-left-aligned or center-aligned, is sent in the VPS, whereas vert_phase_position_enable_flag is sent in SPS multilayer extension per reference layer;

- In SPS multilayer extension, per reference layer signaling of the scaled reference offsets is tied together with per reference layer signaling of vert_phase_position_enable_flag, although these syntax elements are not necessarily related;

- In slice segment header, vert_phase_position_flag is signaled for all reference layers for which vert_phase_position_enable_flag is equal to 1. However, all current SHVC profiles mandate that, for any picture in the scalable bitstream, the resampling process can be applied at most once. This increases the signaling overhead of slice header unnecessarily;

- vert_phase_position_flag can’t be combined with cross_layer_phase_alignment_flag. This can cause sub-optimal coding efficiency when spatial scalability is combined with interlaced to progressive scalability.

- The current phase derivation process for chroma assumes that the default chroma sample location type (type 0) is used. When non-default chroma sample location is used, coding efficiency may be slightly affected. This was discussed at the last meeting in [1].

In this contribution, two options are provided to improve the syntax signaling flexibility and to harmonize current syntax elements related to resampling phase filter selection.

# Proposed signaling of resampling phase filters

The resampling filters used in the decoding process are selected based on the 4 LSBs of the variables xRef16 and yRef16, which are derived in H.6.2:

The variables phaseX, phaseY, addX and addY are derived as follows:

phaseX = ( cIdx  = =  0 ) ? ( cross_layer_phase_alignment_flag << 1 ) : cross_layer_phase_alignment_flag	(H-1)
phaseY = VertPhasePositionAdjustFlag ? ( VertPhasePositionFlag << 2 ) :
( ( cIdx  = =  0 ) ? ( cross_layer_phase_alignment_flag << 1 ) : cross_layer_phase_alignment_flag + 1)	(H-2)

addX = ( ScaleFactorX * phaseX + 2 ) >> 2		(H-3)
addY = ( ScaleFactorY * phaseY + 2 ) >> 2		(H-4)

The variables xRef16 and yRef16 are derived as follows:

xRef16 = ( ( ( xP − offsetX ) * ScaleFactorX + addX + ( 1 << 11 ) ) >> 12 ) − ( phaseX << 2 )	(H-5)
yRef16 = ( ( ( yP − offsetY ) * ScaleFactorY + addY + ( 1 << 11 ) ) >> 12 ) − ( phaseY << 2 )	(H-6)

Let us set phaseXLuma, phaseYLuma to phaseX and phaseY in (H-5) and (H-6) when cIdx is 0, and set phaseXChroma, phaseYChroma to phaseX and phaseY in (H-5) and (H-6) when cIdx is non-zero. It is proposed to directly signal the set of phaseXLuma, phaseYLuma, phaseXChroma, and phaseYChroma values, instead of deriving phaseX and phaseY using the current flags as in (H-5) and (H-6). Table 1 compares the current syntax (3 flags) and the proposed syntax (4 phase values) for a few different use cases. The last use case (non-default chroma sample location type) is not enabled with the current syntax.

Table 1. Comparison between the current syntax and the proposed syntax

|  | Current syntax |  |  | Proposed syntax |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Cross_layer_phase_alignment_flag (VPS) | vert_phase_position_enable_flag (SPS extension) | vert_phase_position_flag (slice header) | phaseXLuma | phaseYLuma | phaseXChroma | phaseYChroma |
| Top-left aligned DS filter | 0 | 0 | - | 0 | 0 | 0 | 1 |
| Center aligned DS filter | 1 | 0 | - | 2 | 2 | 1 | 2 |
| top field to frame | 0 | 1 | 0 | 0 | 0 | 0 | 0 |
| bottom field to frame | 0 | 1 | 1 | 0 | 4 | 0 | 4 |
| chroma sample location type-2 |  |  |  | 0 | 0 | 0 | 0 |

It is proposed to signal phaseXLuma, phaseYLuma, phaseXChroma and phaseYChroma in the PPS. Two options are provided in the next section:

- Signal at most one set of phaseXLuma, phaseYLuma, phaseXChroma and phaseYChroma in the PPS. This requires no change to the slice header, as the pps_id in slice header can be used to identify the phase set. However, this can require more PPSs and increase the pps_id signaling overhead in slice header. This option also requires the constraint at most one resampling per picture to be applied to all SHVC profiles.

- Signal multiple sets of phaseXLuma, phaseYLuma, phaseXChroma and phaseYChroma in the PPS. This requires adding an index in the slice header to identify which set of phaseXLuma, phaseYLuma, phaseXChroma and phaseYChroma will be used for decoding of the current slice. But this option allows any future SHVC profile to loosen the constraint of at most one resampling per picture, if needed. It is also more efficient as there is no increase in the pps_id signaling overhead.

# Proposed text change

## Proposed text change for option 1: signaling at most one phase set in PPS

### Proposed syntax and semantics

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| pps_pic_parameter_set_id | ue(v) |
| pps_seq_parameter_set_id | ue(v) |
| … |  |
| pps_extension_flag | u(1) |
| if( pps_extension_flag ) { |  |
| for ( i = 0; i < 8; i++ ) |  |
| pps_extension_type_flag[ i ] | u(1) |
| if( pps_extension_type_flag[ 0 ] ) { |  |
| poc_reset_info_present_flag | u(1) |
| resample_phase_set_present_flag | u(1) |
| if ( resample_phase_set_present_flag ) { |  |
| resample_phase_x_luma | u(3) |
| resample_phase_y_luma | u(3) |
| resample_phase_x_chroma | u(3) |
| resample_phase_y_chroma | u(3) |
| } |  |
| } |  |
| if( pps_extension_type_flag[ 7 ] ) |  |
| while( more_rbsp_data( ) ) |  |
| pps_extension_data_flag | u(1) |
| } |  |
| rbsp_trailing_bits( ) |  |
| } |  |

resample_phase_set_present_flag equal to 1 specifies that resample_phase_x_luma, resample_phase_y_luma, resample_phase_x_chroma, and resample_phase_y_chroma are present in the PPS RBSP syntax structure. resample_phase_set_present_flag equal to 0 specifies that resample_phase_x_luma, resample_phase_y_luma, resample_phase_x_chroma, and resample_phase_y_chroma are not present in the PPS RBSP syntax structure.

resample_phase_x_luma specifies the horizontal resampling phase adjustment value used to select the luma resampling filter in the horizontal direction. When resample_phase_x_luma is not present, it is inferred to be equal to 0.

resample_phase_y_luma specifies the vertical resampling phase adjustment value used to select the luma resampling filter in the vertical direction. When resample_phase_y_luma is not present, it is inferred to be equal to 0.

resample_phase_x_chroma specifies the horizontal resampling phase adjustment value used to select the chroma resampling filter in the horizontal direction. When resample_phase_x_chroma is not present, it is inferred to be equal to 0.

resample_phase_y_chroma specifies the vertical resampling phase adjustment value used to select the chroma resampling filter in the vertical direction. When resample_phase_y_chroma is not present, it is inferred to be equal to 1.

### Proposed decoding process change

The change to decoding process is very simple, where the existing equations (H-5) and (H-6) in SHVC draft 6 is replaced with the following:

phaseX = ( cIdx  = =  0 ) ? resample_phase_x_luma : resample_phase_x_chroma	(H-7)
phaseY = ( cIdx  = =  0 ) ? resample_phase_y_luma : resample_phase_y_chroma	(H-8)

## Proposed text change for option 2: signaling multiple phase sets in PPS

### Proposed syntax and semantics in PPS

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| pps_pic_parameter_set_id | ue(v) |
| pps_seq_parameter_set_id | ue(v) |
| … |  |
| pps_extension_flag | u(1) |
| if( pps_extension_flag ) { |  |
| for ( i = 0; i < 8; i++ ) |  |
| pps_extension_type_flag[ i ] | u(1) |
| if( pps_extension_type_flag[ 0 ] ) { |  |
| poc_reset_info_present_flag | u(1) |
| num_resample_phase_sets | ue(v) |
| for ( i = 0; i < num_resample_phase_sets; i ++ ) { |  |
| resample_phase_x_luma[ i ] | u(3) |
| resample_phase_y_luma[ i ] | u(3) |
| resample_phase_x_chroma[ i ] | u(3) |
| resample_phase_y_chroma[ i ] | u(3) |
| } |  |
| if( pps_extension_type_flag[ 7 ] ) |  |
| while( more_rbsp_data( ) ) |  |
| pps_extension_data_flag | u(1) |
| } |  |
| rbsp_trailing_bits( ) |  |
| } |  |

num_resample_phase_sets specifies the number of resample phase set syntax elements resample_phase_x_luma[ i ], resample_phase_y_luma[ i ], resample_phase_x_chroma[ i ], resample_phase_y_chroma[ i ].

resample_phase_x_luma[ i ] specifies the i-th horizontal resampling phase adjustment value used to select the luma resampling filter in the horizontal direction. When resample_phase_x_luma[ i ] is not present, it is inferred to be equal to 0.

resample_phase_y_luma[ i ] specifies the i-th vertical resampling phase adjustment value used to select the luma resampling filter in the vertical direction. When resample_phase_y_luma[ i ] is not present, it is inferred to be equal to 0.

resample_phase_x_chroma[ i ] specifies the i-th horizontal resampling phase adjustment value used to select the chroma resampling filter in the horizontal direction. When resample_phase_x_chroma[ i ] is not present, it is inferred to be equal to 0.

resample_phase_y_chroma[ i ] specifies the i-th vertical resampling phase adjustment value used to select the chroma resampling filter in the vertical direction. When resample_phase_x_chroma[ i ] is not present, it is inferred to be equal to 1.

### Proposed syntax and semantics in slice header

| slice_segment_header( ) { | Descriptor |
| --- | --- |
| … |  |
| if( nuh_layer_id > 0 && !all_ref_layers_active_flag && NumDirectRefLayers[ nuh_layer_id ] > 0 ) { |  |
| inter_layer_pred_enabled_flag | u(1) |
| if( inter_layer_pred_enabled_flag && NumDirectRefLayers[ nuh_layer_id ] > 1) { |  |
| if( !max_one_active_ref_layer_flag ) |  |
| num_inter_layer_ref_pics_minus1 | u(v) |
| if( NumActiveRefLayerPics != NumDirectRefLayers[ nuh_layer_id ] ) |  |
| for( i = 0; i < NumActiveRefLayerPics; i++ ) |  |
| inter_layer_pred_layer_idc[ i ] | u(v) |
| } |  |
| } |  |
| for( i = 0; i < NumActiveRefLayerPics; i++ ) |  |
| if ( vert_phase_position_enable_flag[ RefPicLayerId[ i ] ] ) |  |
| vert_phase_position_flag[ RefPicLayerId[ i ]] | u(1) |
| if( nuh_layer_id > 0 && num_resample_phase_sets > 1 ) { |  |
| for( i = 0; i < NumActiveRefLayerPics; i++ ) |  |
| if( ResampleFromRefLayerFlag[RefPicLayerId[ i ] ] ) |  |
| phase_set_idx[ RefPicLayerId[ i ] ] | u(v) |
| } |  |
| … |  |
| } |  |

phase_set_idx[ i ] specifies the phase set index used to resample the reference layer picture with nuh_layer_id equal to i for decoding of the current slice. When phase_set_idx is not present, it is inferred to be equal to 0. The value of phase_set_idx shall be in the range of 0 and num_resample_phase_sets – 1, inclusive.

### Proposed decoding process

For each reference layer, the variable ResampleFromRefLayerFlag is derived as follows:

for( i = 0; i <  NumActiveRefLayerPics; i++ ) {

refLayerId = RefPicLayerId[ i ]

rlPic is the decoded reference layer picture

refLayerPicWidthInSamplesY is set to the width of rlPic in units of luma samples

refLayerPicHeightInSamplesY is set to the height of rlPic in units of luma samples

refLayerBitDepthY is set to BitDepthY of rlPic

refLayerBitDepthC is set to BitDepthC of rlPic

if (pic_width_in_luma_samples == refLayerPicWidthInSamplesY &&

pic_height_in_luma_samples == refLayerPicHeightInSamplesY &&

scaled_ref_layer_left_offset[ refLayerId ] == 0 && scaled_ref_layer_left_offset[ refLayerId ] == 0 &&

scaled_ref_layer_left_offset[ refLayerId ] == 0 && scaled_ref_layer_left_offset[ refLayerId ] == 0 &&

refLayerBitDepthY == BitDepthY && refLayerBitDepthC == BitDepthC )

ResampleFromRefLayerFlag[refLayerId ] = 0

else

ResampleFromRefLayerFlag[refLayerId ] = 1

}

The process in H.8.1.4 is modified as follows:

ScaledRefLayerLeftOffset = scaled_ref_layer_left_offset[ rLId ] << 1	(H-9)
ScaledRefLayerTopOffset = scaled_ref_layer_top_offset[ rLId ]  << 1	(H-10)
ScaledRefLayerRightOffset = scaled_ref_layer_right_offset[ rLId  ]  << 1	(H-11)
ScaledRefLayerBottomOffset = scaled_ref_layer_bottom_offset[ rLId ]  << 1	(H-12)
VertPhasePositionAdjustFlag = vert_phase_position_enable_flag[ rLId ]	(H-13)
VertPhasePositionFlag = vert_phase_position_flag[ rLId ] 	(H-14)

PhaseSetIdx = phase_set_idx[ rLId ] 		(H-15)

The process in H.6.2 is modified as follows:

phaseX = ( cIdx  = =  0 ) ? resample_phase_x_luma[ PhaseSetIdx ] : resample_phase_x_chroma[ PhaseSetIdx ]	 (H-16)
phaseY = ( cIdx  = =  0 ) ? resample_phase_y_luma[ PhaseSetIdx ] : resample_phase_y_chroma[ PhaseSetIdx ]	 (H-17)

# References

- K. Rapaka, J. Chen, M. Karczewicz, AHG13: Chroma phase offset for SHVC resampling process. JCTVC-P0164. San Jose, Jan 2014.

# Patent rights declaration(s)

InterDigital Communications LLC may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).