| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Valencia, ES, 27 March – 4 April 2014 | Document: JCTVC-Q0108 |
| --- | --- |

| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 8th Meeting: Valencia, ES, 29 March – 4 April 2014 | Document: JCT3V-H0036 |
| --- | --- |

| Title: | MV-HEVC/SHVC HLS: On TSA and STSA pictures |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC and JCT-3V |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Miska M. Hannuksela Nokia Research Center Visiokatu 3, Tampere, Finland | Tel: Email: | +358 7180 08000 firstname.lastname@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

The contribution includes the following proposals:

- It is proposed to remove the constraints on aligning TSA and STSA pictures within an access unit due to the following asserted reasons:

  - The alignment constraints do not guarantee that sub-layer up-switching would be possible across all layers of the target output layer set, when a base-layer TSA or STSA picture at the desired switch-to sub-layer appears. In other words, layer-wise consideration of sub-layer up-switching is needed in any case.

  - Without the alignment constraints, it would be possible to adapt the highest transmitted TemporalId of the highest layer separately from other layers and hence achieve finer-grain bitrate adjustment.

  - Without the alignment constraints, reference pictures for inter prediction can be selected differently across layers, allowing some layers to be coded with more frequent sub-layer up-switch points and some layers with a larger number of reference pictures for inter prediction (aiming at better rate-distortion performance).

- It is proposed to allow STSA pictures with nuh_layer_id greater than 0 to have TemporalId equal to 0. Such STSA pictures enable layer up-switching where a subset of sub-layers of the highest layer can be decoded starting from the up-switch point. It is asserted that the proposed change does not affect the decoding process.

# Background

## Cross-layer alignment constraints of TSA and STSA pictures

MV-HEVC/SHVC currently requires alignment of TSA and STSA pictures towards the root of the layer tree as follows (in subclause F.7.4.2.2):

When one picture picA of a layer layerA has nal_unit_type equal to TSA_N or TSA_R, each picture in the same access unit as picA in a direct or indirect reference layer of layerA shall have nal_unit_type equal to TSA_N or TSA_R.

When one picture picA of a layer layerA has nal_unit_type equal to STSA_N or STSA_R, each picture in the same access unit as picA in a direct or indirect reference layer of layerA shall have nal_unit_type equal to STSA_N or STSA_R.

## Requirement of non-zero TemporalId for TSA and STSA pictures

HEVC version 1 requires TSA and STSA pictures to have TemporalId equal to 0 in subclause 7.4.2.2:

… Otherwise, when nal_unit_type is equal to TSA_R, TSA_N, STSA_R, or STSA_N, TemporalId shall not be equal to 0.

However, this constraint of TemporalId does not necessarily concern enhancement-layer pictures, because in HEVC version 1 bitstreams nuh_layer_id is required to be equal to 0.

# Proposal

## Removal of the cross-layer alignment constraints of TSA and STSA pictures

It is proposed to remove the alignment constraints of TSA and STSA pictures. In other words, the following change is proposed into subclause F.7.4.2.2:

When one picture picA of a layer layerA has nal_unit_type equal to TSA_N or TSA_R, each picture in the same access unit as picA in a direct or indirect reference layer of layerA shall have nal_unit_type equal to TSA_N or TSA_R.

When one picture picA of a layer layerA has nal_unit_type equal to STSA_N or STSA_R, each picture in the same access unit as picA in a direct or indirect reference layer of layerA shall have nal_unit_type equal to STSA_N or STSA_R.

## Allowing TemporalId equal to 0 for STSA pictures with nuh_layer_id greater than 0

It is proposed to modify subclause 7.4.2.2 as follows:

… Otherwise, when nal_unit_type is equal to TSA_R, TSA_N, STSA_R, or STSA_N, TemporalId shall not be equal to 0. Otherwise, if nal_unit_type is equal to TSA or TSA_N, TemporalId shall not be equal to 0. Otherwise, when nuh_layer_id is equal to 0 and nal_unit_type is equal to STSA_R or STSA_N, TemporalId shall not be equal to 0.

# Motivation

## Removal of the cross-layer alignment constraints of TSA and STSA pictures

The decision on the TSA and STSA alignment constraints was made in the JCTVC-N meeting as response to JCTVC-N0084. The motivation of the alignment constraints seems to have been that to enable temporal sub-layer up switching capability to be valid across layers. However, as the constraints apply only from an enhancement layer towards the root of the same layer tree, they do not guarantee that sub-layer up-switching would be possible across all layers at the target output layer set, when a base-layer TSA or STSA picture at the desired switch-to sub-layer appears. It is therefore questioned what functionality is achieved with the above constraints. Furthermore, it is asserted that that the constraints disable or limit the flexibility in the following use cases:

- Layer-wise decision of the maximum TemporalId of the pictures that are transmitted. When the enhancement layer contains more frequent TSA or STSA pictures than in the base layer, finer-grain bitrate adjustment can be performed than what can be achieved by determining the number of layers and/or the number sub-layers orthogonally.

It is remarked that when the alternative output layer mechanism is in use and there is no picture at the target output layer, a picture from the lower layer is to be output. Consequently, even if pictures from the target output layer are omitted from transmission, the output picture rate remains unchanged.

Bitrate adjustment may be used for example for providing so-called fast start-up in streaming services, where the bitrate of the transmitted stream is lower than the channel bitrate after starting or random-accessing the streaming in order to start playback immediately and to achieve a buffer occupancy level that tolerates occasional packet delays and/or retransmissions. Bitrate adjustment is also used when matching the transmitted stream bitrate with the prevailing channel throughput bitrate.

In this use case it is possible to use a greater number of reference pictures in the base layer to achieve better rate-distortion performance.

Figure 1 illustrates an example when the base layer (BL) has fewer TSA pictures than the enhancement layer (EL). It is remarked that some prediction arrows from TID0 pictures are not included in the illustration (but can be concluded from the DPB dump). The DPB dump considers pictures marked as "used for reference" but does not consider pictures marked as "needed for output" (which might have already been marked "unused for reference").

Figure 1. Example of EL having more frequent TSA pictures than the BL.

In Figure 2, it is assumed that it has been necessary to reduce the bitrate of the first GOP when the bitstream is transmitted. In this example, only the pictures with TemporalId (TID) equal to 0 are transmitted for the first GOP. It is further assumed that the bitstream can be transmitted at its full bitrate starting from the second GOP. As the second GOP in EL starts with TSA pictures, it is possible to start transmitting EL pictures with all TID values.

Figure 2. Omission of TID>0 from the first EL GOP.

- Transmitting only the sub-layers needed for inter-layer prediction from non-output layers, and stepwise increasing the number of transmitted sub-layers from output layers.

In this use case, it is assumed that pictures of only some TemporalId values are used as reference for inter-layer prediction as indicated by the max_tid_il_ref_pics_plus1 syntax element. It is further assumed that the sender knows that the receiver uses an output layer set, where only the EL is output. Consequently, the sender omits the transmission of BL pictures with a TemporalId value such that it is indicated not to be used as reference for inter-layer prediction.

It is further assumed that the sender performs bitrate adjustment by selecting adaptively the maximum TemporalId that is transmitted from the EL.

Figure 3 presents an example, which is similar to the example in Figure 1 but where BL pictures with TemporalId greater than or equal to 2 are not used as reference for inter-layer prediction, i.e. max_tid_il_ref_pics_plus between the base and enhancement layer is equal to 2.

Figure 3. Example of EL having more frequent TSA pictures than the BL, and where BL pictures up to TID 1 can be used as reference for inter-layer prediction.

In Figure 4, it is assumed that the sender is aware that the receiver uses an output layer set where only the enhancement layer is an output layer and hence the transmission of BL sub-layers that are not used as reference for inter-layer prediction is omitted. It is also assumed it has been necessary to reduce the bitrate of the first GOP when the bitstream is transmitted. In this example, the EL pictures with TemporalId in the range of 0 to 2, inclusive, are transmitted for the first GOP. It is further assumed that the bitstream can be transmitted at its full bitrate starting from the second GOP. As the second GOP in EL starts with TSA pictures, it is possible to start transmitting EL pictures with all TID values.

Figure 4. EL as target output layer, BL pictures not used as references for inter-layer prediction are not transmitted, highest TID is equal to 2 for the first EL GOP and equal to 3 for the second EL GOP.

## Allowing TemporalId equal to 0 for STSA pictures with nuh_layer_id greater than 0

It seems that no changes in the decoding process or HRD are needed when allowing STSA pictures with nuh_layer_id greater than 0 to have TemporalId equal to 0. An STSA picture with TemporalId equal to 0 can be used to access some sub-layers of the layer, while higher sub-layers may have inter prediction dependencies on pictures that precede the STSA picture in decoding order.

An example is presented in Figure 5. In this example there are no TSA or STSA pictures among the presented pictures (5, 7, 8, C, D, F, G) of TID 3 of the EL.

Figure 5. EL STSA picture with TemporalId equal to 0.

In Figure 6, it is assumed that the sender performs bitrate adjustment by selecting adaptively the maximum TemporalId that is transmitted from the EL. For the first GOP, no EL pictures are transmitted. For the second GOP, the sender determines to increase the video bitrate and transmits as many EL sub-layers as possible. As there are STSA pictures available at TID 0, 1, and 2 (i.e. pictures 1 A, and B, respectively), the sender switches up to transmit sub-layers with TID 0 to 2 starting from the second GOP of the EL. Switching up to TID 3 of the enhancement layer can take place later, when there is an EL IRAP picture or an EL TSA or STSA picture with TID equal to 3. It is noted that if the use of alternative output layers is enabled, pictures would be output constantly at "full" picture rate in this example.

Figure 6. First EL GOP not transmitted, and sub-layer up-switching to TID 2 at the second EL GOP.

It is noted that TSA pictures with TemporalId equal to 0 need not be allowed, as they are equivalent to CRA or IDR pictures.

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).