| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Valencia, ES, 27 March – 4 April 2014 | Document: JCTVC-Q0111 |
| --- | --- |

| Title: | Errata report: Parsing issue for picture timing SEI message |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposed correction |  |  |
| Author(s) or Contact(s): | Yongjun Wu Lihua Zhu Gary J. Sullivan Fyodor Kyslov Shyam Sadhwani Microsoft Corp. One Microsoft Way Redmond WA 98052-6399 USA | Tel: Email: | +1-425-703-5308 yongjunw@microsoft.com lizhu@microsoft.com garysull@microsoft.com fkyslov@microsoft.com shyams@microsoft.com |
| Source: | Microsoft Corp. |  |  |

_____________________________

# Abstract

This contribution asserts that there can be a parsing problem for the picture timing (PT) SEI message under certain circumstances, and suggests corrective actions to provide additional information in the standard and to prohibit a particular pathological combination. One discussed case is when CpbDpbDelaysPresentFlag is equal to 1 (and thus PT SEI messages are present) but NalHrdBpPresentFlag and VclHrdBpPresentFlag are both equal to 0 (and thus buffering period SEI message are not present). It is proposed to disallow this combination, as it is suggested that this combination does not seem particularly sensible or to represent any real application scenario and was not fully considered in the development of the standard. The second discussed case is when frame_field_info_present_flag is equal to 1 and CpbDpbDelaysPresentFlag is equal to 0, and it is proposed to add information to the standard to assist decoders in parsing PT SEI messages and to discourage encoders from generating bitstreams that could cause parsing problems.

# Problem statement

If a buffering period (BP) SEI message is present in a CVS, the active SPS is identified by the syntax of bp_seq_parameter_set_id, and the BP SEI message must precede the picture timing (PT) SEI message when they are both present. If the BP SEI message is not present but the PT SEI message is present, the SPS ID could be unknown and it could be difficult to parse the PT SEI message (e.g., because the syntax of the PT SEI message depends on length parameters sent in the SPS). In principle, some information could be conveyed "by external means", but the syntax was designed to require the BP SEI message to be the first SEI message when present, so that it can activate the SPS and remove any potential ambiguity before the PT SEI message arrives that has a dependency on the active SPS for its parsing.

The PT SEI message is present under the following conditions:

- It is present when frame_field_info_present_flag is equal to 1, in order to provide values of pic_struct, source_scan_type, and duplicate_flag.

- It is present when CpbDpbDelaysPresentFlag is equal to 1, in order to provide CPB and DPB removal delay information.

The BP SEI message is not present when NalHrdBpPresentFlag and VclHrdBpPresentFlag are both equal to 0. When the BP SEI message is not present in a CVS (if no "external means" resolves the issue), the decoder may not know the value of frame_field_info_present_flag (since it does not have the SPS ID) and thus may not know whether the PT SEI message is carrying pic_struct information, CPB/DPB removal delay information, or both. In such a case, the decoder would also not know how many bits are used to code the syntax elements of the CPB/DPB removal delay information.

We note that when NalHrdBpPresentFlag and VclHrdBpPresentFlag are both equal to 0, the amount of data in the payload of a PT SEI message must always be exactly 7 bits. The only way that the amount of data in the payload could otherwise be that small would be for frame_field_info_present_flag to be equal to 0 and to have very small values of au_cpb_removal_delay_length_minus1 and dpb_output_delay_length_minus1.

# Suggested action

We note that the CPB/DPB removal delays are expressed in units of time increments relative to the start of the buffering period, so awareness of the buffering period initialization point is crucial to interpreting them. We therefore suggest that the combination in which CPB/DPB removal delays are present in PT SEI messages but BP SEI messages are not present does not seem sensible.

We therefore suggest prohibiting the pathological combination in which CpbDpbDelaysPresentFlag is equal to 1 and NalHrdBpPresentFlag and VclHrdBpPresentFlag are both equal to 0.

We also suggest adding two types of informative remarks to the standard:

- To suggest that decoders check the amount of data in the payload when parsing a PT SEI message that is not preceded by a BP SEI message for the IRAP picture that begins the CVS. If the combination described above is prohibited, the amount of data in the payload of a PT SEI message must always be 8 bits (7 bits of syntax followed by a bit equal to 1 for the payload_bit_equal_to_one). If a situation with an unanticipated quantity of data is identified in a PT SEI message, it may be advisable for the decoder to delay parsing the PT SEI message until the SPS ID is determined.

- To suggest that encoders should not use values of au_cpb_removal_delay_length_minus1 and dpb_output_delay_length_minus1 that are less than 4 when frame_field_info_present_flag is equal to 0, to avoid the case where CPB/DPB removal delay information could become confused with pic_struct information (e.g., if a BP SEI message is lost).

We also suggest being careful in the future about using the payload_extension_present() mechanism to extend the PT SEI message, since this could affect decoders that use the amount of data in that SEI message to detect how to parse it.

# Patent rights declaration(s)

The authors of this contribution are not aware of current or pending patent rights regarding the specific modifications proposed in this contribution. However, Microsoft Corporation may have current or pending patent rights relating to other related aspects of the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).