| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Valencia, ES, 27 March – 4 April 2014 | Document: JCTVC-Q0112 |
| --- | --- |

| Title: | AhG8: In-loop color-space transform |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Li Zhang Jianle Chen Joel Sole Marta Karczewicz 5775 Morehouse Drive San Diego, CA 92121-1714 USA | Tel: Email: | + 1-858-651-6660 lizhang@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

This contribution proposes an in-loop adaptive color-space transform for HEVC Range Extensions. The color-space transform converts prediction error or original signals in 4:4:4 chroma format into those in a sub-optimal color space. As a result, redundancy among three color components is further reduced. . For CU with intra BC and inter mode, the color-space transform is applied to prediction error. For CU with intra modes, the color-space transform is applied to both neighboring reference samples and the current block, and then prediction residue is generated. One flag is signaled for each CU to indicate the usage of color-space transform. Experimental results show that the proposed method can achieve 22.0%, 26.9%, 23.1% BD-rate savings respectively for RGB, text & graphics with motion, 720p / RGB, mixed content, 1440p/ RGB animation, respectively in Low-delay B configurations under the test condition of the Call for Proposal for Screen Content Coding. The proposed method can also reportedly achieve 18.1%, 20.8% and 9.1% BD-rate reduction respectively for RGB SC, RGB animation, and optional RGB SC in Low-delay B Main-Tier configurations under the AhG8 common test conditions.

# Introduction

In the range extension of HEVC, the cross-component prediction (CCP) method is employed to predict the chroma residuals from luma residuals. Although it helps a lot in removing the correlation between luma and chroma components, redundancy among the color components still exists. To further decorrelate this redundancy, a CU-level Adaptive Color-space Transform (ACT) is proposed.

# Proposed Method

The proposed ACT converts one color-space to another one wherein the correlations among three color components are further reduced. It was found that ACT can be applied jointly with CCP to minimize the redundancy among the color components.

In ACT, each CU adaptively enables or disables color-space transform based on a rate-distortion criterion. For CU with intra BC and inter mode, the color-space transform is performed in the residual domain, i.e., just before invoking the CCP and conventional transform. For intra modes, the color-space transform is applied to both the prediction and the original block, i.e., before the residual generation process. A flag is signaled to indicate the usage of color-space transform. For intra BC and inter modes, such a flag is signaled only when there is at least one non-zero coefficient in the current CU.

Two different color-space transforms are applied depending on whether the CU is coded losslessly or in a lossy manner (depending on an SPS level). The forward and the inverse color-space transforms for lossy coding, are modified YCoCg transform, defined as follows:

Forward:  C'0C'1C'2=12120−2−12−1C0C1C2 	Inverse:	  C0C1C2=11−11011−1−1C'0C'1C'24

wherein the original color space (C0, C1, C2) may correspond to (R, G, B) or (Y, U, V). For lossless coding, the YCoCg-R transform in [1] is used.

The forward color transform used in lossy coding is not normalized, which results in signal amplification when the transform is applied. Considering that the norm of the forward transform is roughly equal to square root of 6, a delta QP of 8 is used to compensate such amplifier. That is, when the color-space transform is applied, the quantization parameter is set equal to QP+8, where QP is the ‘normal’ QP value for the CU. Meanwhile, the corresponding bit-depth is further increased by 2.

# Complexity Analysis

## Encoder

To avoid doubling the encoder complexity by searching all the possible modes twice for each CU, one pass without color-space transform and the other pass with color-space transform, the following fast encoding methods are employed in this contribution.

- For intra mode coding, the best luma and chroma modes are shared between the two passes (with or without color-space transform).

- For intraBC and inter modes, the motion information and predictors are shared between the two passes.

- In addition, the order of checking the rate-distortion cost of enabling and disabling color-space transform is dependent on the original color format. For RGB sequences, the rate-distortion cost for enabling color-space transform is firstly checked and the order is switched for YUV sequences. The second pass is only invoked when there is at least one non-zero coefficient in the first pass.

With above methods, the encoding complexity of ACT is controlled within a reasonable range (for typical cases, less than 20% in terms of encoding time).

## Decoder

The decoder complexity of ACT involves the inverse color-space transform after inverse transform and CCP. The additional complexity is minor since only addition and shift operations are required.

# Simulation Results

## Coding performance of ACT

In this section, the experimental results of the proposed method are presented. The method is tested on the top of HM13.0_Rext6.0. The test conditions of the CfP on SCC [2] and those of AHG8 [3] are used.

For the lossy coding, Table 1 Coding gains of ACT compared to anchor (SCC CfP lossy test conditions) reports the coding gain of ACT under SCC CfP test conditions and Table 2 reports the coding gain of ACT under AhG8 test conditions.

For the lossless coding, the average coding gain of ACT under SCC test conditions and AhG8 test conditions are reported in Table 3 and Table 4, respectively.

Table 1 Coding gains of ACT compared to anchor (SCC CfP lossy test conditions)

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| RGB, text & graphics with motion, 1080p | -13.4% | -5.3% | -4.7% |
| RGB, text & graphics with motion,720p | -19.3% | -6.0% | -9.5% |
| RGB, mixed content, 1440p | -19.4% | -9.7% | -9.6% |
| RGB, mixed content, 1080p | -16.3% | -6.9% | -6.7% |
| RGB, Animation, 720p | -23.0% | -18.5% | -15.1% |
| YUV, text & graphics with motion, 1080p | -1.0% | -0.9% | -1.2% |
| YUV, text & graphics with motion,720p | -0.4% | -1.3% | -1.9% |
| YUV, mixed content, 1440p | 0.2% | -1.4% | -1.4% |
| YUV, mixed content, 1080p | -0.6% | -1.2% | -1.1% |
| YUV, Animation, 720p | -0.4% | -5.8% | -2.7% |
| Enc Time[%] | 124% |  |  |
| Dec Time[%] | 106% |  |  |
|  | Random Access |  |  |
|  | Y | U | V |
| RGB, text & graphics with motion, 1080p | -12.2% | -2.8% | -2.2% |
| RGB, text & graphics with motion,720p | -21.1% | -6.5% | -11.2% |
| RGB, mixed content, 1440p | -25.4% | -12.9% | -12.6% |
| RGB, mixed content, 1080p | -21.4% | -8.0% | -7.0% |
| RGB, Animation, 720p | -23.6% | -17.6% | -12.7% |
| YUV, text & graphics with motion, 1080p | -1.0% | -1.5% | -2.0% |
| YUV, text & graphics with motion,720p | -1.3% | -1.8% | -3.1% |
| YUV, mixed content, 1440p | 0.0% | -1.8% | -1.7% |
| YUV, mixed content, 1080p | -0.8% | -2.6% | -2.0% |
| YUV, Animation, 720p | -0.3% | -7.0% | -3.7% |
| Enc Time[%] | 120% |  |  |
| Dec Time[%] | 100% |  |  |
|  | Low delay B |  |  |
|  | Y | U | V |
| RGB, text & graphics with motion, 1080p | -10.8% | -0.6% | -0.1% |
| RGB, text & graphics with motion,720p | -22.0% | -5.8% | -10.3% |
| RGB, mixed content, 1440p | -26.9% | -11.6% | -10.7% |
| RGB, mixed content, 1080p | -21.5% | -6.3% | -5.2% |
| RGB, Animation, 720p | -23.1% | -14.4% | -7.6% |
| YUV, text & graphics with motion, 1080p | -0.7% | -1.7% | -2.0% |
| YUV, text & graphics with motion,720p | -1.3% | -2.9% | -4.8% |
| YUV, mixed content, 1440p | 0.3% | -2.9% | -2.7% |
| YUV, mixed content, 1080p | -0.5% | -2.8% | -2.4% |
| YUV, Animation, 720p | 0.4% | -7.6% | -4.8% |
| Enc Time[%] | 123% |  |  |
| Dec Time[%] | 103% |  |  |

Table 2 Coding gains of ACT compared to anchor (AhG8 lossy test conditions)

|  | All Intra Main-tier |  |  | All Intra High-tier |  |  | All Intra Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| RGB 4:4:4 SC | -15.1% | -5.0% | -7.6% | -11.5% | -4.5% | -6.4% | -9.3% | -4.0% | -5.2% |
| RGB 4:4:4 Animation | -21.0% | -14.2% | -10.9% | -19.3% | -14.9% | -12.0% | -17.3% | -13.8% | -11.5% |
| YCbCr 4:4:4 SC | -0.3% | -1.0% | -1.6% | -0.5% | -0.8% | -1.3% | -0.6% | -0.6% | -0.9% |
| YCbCr 4:4:4 Animation | 0.1% | -2.8% | -1.7% | -0.5% | -3.7% | -1.6% | -1.0% | -3.5% | -1.2% |
| RangeExt | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB 4:4:4 SC (Optional) | -10.6% | -8.5% | -8.3% | -9.2% | -7.6% | -7.2% | -9.3% | -8.2% | -8.2% |
| YCbCr 4:4:4 SC (Optional) | -1.2% | -0.5% | -0.7% | -1.1% | -0.6% | -0.2% | -1.2% | -0.7% | -0.6% |
| Enc Time[%] | 121% |  |  | 124% |  |  | 127% |  |  |
| Dec Time[%] | 107% |  |  | 107% |  |  | 107% |  |  |
|  | Random Access Main-tier |  |  | Random Access High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| RGB 4:4:4 SC | -17.0% | -5.6% | -8.8% | -13.6% | -5.0% | -7.6% |  |  |  |
| RGB 4:4:4 Animation | -19.9% | -12.6% | -7.7% | -19.7% | -13.6% | -8.6% |  |  |  |
| YCbCr 4:4:4 SC | -0.9% | -1.3% | -2.4% | -1.2% | -1.2% | -2.2% |  |  |  |
| YCbCr 4:4:4 Animation | 0.1% | -3.5% | -2.2% | -0.3% | -4.3% | -2.1% |  |  |  |
| RangeExt | 0.1% | -0.2% | -0.2% | 0.1% | -0.1% | -0.2% |  |  |  |
| RGB 4:4:4 SC (Optional) | -9.0% | -7.1% | -6.9% | -8.0% | -6.6% | -6.3% |  |  |  |
| YCbCr 4:4:4 SC (Optional) | -1.6% | -0.8% | -1.2% | -1.4% | -0.7% | -1.0% |  |  |  |
| Enc Time[%] | 111% |  |  | 114% |  |  |  |  |  |
| Dec Time[%] | 105% |  |  | 105% |  |  |  |  |  |
|  | Low delay B Main-tier |  |  | Low delay B High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| RGB 4:4:4 SC | -18.1% | -5.4% | -8.6% | -14.3% | -5.3% | -7.7% |  |  |  |
| RGB 4:4:4 Animation | -20.8% | -10.7% | -5.7% | -19.8% | -12.0% | -7.0% |  |  |  |
| YCbCr 4:4:4 SC | -0.8% | -2.7% | -4.1% | -1.2% | -2.4% | -3.6% |  |  |  |
| YCbCr 4:4:4 Animation | 0.3% | -3.7% | -2.6% | 0.0% | -4.7% | -2.6% |  |  |  |
| RangeExt | 0.1% | -0.2% | -0.2% | 0.1% | -0.1% | -0.2% |  |  |  |
| RGB 4:4:4 SC (Optional) | -9.1% | -6.8% | -6.4% | -9.1% | -7.3% | -7.1% |  |  |  |
| YCbCr 4:4:4 SC (Optional) | -0.8% | -0.8% | -1.0% | -0.5% | -0.7% | -1.0% |  |  |  |
| Enc Time[%] | 108% |  |  | 110% |  |  |  |  |  |
| Dec Time[%] | 104% |  |  | 103% |  |  |  |  |  |

Table 3 BitRate Savings of ACT compared to anchor (SCC CfP lossless test conditions)

|  | Bit-rate savings |  |  |
| --- | --- | --- | --- |
|  | AI | RA | LB |
| RGB, text & graphics with motion, 1080p | 0.6% | 1.0% | 0.7% |
| RGB, text & graphics with motion,720p | 0.8% | 1.2% | 1.3% |
| RGB, mixed content, 1440p | 1.8% | 1.6% | 1.8% |
| RGB, mixed content, 1080p | 2.5% | 2.3% | 2.4% |
| RGB, Animation, 720p | 3.9% | 2.4% | 2.5% |
| YUV, text & graphics with motion, 1080p | 0.5% | 0.3% | 0.3% |
| YUV, text & graphics with motion,720p | 0.1% | 0.2% | 0.2% |
| YUV, mixed content, 1440p | 1.8% | 1.6% | 1.8% |
| YUV, mixed content, 1080p | 2.5% | 2.3% | 2.4% |
| YUV, Animation, 720p | 3.9% | 2.4% | 2.5% |
| Enc Time[%] | 134% | 121% | 128% |
| Dec Time[%] | 108% | 98% | 105% |

Table 4 Coding gains of ACT compared to anchor (AhG8 lossless test conditions)

|  | Average bit-rate increase |  |  |
| --- | --- | --- | --- |
|  | AI | RA | LB |
| RGB 4:4:4 SC | -0.8% | -1.1% | -1.3% |
| RGB 4:4:4 Animation | -1.8% | -1.3% | -1.3% |
| YCbCr 4:4:4 SC | -0.1% | -0.2% | -0.2% |
| YCbCr 4:4:4 Animation | -0.5% | -0.2% | -0.2% |
| RangeExt | 0.0% | 0.0% | 0.0% |
| RGB 4:4:4 SC (Optional) | -4.7% | -5.5% | -5.1% |
| YCbCr 4:4:4 SC (Optional) | -0.2% | -0.2% | -0.1% |
| Enc Time[%] | 116% | 115% | 121% |
| Dec Time[%] | 100% | 100% | 100% |

## Comparisons between ACT and pre/post processing

To further evaluate the coding performance of the ACT, results of the proposed ACT compared to pre/post processing are also provided. Here, we use the YCoCg transform [1] as the counterpart and only RGB test sequences are used for performance evaluation.

Table 5 and Table 6 show the results for lossy coding under the CfP SCC and AhG8 test conditions, respectively. Since the norm of forward YCoCg transform is roughly equal to (square root of 6)/4, for a fair comparison, QP used in the pre/post processing method is reduced by 4 to make the two methods have similar bitrates.

Table 5 Coding gains of ACT compared to pre/post processing (SCC CfP test conditions)

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| RGB, text & graphics with motion, 1080p | -6.7% | -2.1% | -1.9% |
| RGB, text & graphics with motion,720p | -5.7% | -2.0% | -0.6% |
| RGB, mixed content, 1440p | -2.6% | -0.4% | -0.5% |
| RGB, mixed content, 1080p | -3.1% | -0.5% | -1.0% |
| RGB, Animation, 720p | -2.9% | -3.2% | -1.6% |
|  | Random Access |  |  |
|  | Y | U | V |
| RGB, text & graphics with motion, 1080p | -5.9% | 1.4% | 1.6% |
| RGB, text & graphics with motion,720p | -9.2% | -5.0% | -3.1% |
| RGB, mixed content, 1440p | -5.7% | -3.4% | -3.4% |
| RGB, mixed content, 1080p | -4.8% | 2.4% | 2.1% |
| RGB, Animation, 720p | -6.0% | -5.3% | -2.2% |
|  | Low delay B |  |  |
|  | Y | U | V |
| RGB, text & graphics with motion, 1080p | -4.1% | 2.9% | 3.3% |
| RGB, text & graphics with motion,720p | -5.8% | -1.5% | 0.9% |
| RGB, mixed content, 1440p | -2.7% | 0.5% | 1.1% |
| RGB, mixed content, 1080p | -2.1% | 4.6% | 4.9% |
| RGB, Animation, 720p | -4.6% | -2.9% | 1.1% |

Table 6 Coding gains of ACT compared to pre/post processing (AhG8 test conditions)

|  | All Intra Main-tier |  |  | All Intra High-tier |  |  | All Intra Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| RGB 4:4:4 SC | -5.5% | -3.7% | -2.4% | -7.5% | -7.7% | -5.8% | -11.9% | -11.9% | -9.2% |
| RGB 4:4:4 Animation | -5.1% | -5.0% | -3.3% | -7.3% | -9.4% | -7.3% | -12.7% | -16.2% | -13.6% |
|  | Random Access Main-tier |  |  | Random Access High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| RGB 4:4:4 SC | -8.7% | -6.2% | -4.5% | -10.6% | -10.4% | -7.8% |  |  |  |
| RGB 4:4:4 Animation | -5.3% | -3.4% | 0.4% | -7.3% | -7.7% | -4.0% |  |  |  |
|  | Low delay B Main-tier |  |  | Low delay B High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| RGB 4:4:4 SC | -5.7% | -3.5% | -1.5% | -7.4% | -7.9% | -5.3% |  |  |  |
| RGB 4:4:4 Animation | -2.1% | 1.0% | 4.6% | -3.1% | -3.8% | -0.4% |  |  |  |

# References

- Henrique S. Malvar, Gary J. Sullivan, and Sridhar Srinivasan, “Lifting-based reversible color transformations for image compression,” SPIE Proceedings, Vol. 7073, DOI: 10.1117/12.797091, Oct. 2008.

- ISO/IEC JTC1/SC29/WG11 (MPEG) and ITU-T SG16 WP3 (VCEG), “Joint Call for Proposals for coding of screen content,” Jan. 2014

- C. Rosewarne, K. Sharman, and D. Flynn, “Common test conditions and software reference configurations for HEVC range extensions,” JCTVC-P1006, San José, US, 9–17 Jan. 2014.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).