| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Valencia, ES, 27 March – 4 April 2014 | Document: JCTVC-Q0114 |
| --- | --- |

| Title: | Block vector prediction method for Intra block copy |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chao Pang, Joel Sole, Rajan Joshi, Marta Karczewicz 5775 Morehouse Drive San Diego, CA 92121-1714 USA | Tel: Email: | + 1-858-658-3779 cpang@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

In this document, block vector prediction method for Intra block copy is proposed. In the proposed method, either spatial neighboring block vector or default vector is used as block vector predictor. Reported average BD-rates are -1.4%, -1.0% and -0.7% for YUV 4:4:4 SC sequences in AI MT, RA MT and LB MT, respectively.

# Introduction

In current HEVC Range Extensions, the last decoded block vector (BV) is used as BV predictor for one PU with Intra block copy (Intra BC) mode except that the PU is the first Intra BC PU in one CTU. For the first Intra BC PU in one CTU, its BV predictor is (-CUWidth, 0). The block vector difference (BVD) is coded by sharing the motion vector difference (MVD) coding method in HEVC.

# Proposed method

In the proposed method, either the left or above BV is selected as predictor. If one of them is unavailable, (-2*W, 0) is used instead, where W is the PU width. If both of them are unavailable, (-2*W, 0) and (2*W, 0) are used instead. To avoid the line buffer, the above BV out of current CTU is considered unavailable.

# Results

In this section, experimental results are presented for the proposed BV prediction method. The proposed method is implemented on top of HM13.0_RExt6.0, and AHG8 test conditions [1] are used. The performance is compared to the anchor for lossy (Table 1) and lossless (Table 2) configuration in terms of BD-rates and rate savings, respectively.

Table 1 Lossy results

|  | All Intra Main-tier |  |  | All Intra High-tier |  |  | All Intra Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class F | -0.7% | -0.7% | -0.7% | -0.6% | -0.5% | -0.5% | -0.4% | -0.4% | -0.4% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB 4:4:4 SC | -1.3% | -1.4% | -1.4% | -1.1% | -1.2% | -1.1% | -0.9% | -1.0% | -0.9% |
| RGB 4:4:4 Animation | 0.1% | 0.1% | 0.1% | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% |
| YCbCr 4:4:4 SC | -1.4% | -1.4% | -1.5% | -1.2% | -1.3% | -1.3% | -1.0% | -1.1% | -1.1% |
| YCbCr 4:4:4 Animation | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% |
| RangeExt | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB 4:4:4 SC (Optional) | -2.2% | -2.1% | -2.3% | -2.2% | -2.3% | -2.4% | -1.8% | -1.5% | -1.5% |
| YCbCr 4:4:4 SC (Optional) | -2.4% | -2.3% | -2.5% | -2.2% | -2.2% | -2.2% | -1.9% | -1.9% | -1.9% |
| Enc Time[%] | 101% |  |  | 100% |  |  | 99% |  |  |
| Dec Time[%] | 101% |  |  | 100% |  |  | 99% |  |  |
|  | Random Access Main-tier |  |  | Random Access High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | -0.5% | -0.4% | -0.5% | -0.4% | -0.3% | -0.3% |  |  |  |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| RGB 4:4:4 SC | -1.1% | -1.1% | -1.1% | -0.9% | -0.9% | -0.9% |  |  |  |
| RGB 4:4:4 Animation | 0.1% | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% |  |  |  |
| YCbCr 4:4:4 SC | -1.0% | -1.0% | -1.1% | -0.9% | -0.9% | -1.0% |  |  |  |
| YCbCr 4:4:4 Animation | 0.0% | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% |  |  |  |
| RangeExt | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| RGB 4:4:4 SC (Optional) | -1.6% | -1.7% | -1.6% | -1.8% | -1.8% | -1.8% |  |  |  |
| YCbCr 4:4:4 SC (Optional) | -2.2% | -2.2% | -2.2% | -2.3% | -2.3% | -2.3% |  |  |  |
| Enc Time[%] | 99% |  |  | 99% |  |  |  |  |  |
| Dec Time[%] | 105% |  |  | 104% |  |  |  |  |  |
|  | Low delay B Main-tier |  |  | Low delay B High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | -0.3% | 0.2% | -0.5% | -0.2% | -0.1% | -0.3% |  |  |  |
| Class B | 0.0% | -0.2% | -0.3% | 0.0% | -0.1% | -0.2% |  |  |  |
| RGB 4:4:4 SC | -0.6% | -0.7% | -0.7% | -0.6% | -0.7% | -0.7% |  |  |  |
| RGB 4:4:4 Animation | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| YCbCr 4:4:4 SC | -0.7% | -1.0% | -1.2% | -0.8% | -0.9% | -1.1% |  |  |  |
| YCbCr 4:4:4 Animation | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% |  |  |  |
| RangeExt | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.1% |  |  |  |
| RGB 4:4:4 SC (Optional) | -1.5% | -1.5% | -1.6% | -1.9% | -1.8% | -1.8% |  |  |  |
| YCbCr 4:4:4 SC (Optional) | -1.9% | -1.8% | -1.8% | -2.4% | -2.4% | -2.3% |  |  |  |
| Enc Time[%] | 99% |  |  | 99% |  |  |  |  |  |
| Dec Time[%] | 101% |  |  | 101% |  |  |  |  |  |

Table 2 Lossless results

| Bit-rate saving (Average) | AI Main | RA Main | LB Main |
| --- | --- | --- | --- |
| Class F | 0.0% | 0.1% | 0.1% |
| Class B | 0.0% | 0.0% | 0.0% |
| SC RGB 444 | -0.3% | -0.1% | -0.2% |
| Animation RGB 444 | 0.0% | 0.0% | 0.0% |
| SC YUV 444 | -0.3% | -0.4% | -0.2% |
| Animation YUV 444 | 0.0% | 0.0% | 0.0% |
| RangeExt | 0.0% | 0.0% | 0.0% |
| SC GBR 444 Optional | 1.3% | 1.2% | 2.2% |
| SC YUV 444 Optional | -0.7% | -0.8% | -1.0% |
| Enc Time[%] | 107% | 105% | 109% |
| Dec Time[%] | 105% | 107% | 110% |

# References

[1]C. Rosewarne, K. Sharman, and D. Flynn, “Common test conditions and software reference configurations for HEVC range extensions”, JCTVC-P1006, San José, US, January 2014.

# Patent rights declaration(s)

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).