| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Valencia, ES, 27 March – 4 April 2014 | Document: JCTVC-Q0116_v2 |
| --- | --- |

| Title: | RExt HLS: Picture referencing across CRA pictures |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Rickard Sjöberg Jonatan Samuelsson Ying Wang Ericsson Research Färögatan 6 16480 Stockholm | Tel: Email: | +46 761153563 rickard.sjoberg@ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

This contribution is a follow-up of contribution JCTVC-P0118 and claims that the HEVC/RExt restrictions that prohibit prediction between trailing and leading pictures hurt compression efficiency, especially for field coding referencing structures. The contribution reports that the proposed changes provide a 1.7% average bit-rate reduction for all eight test sequences used within the MPEG AHG on study of interlaced coding in HEVC using QPs [22, 27, 32, 37]. Using QPs [17, 22, 27, 32] the reported bit-rate reduction is 1.5%. Source code patches for generating this result are attached with the contribution.

The contribution proposes to use a reserved NUT to enable prediction across IRAP pictures while still enable usage of the existing picture types such as CRA, RADL, RASL.

The contribution claims that even though it is possible to indicate random access capability using recovery point SEI messages with TRAIL pictures, the contribution states that it is strongly preferable to be able to use the IRAP NAL unit types to in order to perform random-access-point detection at NAL unit header level and make use of the features associated with IRAP pictures such as alternative HRD parameters etc.

The following elements are proposed:

- Use a reserved VCL NAL unit type (type 11 is proposed) to be used for a new picture type, here called a CTR picture. The CTR picture is a trailing picture that, in contrast to the existing trailing picture types, can be referenced by leading pictures.

- Add a constraint that a CTR picture must immediately follow the associated IRAP picture in decoding order.

- Add a constraint that there can be at most one CTR picture for each IRAP picture (IDR, BLA, CRA).

The proposal notes that an alternative solution that was discussed at the last meeting was to add a flag in the RExt SPS extension to allow one trailing picture that directly follows an IRAP picture in decoding order to be used for reference by leading pictures associated with the same IRAP picture.

Version 2 of this contribution contains results for 8 sequences instead of 6, as well as results for QPs [17, 22, 27, 32].

# Background and Problem Statement

This contribution is a follow-up of contribution JCTVC-P0118 [1]. In the current HEVC Range Extensions draft specification [2] there are restrictions associated with IRAP picture types. One of those restrictions is the following:

When a picture is a leading picture, it shall precede, in decoding order, all trailing pictures that are associated with the same IRAP picture.

As a result, all leading pictures must precede all trailing pictures of the associated CRA picture, thus it is impossible for pictures before the CRA in output order to use pictures that are output after the CRA for reference. This reduces prediction efficiency.

Consider Figure 1 which shows a field coding structure. The output order is left to right and the decoding order is indicated by numbers inside the pictures. Let the picture immediately following the CRA picture be called the CTR picture. Due to the restriction above, the CTR picture needs to follow all leading pictures in decoding order and be coded as B8. This prohibits the dotted prediction arrows which mean that pictures B3 and B7 have no backward prediction of the same field parity. If the restriction would not apply, the CTR picture could have been coded as B1 and the dotted prediction arrow would have been allowed.

Figure 1.  Example of structure in field coding

# Proposal

We propose to introduce a NAL unit type called CRA trailing reference (CTR_NUT) and change the restriction of IRAP pictures.

When a picture is a leading picture, it shall precede, in decoding order, all trailing pictures with nal_unit_type not equal to CTR_NUT that are associated with the same IRAP picture.

We also propose to add the following restrictions:

A CTR picture associated with an IRAP picture shall directly follow the associated IRAP picture in decoding order.

An IRAP picture shall not have more than one associated CTR picture.

We propose to use NAL unit type 11 for CTR_NUT:

Table 1: Proposed change to NAL unit types

| nal_unit_type | Name of nal_unit_type | Content of NAL unit and RBSP syntax structure | NAL unit type class |
| --- | --- | --- | --- |
| 11 | CTR_NUT | Coded slice segment of a CTR picture slice_segment_layer_rbsp( ) | VCL |
| 11 13 15 | RSV_VCL_R11 RSV_VCL_R13 RSV_VCL_R15 | Reserved non-IRAP sub-layer reference VCL NAL unit types | VCL |

A decoder/node can recognize this type and treat it as a special trailing picture. Figure 2 shows an example bitstream structure, the decoder/node can simply discard all NAL units between the CTR picture and the first non-CTR trailing picture of the associated IRAP picture.

Figure 2. Example of bitstream structure with proposal

# Results

A compression efficiency test was done using HM-12.1+RExt-5.0rc1 [3] and the \cfg\misc\encoder_randomaccess_field_coding.cfg configuration. Eight available test sequences used within the MPEG AHG on study of interlaced coding in HEVC were used. Table 2 shows the results of enabling prediction across CRA pictures. QPs 22, 27, 32, and 37 were used. As shown in Table 2, the proposal reduces the average BD-rate by 1.7%.

Table 2: Luma BD-rate performance vs HM-12.1+RExt-5.0rc1 (QP 22, 27, 32, 37)

| Sequence | BDR | Bit-rate range |
| --- | --- | --- |
| MusicHD1_1088_300 | -2.28% | 4-47 Mbps |
| CheersHD2_1088_300 | -0.36% | 5-52 Mbps |
| CheersHD4_1088_300 | -0.56% | 9-103 Mbps |
| MadHD1_1088_300 | -2.80% | 1.5-14 Mbps |
| Tennis_1920x1088_420_60i_8bit | -4,68% | 2-31 Mbps |
| Basketball_1920x1088_420_60i_8bit | -1,68% | 4-67 Mbps |
| CattleDogs_1920x1088_420_60i_8bit | -0.68% | 1-18 Mbps |
| WaveRocks_1920x1088_420_60i_8bit | -0.42% | 2-36 Mbps |
| Average | -1,68% |  |

Table 3: Luma BD-rate performance vs HM-12.1+RExt-5.0rc1 (QP 17, 22, 27, 32)

| Sequence | BDR | Bit-rate range |
| --- | --- | --- |
| MusicHD1_1088_300 | -2.02% | 8-122 Mbps |
| CheersHD2_1088_300 | -0.33% | 10-114 Mbps |
| CheersHD4_1088_300 | -0.51% | 21-207 Mbps |
| MadHD1_1088_300 | -2.97% | 3-38 Mbps |
| Tennis_1920x1088_420_60i_8bit | -3.56% | 4-80 Mbps |
| Basketball_1920x1088_420_60i_8bit | -0.92% | 8-173 Mbps |
| CattleDogs_1920x1088_420_60i_8bit | -1.10% | 2-59 Mbps |
| WaveRocks_1920x1088_420_60i_8bit | -0.50% | 5-85 Mbps |
| Average | -1.49% |  |

Table 2 indicates that the average bit-rate savings is 1.7% and 1.5% for main and high tier respectively.

Attached to this contribution are two source code patches that can be applied to HM-12.1+RExt-5.0rc1 in order to compile the anchor and test codecs used for generating the results shown in Table 2 and Table 3. The same cfg\misc\encoder_randomaccess_field_coding.cfg configuration file should be used for both anchor and test; besides QP, only parameters in the “File I/O” section needs to be changed.

# References

[1] “RExt HLS: Picture referencing across CRA pictures”, JCTVC-P0118v4, Sjöberg et al, 16th Meeting: San José, US, 9–17 Jan. 2014.

[2] “High Efficiency Video Coding (HEVC) Range Extensions text specification: Draft 6”, JCTVC-P1005_v1.doc, 16th Meeting: San José, US, 9–17 Jan. 2014.

[3] https://hevc.hhi.fraunhofer.de/svn/svn_HEVCSoftware/tags/HM-12.1+RExt-5.0rc1

# Patent rights declaration(s)

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).