| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Valencia, ES, 27 March – 4 April 2014 | Document: JCTVC-Q0118 |
| --- | --- |

| Title: | Interlace Coding in HEVC v.1 |  |  |
| --- | --- | --- | --- |
| Status: | Input |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Alexandros Tourapis David Singer 1 Infinite Loop Cupertino, CA 95014 USA | Tel: Email: | atourapis@apple.com singer@apple.com |
| Source: | Apple Inc. |  |  |

_____________________________

# Abstract

In this contribution we present several observations and try to address several issues with regards to interlace encoding support in HEVC v.1 and its HM reference software. Unlike its predecessors, the first version of the HEVC standard does not provide explicit coding tools for the handling of interlace. However, it is suggested that given the further flexibility that the existing HEVC coding tools provide, as well as the fewer restrictions imposed by the standard on field picture coding, further coding gains could be achieved.

# Introduction

The High Efficiency Video Coding (HEVC) standard has adopted a variety of new video coding tools in an attempt to achieve improved coding efficiency versus previous coding standards. This includes new methods for intra/spatial and inter/temporal prediction, new post-filtering mechanisms (e.g. SAO), more transforms and improved signaling of information among others. However, no explicit attempt was made to improve the HEVC standard’s coding efficiency for the encoding of interlace material. It was instead felt that the new coding tools introduced would also provide a sufficient enough performance improvement versus preceding standards  (e.g. MPEG-4 AVC/H.264), without though further penalizing implementations with having to support such explicit purpose coding tools. Initial experiments substantiated these claims showing performance benefits of 30-50% for interlace coding using HEVC v1 versus MPEG-4 AVC and its more explicit interlace coding tools, i.e. use of Picture Level Adaptive Frame Field (PicAFF) coding among others.

In this document we explore the current implementation of the HEVC reference software (HM) and of its handling of interlace material, and suggest additional methods, in the context of non-normative optimizations, that could be used to potentially further enhance the performance of interlace material. This report is not written with the intention to discourage further investigation, given a real need, on new coding methods that explicitly would target interlace material, but instead to provide some insight and assist in the development of better encoding techniques for people who may be restricted in using HEVC v1 encoders and decoders as well as in better developing new normative tools, given the limitations that interlacing coding in the context of HEVC v1 may still have.

# Picture Reference Handling

## Available References

There seems to be some misunderstanding on how many references the HEVC standard can support in the case of field coding. Unlike what seems to be believed, the standard appropriately considers the field picture’s luma resolution when determining the value of MaxDpbSlice. This can be seen in Annex.4.1 of the HEVC specification where is it stated that:

The value of sps_max_dec_pic_buffering_minus1[ HighestTid ] + 1 shall be less than or equal to MaxDpbSize, which is derived as follows:

if( PicSizeInSamplesY  <=  ( MaxLumaPs  >>  2 ) )
	MaxDpbSize = Min( 4 * maxDpbPicBuf, 16 )
else if( PicSizeInSamplesY  <=  ( MaxLumaPs  >>  1 ) )
	MaxDpbSize = Min( 2 * maxDpbPicBuf, 16 )	(A-1)
else if( PicSizeInSamplesY  <=  ( ( 3 * MaxLumaPs )  >>  2 ) )
	MaxDpbSize = Min( ( 4 * maxDpbPicBuf ) / 3, 16 )
else
	MaxDpbSize = maxDpbPicBuf

where MaxLumaPs is specified in Error: Reference source not found and maxDpbPicBuf is equal to 6.

In the above, given that the resolution of a field picture is half of that of a frame picture, twice the references versus frame pictures will be allocated (up to a maximum of 16). No other restriction seems to exist in the specification that would limit the number of references explicitly for field coding. For a level 4.1 encoding of 1080i material, for example, PicSizeInSamplesY would be equal to 1036800 and MaxLumaPs would be equal to 2228224, resulting in:

MaxDpbSize = Min(2*maxDpbPicBuf,16) = Min(2*6,16) = 12

Unfortunately, this misconception seems to also be, unintentionally, driven by the example field coding configuration files that have been provided with the HM software reference software. Likely for convenience, the number of references when translating the random access hierarchical coding structure for frames into fields were kept the same, unintentionally limiting the coding capabilities of HEVC when encoding interlace material.  Ideally, for a frame based 4 level hierarchy as shown in Figure 1, the DPB would progress in a way that all available slots are effectively utilized for inter prediction.   This figure also identifies the progress of the DPB per coded picture.

Figure 1. Coding Order and DPB progression for a Frame Sequence using a Random Access GOP.

A similar behavior is also desired for a field-based hierarchy of the same level. However, given the increase of available reference stores, now more pictures will be available for inter-prediction, even though these references could (but not necessarily) point to the same reference samples as in the frame coding case. In Figure 2 we show the available references (limited to a max of 10 pictures excluding the current picture) in the DPB when coding the first of the two fields in a field pair as an example. It should be noted that for the second field, the first field could also be available as a reference if it is so desired. For 1080i material and for level 4.1, given that we could use up to 11 pictures (excluding the current one) for inter prediction, it would not be necessary to also remove any additional picture from the DPB if we so desire (note that in MPEG-4 AVC there were some restrictions in terms of availability of field references that could impact performance).

Figure 2. Coding Order and DPB progression for a Field Sequence using a Random Access GOP.

## Reference Order

The MPEG-4 AVC standard specifies a default reference order for field pictures that also takes in account field parity. This is based on the assumption that a field picture may have much more correlation with its identical parity pictures than the opposite parity ones, even if the later are closer in time. This is especially true in the scenario of static or semi-static images. In general though, there are no guarantees that the field parity biased mode would always be better than the display order biased mode and figuring out the best picture reference order in each list would be desired.

The currently provided configuration files for field coding in HEVC seem to ignore this observation and essentially only use the display order as a mechanism for ordering the references in each list. Even worse, for the low-delay coding mode, in many cases no attempt is made in making the two lists (List_0 and List_1) different from one another. This may result in some inefficiency when performing prediction, due to wasting bits when signaling reference indices. Even though entropy coding may somewhat reduce this effect, early on decisions, where good enough statistics are not yet available could be adversely affected.

The MPEG-4 AVC/H.264 JM reference software already provides an exemplary scheme for reference reordering based on hierarchical motion estimation and distortion analysis. A similar scheme could be adopted in the HM or other future HEVC implementations, but we understand that that may be difficult at this stage. A few alternatives may exist with different implementation, performance, and complexity characteristics:

- Perform a “Sequence level” adaptation. Code the same sequence using both field-parity bias and display-order bias for the reference lists and select the one that yields the best performance.

- Perform a “picture level” adaptation. First, encode the picture in a preferred mode (i.e. field-parity bias mode). Then re-encode the same picture in an alternate mode (i.e. display-order bias mode) and select the one that performs the best. The second pass could be conditioned based on the referencing behavior of the first pass (i.e. if prediction is happening mainly from the same parity pictures, then we likely do not need to perform a second pass; alternatively we could also reorder all pictures based on the statistics of the first pass). It should be noted that the complexity of this is inline with the two-pass approach currently used for field vs. frame coding in the context of a PicAFF framework.

In general, the flexibility of adjusting the order and number of references in field coding should not be at all ignored when one is designing a field picture HEVC encoder.

# Hierarchical Coding Structures and Field Pictures

## Field Pairing

Unlike previous standards that seemed to have restricted field pictures to mainly be handled in pairs (with the noted exception of dangling fields in MPEG-4 AVC, which are commonly avoided by most encoder implementations), HEVC v1 does not currently appear to impose such a restriction. This means that one may desire to use a much more flexible coding structure that could potentially provide further coding benefits when using HEVC for the encoding of interlace material. The simplest example of such a coding structure is shown in Figure 3.

Figure 3. Example of a coding order with no field pairing.

An alternative coding order that follows the same principles but also now takes in account the possibility that someone may desire to have random access capabilities at key points at the “frame” level is shown in Figure 4. In general, it should be emphasized that unlike the deep and rather fixed hierarchical structures used in HEVC tests and evaluations, real implementations tend to use much more flexible (and commonly smaller) hierarchies. Longer hierarchies are also more useful, in terms of coding performance, for static scenes, while it may be less desirable to use a long hierarchy when encountering high motion scenes or rapid scene changes. There are also scenarios in field coding where a single frame may consist of two different scenes and having the ability to adapt the depth of the hierarchy can be rather advantageous in terms of coding performance.

Figure 4. Example of a coding order with field pairing only at random access points (Frames 0 and 9).

## Alternate Parity Bias

Figure 5. Complementary Field Pair relationship in current HEVC HM encodings

In another observation, we have noticed that current HEVC field coding examples using the HM seem to always consider a strict coding order between the top (or first) and bottom (or second) fields in the complementary field pair. That is, the HM always codes the first field in display order first, followed by the bottom one (Figure 5). In addition to that, the HM imposes at key intervals a higher QP and lambda value for the second field, compromising its quality. In some cases, there could be a quality difference between the two fields of more than 1dB. That may have an adverse effect in performance especially subjectively since the second field may always have a worse quality than the first. One should not also ignore that commonly interlace material have to be de-interlaced first before displaying. Since the two fields may have asymmetric quality, that could potentially create issues to a de-interlacer and may result in poor de-interlacing performance and more artifacts.

We believe that this issue could be addressed by alternating the coding priority between top and bottom fields. This can be done easily while still maintaining the complementary field pair relationship, if that is desired. An example of how this could be done is shown in Error: Reference source not found. In this example, the encoder alternates the priority of the first and second field, which may include its coding order as well as its quality control coding parameters (e.g. QP and lambda). This removes the quality bias and can more evenly allocate quality between the two fields. Having also the second field at times coded ahead of the first field may also benefit bi-prediction performance.

Figure 6. Unbiased Coding Order for Field Pairs in a Random Access Hierarchy

Although the above may appear more straightforward for the Random Access scenario, a similar concept can also be considered for the Low Delay case, assuming however that a delay of one field picture is permissible. If it is not, however, then one could argue that even the concept of frame coding, within the context of PicAFF, should not also be allowed since that would also introduce an additional field picture delay.

Figure 7. Bottom biased Low delay (1 field) coding order

Figure 7 presents one example of how a “low-delay” coding structure (with 1 field delay) could be performed, with a higher bias, except for the IRAP point, for the bottom field picture. One observation again in this structure is the likelihood of better exploiting bi-prediction given the coding order of the two pictures.

Figure 8. Un-biased Low delay (1 field) coding order

In Figure 8 we present an unbiased “low-delay” coding structure where we alternate the coding order of the first and second field picture for every pair.

One observation is that commonly PicAFF implementations employ a multi-pass process, i.e. encoding the samples first in frame or field representations, followed by a subsequent encoding in the other mode. Then the encoder selects the one that gives the “best” performance, given a predefined criterion (i.e. based on a lagrangian optimization decision) as the final coding mode. Given our earlier discussions on alternating the coding order of the two field pictures, we can observe that a multi-pass approach could also be used when encoding only in field mode. Here, however, the first representation would be using a “First-field” first encoding, and the second representation using a “second-field” first encoding. Additional considerations on the QP and lambda values that are used for the two field pictures could also be considered. This approach would retain the same complexity as the frame coding approach within the context of PicAFF, while still being within the normative constraints of HEVC v.1. Low complexity, one pass strategies that decide the order and quantization values could also be used, similar to the decision mechanisms other, existing lower complexity interlace encoders use for determining whether a frame or field coding mode should be used for the encoding of these samples. These schemes are however outside the scope of this document.

# Quantizer and Lagrangian Parameter Settings

One aspect of the encoding that seems to be somewhat ignored when testing the HM reference software is what impact a particular Quantizer (QP) and its associated Lagrangian parameter that is used within the rate distortion optimization process may have to the encoding process. Unfortunately, this seems especially true in the context of Interlace encoding.

Looking in particular at the very first image of an interlace sequence, one may observe the following:

- Since that first picture will be intra coded in frame mode, only spatial prediction would essentially be utilized.

- If in field mode, the first field will be using intra prediction, while the second field may also utilize inter prediction. For static or 3:2 pulldown sequences, inter prediction could also be seen as a different form of intra prediction.

- If the content is very static or involves a 3:2 pulldown scenario (such as in the case of the sequence MAD) it is expected that segmenting a frame into field images would result in aliased material that may be harder, especially if the samples are coded as intra, to encode compared to encoding them in frame mode. This is especially true for the first field picture. However, the fact that the second field can use inter prediction could compensate some of that loss.

- Modern codec implementations, such as the HM, use rate distortion optimization techniques for making encoding decisions. These are commonly of the form:	

where D is a measure of distortion, R is an estimate or the actual rate involved in encoding the signal, and  is the lagrangian parameter which controls the coding process and essentially the importance of quality versus bitrate. A lower  value will give higher importance to quality than bitrate versus a higher one and vice versa. Commonly, the decision of the  parameter is based on the quantization parameter given past empirical experiments in other codecs, but may also be based on the coding structure that is used, the distance between references, the importance of the current picture and so on. Unfortunately, these are aspects that have not yet been carefully looked at in the context of HEVC and field coding.

One may question why these points are important. If we examine point (c) what one may immediately observe is that the encoding of a field picture in intra mode, especially for static content, could very likely lead to a higher amount of bits than half of the bits used for encoding the same picture in frame mode and using the same QP. Unfortunately, and given the lagrangian optimization process, this would also likely have the effect that the encoder, assuming an equal  value in both field and frame coding modes, would also try to compensate this increase in bits by making worse coding decisions and compromising quality. This compromise in quality then propagates across subsequent field pictures that could be using this as reference making these pictures more difficult to encode.

| (a) | (b) |
| --- | --- |

Figure 9. Bits and PSNR relationship for the first field picture versus 
the first frame picture in the MAD sequence

The results unfortunately substantiate the above claims. In Figure 9 one can essentially see the bits ratio (a) and the PSNR difference (b) of the very first field picture in the MAD sequence, versus the corresponding numbers in frame mode. The number of bits in field mode are between 69% and 73.5% those of the frame ones, which, given the Lagrangian optimization process, also affect distortion substantially. For the highest QP, the PSNR difference is close to 1.7dB lower for the field-coding mode, vs. that of the frame mode.

To further understand the impact of this behavior one should also examine the coding performance of the entire first frame picture in the sequence MAD, encoded in either frame mode or in field mode (as two field pictures). The rate distortion curve, using the default values in the HM but with equal QP for both fields in field mode, is shown in Figure 10.

Figure 10. Coding performance of the first frame of the sequence MAD, 
using the “default” QP and  values in the HM reference software

What one can immediately see from this figure is that, especially at the higher QP values, both the bitrate and the PSNR in the field mode are substantially lower than those of the frame mode. In the case of QP 37, for example the difference in bitrate seems to be ~17%, resulting in a quality degradation of more than 1dB.  This rate and quality allocation difference, as expected, would have a rather detrimental impact on subsequent pictures that could use these samples as references. This is in fact also true for the second field picture as well since, if that uses a better reference picture for prediction could also improve its quality.

These results suggest that there is a considerable fallacy in how the lambda parameters are derived in the case of field coding. Instead of using a similar determination for the lambda parameters in field mode, especially for the very first picture in the sequence or for all intra pictures, and one should consider a much lower lambda (or QP) when encoding these pictures compared to the one used for frame coding. This should allow field coding to use a more similar amount of bits as those used in frame mode, improve the quality of the related samples, and therefore result in better reference pictures for future prediction. The necessary adjustment appears to be at least QP dependent, but should also depend on the content characteristics among others.

This concept can also be combined and improved with multi-pass encoding. Multi-pass encoding could consider one representation where the two field pictures are encoded using one set of QP values and lambda (i.e. the currently used values in the HM), and a second representation where the QP values and lambda are adjusted to mimic the bitrate and quality behavior of frame mode. The mode that is considered as best, given a frame level rate distortion comparison, is selected for encoding. Note that in more sophisticated encoders, pre-analysis steps may exist that could provide rather accurate information about the characteristics of the image and how that should be best coded.

Although it would likely not perform the same as the “frame level” (or more accurately “complementary field pair level”) multi-pass scheme, one could simply utilize a “field level” multi-pass approach to emulate somewhat the above behavior at a somewhat lower complexity and encoding delay. That is, every field can be coded twice with different coding parameters, e.g. QP and lambda, while also using an “estimate” of its impact to subsequent pictures. This estimate could be based on past coding results or pre-analysis information. Depending on the accuracy of the estimate, this could potentially boost the performance of field coding in the context of HEVC.

# Summary and Recommendations

Without being exhaustive, several limitations and issues have been identified in the context of field encoding using the HM implementation of the HEVC standard. These include, among others:

- reference management,

- reference handling and reordering,

- coding structures and order,

- Bit/Quality allocation and QP/lambda relationships in field coding.

We recommend carefully addressing all these issues since we believe that these should have a substantial impact in coding performance for interlace material. Bit/Quality allocation should also be carefully investigated, and we believe that the consideration of a multi-pass scheme at the complementary field pair/frame level may be advisable. The lagrangian parameters, and potentially the QP values used, especially for intra-coded pictures, need to be carefully adjusted given the very different characteristics of field pictures versus frame pictures.

# Conclusion

This document has provided some insight and suggestions on how field coding could potentially be further improved in the context of HEVC v1. A variety of issues were identified and a few proposed solutions were suggested.

# Acknowledgement

We would like to especially thank Mr. Chad Fogg for several comments as well as insights that he provided in relationship to this document as well as interlace coding in general.

# Patent rights declaration(s)

Apple Inc. does not have any current or pending patent rights relating to the technology described in this contribution.

Obviously, frame picture encoders, although can also use reordering, have less flexibility than field picture encoders given the fewer number of available pictures as well as the characteristics of these pictures.

This observation is only conditional to the restriction that one wishes to retain the pairwise coding order when using the field coding mode in HEVC.

Note that altering the bias could also be done with the structure of Figure 7, while, however altering the QP and lambda values only and not the coding order. We cannot comment which one is better though.

As we suggested earlier, this can be seen as an “alternative” intra prediction mode. It should be noted that there are a few companies that have exploited this concept and designed alternative “still” image codecs with better performance than traditional coding schemes. Naming these companies is though outside the scope of this document.

Note that since we used equal QP values and lambda for both fields, that quality difference is somewhat equally distributed across all samples.

We claim that lambda should be mainly adjusted for every intra picture and we can maybe avoid performing a readjustment for inter pictures (unless these are pictures correspond to a scene change that has not been appropriately detected by an encoder). For normal inter pictures, maybe no or less of an adjustment is needed.

As an example, a pre-analysis module could test the coding behavior of a frame if that was coded, possibly in a low complexity mode or even as a lower resolution, as one frame picture or as two field pictures. This could be done even if the encoder is operating only in field mode. Given the relationship of these two encoding passes, the encoder could then determine the QP values and lambda for a final encoding. Strategies like these are quite common in software encoders such as the x264 encoder.