| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Valencia, ES, 27 March – 4 April 2014 | Document: JCTVC-Q0119r1 |
| --- | --- |

| Title: | Increased resolution for scaled reference layer offset |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Koohyar Minoo, David Baylon, Ajay Luthra 6450 Sequence Drive San Diego, CA 92121 USA | Tel: Email: | +1 (858) 404-2521 koohyar.minoo@arrisi.com david.baylon@arrisi.com ajay.luthra@arrisi.com |
| Source: | ARRIS |  |  |

_____________________________

# Abstract

This contribution presents an approach to signaling phase offsets to improve inter-layer prediction precision and hence the compression performance of SHVC. The proposed method increases the resolution of the scaled reference layer offset to provide phase alignment between layers.

# Introduction

For the case of fixed filters, in order to accommodate for phase shift differences between the BL and EL samples (e.g. due to chroma sampling, field/frame data, etc.), it is proposed that phase offset adjustment parameters be signaled.  Currently, the draft SHVC text [1] only supports left and center alignment between layers, and the signaling to support field to frame scalability is very narrow focused.  The proposed method harmonizes existing scaled reference layer offset signaling with phase offset signaling that allows for more general sub-pixel alignment between layers that includes both horizontal and vertical directions.

# Proposed resolution increase for scaled reference layer offset

In the proposed method, the resolution of the scaled reference layer offset is increased from 2-integer pel.  While the original coarser resolution allows for selection of a region in the scaled reference layer, the additional proposed finer resolution allows for finer local phase offset adjustment between layers.  Corresponding modifications to semantics in the draft SHVC text [1] are indicated below, where deletions are crossed out in yellow, additions are highlighted in green, and moved text is highlighted in blue.

- F.7.3.2.2.1 Sequence parameter set multilayer extension syntax

| sps_multilayer_extension( ) { | Descriptor |
| --- | --- |
| inter_view_mv_vert_constraint_flag | u(1) |
| num_scaled_ref_layer_offsets | ue(v) |
| for( i = 0; i < num_scaled_ref_layer_offsets; i++) { |  |
| scaled_ref_layer_id[ i ] | u(6) |
| scaled_ref_layer_left_offset[ scaled_ref_layer_id[ i ] ] | se(v) |
| scaled_ref_layer_top_offset[ scaled_ref_layer_id[ i ] ] | se(v) |
| scaled_ref_layer_right_offset[ scaled_ref_layer_id[ i ] ] | se(v) |
| scaled_ref_layer_bottom_offset[ scaled_ref_layer_id[ i ] ] | se(v) |
| vert_phase_position_enable_flag[ scaled_ref_layer_id[ i ] ] | u(1) |
| } |  |
| } |  |

- F.7.3.2.3	Picture parameter set RBSP syntax

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| pps_pic_parameter_set_id | ue(v) |
| pps_seq_parameter_set_id | ue(v) |
| … |  |
| pps_extension_flag | u(1) |
| if( pps_extension_flag ) { |  |
| for ( i = 0; i < 8; i++ ) |  |
| pps_extension_type_flag[ i ] | u(1) |
| if( pps_extension_type_flag[ 0 ] ) |  |
| poc_reset_info_present_flag | u(1) |
| if( pps_extension_type_flag[ 1 ] ) |  |
| pps_multilayer_extension( ) |  |
| if( pps_extension_type_flag[ 7 ] ) |  |
| while( more_rbsp_data( ) ) |  |
| pps_extension_data_flag | u(1) |
| } |  |
| rbsp_trailing_bits( ) |  |
| } |  |

- F.7.3.2.3.1	Picture parameter set multilayer extension syntax

| pps_multilayer_extension( ) { | Descriptor |
| --- | --- |
| num_scaled_ref_layer_offsets | ue(v) |
| for( i = 0; i < num_scaled_ref_layer_offsets; i++) { |  |
| scaled_ref_layer_id[ i ] | u(6) |
| scaled_ref_layer_left_offset[ scaled_ref_layer_id[ i ] ] | se(v) |
| scaled_ref_layer_top_offset[ scaled_ref_layer_id[ i ] ] | se(v) |
| scaled_ref_layer_right_offset[ scaled_ref_layer_id[ i ] ] | se(v) |
| scaled_ref_layer_bottom_offset[ scaled_ref_layer_id[ i ] ] | se(v) |
| scaled_ref_layer_left_phase[ scaled_ref_layer_id[ i ] ] | se(v) |
| scaled_ref_layer_top_phase[ scaled_ref_layer_id[ i ] ] | se(v) |
| ref_layer_horizontal_delta[ scaled_ref_layer_id[ i ] ] | se(v) |
| ref_layer_vertical_delta[ scaled_ref_layer_id[ i ] ] | se(v) |
| ref_layer_horizontal_delta_chroma [ scaled_ref_layer_id[ i ] ] | ue(v) |
| ref_layer_vertical_delta_chroma [ scaled_ref_layer_id[ i ] ] | ue(v) |
| } |  |
| scaled_ref_layer_left_phase_chroma | ue(v) |
| scaled_ref_layer_top_phase_chroma | ue(v) |
| } |  |

- F.7.4.3.3	Picture parameter set RBSP semantics

The specifications in subclause 7.4.3.3 apply, with the following modifications:

pps_extension_type_flag[ i ] shall be equal to 0, for i in the range of 1 to 6, inclusive, in bitstreams conforming to this version of this Specification. pps_extension_type_flag[ 0 ] equal to 1 specifies that poc_reset_info_present_flag is present in the PPS RBSP syntax structure. pps_extension_type_flag[ 0 ] equal to 0 specifies that poc_reset_info_present_flag is not present in the PPS RBSP syntax structure. The value of 1 for pps_extension_type_flag[ i ], for i in the range of 2 to 6, inclusive, is reserved for future use by ITU-T | ISO/IEC. pps_extension_type_flag[ 1 ] equal to 1 specifies that the pps_multilayer_extension syntax structure is present. pps_extension_type_flag[ 1 ] equal to 0 specifies that the pps_multilayer_extension syntax structure is not present. The value of 1 for pps_extension_type_flag[ i ], for i in the range of 1 to 7, inclusive, is reserved for future use by ITU-T | ISO/IEC. pps_extension_type_flag[ 7 ] equal to 0 specifies that no pps_extension_data_flag syntax elements are present in the PPS RBSP syntax structure. Decoders shall ignore all pps_extension_data_flag syntax elements that follow the value 1 for pps_extension_type_flag[ 7 ] in an PPS NAL unit.

- F.7.4.3.3.1	Picture parameter set multilayer extension semantics

scaled_ref_layer_left_phase[ scaled_ref_layer_id[ i ] ] specifies the horizontal luma offset between nuh_layer_id equal to scaled_ref_layer_id[ i ] and the current picture in units of ½ luma samples. This is a signed value between -2 to +2. When not present, the value of scaled_ref_layer_left_phase[ scaled_ref_layer_id[ i ] ] is inferred to be equal to 0.

scaled_ref_layer_top_phase[ scaled_ref_layer_id[ i ] ] specifies the vertical luma offset between nuh_layer_id equal to scaled_ref_layer_id[ i ] and the current picture in units of ½ luma samples. This is a signed value between -2 to +2. When not present, the value of scaled_ref_layer_top_phase[ scaled_ref_layer_id[ i ] ] is inferred to be equal to 0.

ref_layer_horizontal_delta[ scaled_ref_layer_id[ i ] ] specifies the horizontal luma offset between nuh_layer_id equal to scaled_ref_layer_id[ i ] and the current picture in units of 1/8 luma samples. This is a signed value between -8 to 8. When not present, the value of ref_layer_horizontal_delta[ scaled_ref_layer_id[ i ] ] is inferred to be equal to 0.

ref_layer_vertical_delta[ scaled_ref_layer_id[ i ] ] specifies the vertical luma offset between nuh_layer_id equal to scaled_ref_layer_id[ i ] and the current picture in units of 1/8 luma samples. This is a signed value between -8 to +8. When not present, the value of ref_layer_vertical_delta[ scaled_ref_layer_id[ i ] ] is inferred to be equal to 0.

ref_layer_horizontal_delta_chroma[ scaled_ref_layer_id[ i ] ] specifies the horizontal offset between the chroma samples and luma samples in nuh_layer_id equal to scaled_ref_layer_id[ i ] in units of ¼ luma samples. This is an unsigned value between 0 to 4. When not present, the value of ref_layer_horizontal_delta_chroma[ scaled_ref_layer_id[ i ] ] is inferred to be equal to 2.

ref_layer_vertical_delta_chroma[ scaled_ref_layer_id[ i ] ] specifies the vertical offset between the chroma samples and luma samples in nuh_layer_id equal to scaled_ref_layer_id[ i ] in units of ¼ luma samples. This is an unsigned value between 0 to 4. When not present, the value of ref_layer_vertical_delta_chroma [ scaled_ref_layer_id[ i ] ] is inferred to be equal to 2.

scaled_ref_layer_left_phase_chroma specifies the horizontal chroma offset relative to luma in units of ¼ luma samples. This is an unsigned value between 0 to 4. When not present, the value of scaled_ref_layer_left_phase_chroma is inferred to be equal to 2.

scaled_ref_layer_top_phase_chroma specifies the vertical chroma offset relative to luma in units of ¼ luma samples. This is an unsigned value between 0 to 4. When not present, the value of scaled_ref_layer_top_phase_chroma is inferred to be equal to 2.

- H.8.1.4	Resampling process for inter-layer reference pictures

The variables ScaledRefLayerLeftOffset, ScaledRefLayerTopOffset, ScaledRefLayerRightOffset and ScaledRefLayerBottomOffset are derived as follows:

ScaledRefLayerLeftOffset = scaled_ref_layer_left_offset[ rLId ] << 1	(H-1)
ScaledRefLayerTopOffset = scaled_ref_layer_top_offset[ rLId ]  << 1	(H-2)
ScaledRefLayerRightOffset = scaled_ref_layer_right_offset[ rLId  ]  << 1	(H-3)
ScaledRefLayerBottomOffset = scaled_ref_layer_bottom_offset[ rLId ]  << 1	(H-4)

ScaledRefLayerLeftPhase  = scaled_ref_layer_left_phase[ rLId ]

ScaledRefLayerTopPhase  = scaled_ref_layer_top_phase[ rLId ]

RefLayerHorizontalDelta  = ref_layer_horizontal_delta [ rLId ]

RefLayerVerticalDelta  = ref_layer_vertical_delta [ rLId ]

RefLayerHorizontalDeltaChroma  = ref_layer_horizontal_delta_chroma [ rLId ]

RefLayerVerticalDeltaChroma  = ref_layer_vertical_delta_chroma [ rLId ]

VertPhasePositionAdjustFlag = vert_phase_position_enable_flag[ rLId ]	(H-5)
VertPhasePositionFlag = vert_phase_position_flag[ rLId ] 	(H-6)

H.6.2	Derivation process for reference layer sample location used in resampling

Inputs to this process are

–	a variable cIdx specifying the color component index,

–	a sample location ( xP, yP ) relative to the top-left sample of the color component of the current picture specified by cIdx.

Output of this process is a sample location ( xRef16, yRef16 ) specifying the reference layer sample location in units of 1/16-th sample relative to the top-left sample of the reference layer picture.

The variables offsetX and offsetY are derived as follows:

offsetX = ScaledRefLayerLeftOffset / ( ( cIdx = = 0)  ?  1 :  SubWidthC)	(H-7)
offsetY = ScaledRefLayerTopOffset / ( ( cIdx = = 0)  ?  1 :  SubHeightC)	(H-8)

The variables phaseX, phaseY, deltaX, and deltaY, addX and addY are derived as follows:

phaseX = ( cIdx  = =  0 ) ? ( cross_layer_phase_alignment_flag << 1 ) : cross_layer_phase_alignment_flag	(H-9)
phaseY = VertPhasePositionAdjustFlag ? ( VertPhasePositionFlag << 2 ) : 
( ( cIdx  = =  0 ) ? ( cross_layer_phase_alignment_flag << 1 ) : cross_layer_phase_alignment_flag + 1)	(H-10)

phaseX = ( cIdx  = =  0 ) ? (ScaledRefLayerLeftPhase <<2)  : (ScaledRefLayerLeftPhase <<1 + scaled_ref_layer_left_phase_chroma)	(H-11)

phaseY = ( cIdx  = =  0 ) ? (ScaledRefLayerTopPhase <<2)  : (ScaledRefLayerTopPhase <<1 + scaled_ref_layer_top_phase_chroma)	(H-12)

deltaX = ( cIdx  = =  0 ) ? (RefLayerHorizontalDelta <<1)  : (RefLayerHorizontalDelta + RefLayerHorizontalDeltaChroma <<1)	(H-13) 
deltaY = ( cIdx  = =  0 ) ? (RefLayerVerticalDelta <<1)  : (RefLayerVerticalDelta + RefLayerVerticalDeltaChroma <<1)	(H-14)

addX = ( ScaleFactorX * phaseX + 24 ) >> 23		(H-15) 
addY = ( ScaleFactorY * phaseY + 24 ) >> 23		(H-16)

The variables xRef16 and yRef16 are derived as follows:

xRef16 = ( ( ( xP – offsetX) * ScaleFactorX + addX + ( 1 << 11 ) ) >> 12 ) – deltaX ( phaseX << 2 )	(H-17)
yRef16 = ( ( ( yP − offsetY) * ScaleFactorY + addY + ( 1 << 11 ) ) >> 12 ) − deltaY ( phaseY << 2 )	(H-18)

Benefits of proposed approach

Some benefits of the proposed approach include:

- Arbitrary phase offsets per filter and color are allowed in each dimension.

- Accommodates top-left sample and center sample position alignment

- Provides for appropriate offsets in field to frame scalablity.

An example of field to frame scalability is from 1080i to 1080p, where a “de-interlaced” frame is generated from a field.  In this case, spatial interpolation in the vertical direction needs to have an appropriate vertical phase offset to accommodate for the different fields (top vs. bottom) and/or for different chroma sampling (e.g. 4:2:0).  For example, for the luma bottom field a phase offset of 0.5 pixel relative to the top field is needed.  Another example which can be properly handled with the proposed signaling is scalability from 1080i to 4k.

# Reference

[1]  J. Chen, J. Boyce, Y. Ye, M. Hannuksela, G. Sullivan, Y. Wang, “High efficiency video coding (HEVC) scalable extension Draft 5,” JCTVC-P1008_v4, January 2014.

# Patent rights declaration(s)

ARRIS Group, Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).