| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Valencia, ES, 27 March – 4 April 2014 | Document: JCTVC-Q0120r1 |
| --- | --- |

| Title: | Re-sampling using existing phase offset flag signaling |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Koohyar Minoo, David Baylon 6450 Sequence Drive San Diego, CA 92121 USA | Tel: Email: | +1 (858) 404-2521 koohyar.minoo@arrisi.com david.baylon@arrisi.com |
| Source: | ARRIS |  |  |

_____________________________

# Abstract

This contribution proposes to modify the semantics of the existing phase offset flag signaling for spatial scalability to accommodate additional phase offset cases which are not currently included.  In particular, the proposed approach includes cases where field to frame scalability is combined with additional spatial scalability, e.g. 1080i to 4k. The suggested correction in phase calculation improves the coding efficiency compared to no vertical phase correction by 5.1% and 0.5% for All Intra and Random access in a conducted test.

# Introduction

The current draft SHVC text [1] provides for left and center alignment between layers.  For field to frame scalability, additional syntax was provided specifically to handle cases such as from 1080i to 1080p.  However, it does not properly handle cases such as 1080i to 4k, which can be considered as having additional scalability from 1080p to 4k after deinterlacing.  This contribution proposes to use the same offset syntax flags (with a change where one is signaled) to cover additional cases such as this.

# Proposed re-sampling using existing phase offset flag signaling

In the current HEVC scalable extension draft [1], phase offset flags are signaled to indicate alignment between layers.  It is proposed that the same three flags currently in [1] be used, but their semantics be modified to accommodate additional phase offset cases.  In addition, it is proposed that the CrossLayerPhaseAlignmentFlag = cross_layer_phase_alignment_flag[rLId] be signaled at the SPS instead of at the VPS.  The semantics of the proposed approach are as follows:

H.6.2	Derivation process for reference layer sample location used in resampling

Inputs to this process are

–	a variable cIdx specifying the color component index,

–	a sample location ( xP, yP ) relative to the top-left sample of the color component of the current picture specified by cIdx.

Output of this process is a sample location ( xRef16, yRef16 ) specifying the reference layer sample location in units of 1/16-th sample relative to the top-left sample of the reference layer picture.

The variables offsetX and offsetY are derived as follows:

offsetX = ScaledRefLayerLeftOffset / ( ( cIdx = = 0)  ?  1 :  SubWidthC)	(H-1)
offsetY = ScaledRefLayerTopOffset / ( ( cIdx = = 0)  ?  1 :  SubHeightC)	(H-2)

When cIdx is 0, variables phaseX, deltaX, phaseY, and deltaY are derived using Table 1 as follows:

phaseX = phaseY = CrossLayerPhaseAlignmentFlag <<1

deltaX = CrossLayerPhaseAlignmentFlag <<3

deltaY = ((CrossLayerPhaseAlignmentFlag <<3)>>VertPhasePositionAdjustFlag) + (VertPhasePositionFlag<<3)

When cIdx is 1, variables phaseX, deltaX, phaseY, and deltaY are derived using Table 2 as follows:

phaseX = CrossLayerPhaseAlignmentFlag

deltaX = CrossLayerPhaseAlignmentFlag << 2

phaseY = CrossLayerPhaseAlignmentFlag + 1

deltaY=((( CrossLayerPhaseAlignmentFlag +1)<<2)>>VertPhasePositionAdjustFlag)+(VertPhasePositionFlag<<3)

The variables phaseX, phaseY, addX and addY, are derived as follows:

phaseX = ( cIdx  = =  0 ) ? ( cross_layer_phase_alignment_flag << 1 ) : cross_layer_phase_alignment_flag	(H-3)
phaseY = VertPhasePositionAdjustFlag ? ( VertPhasePositionFlag << 2 ) : 
( ( cIdx  = =  0 ) ? ( cross_layer_phase_alignment_flag << 1 ) : cross_layer_phase_alignment_flag + 1)	(H-4)

addX = ( ScaleFactorX * phaseX + 2 ) >> 2		(H-5) 
addY = ( ScaleFactorY * phaseY + 2 ) >> 2		(H-6)

The variables xRef16 and yRef16 are derived as follows:

xRef16 = ( ( ( xP − offsetX ) * ScaleFactorX + addX + ( 1 << 11 ) ) >> 12 ) − ( phaseX << 2 ) deltaX	(H-7)
yRef16 = ( ( ( yP − offsetY ) * ScaleFactorY + addY + ( 1 << 11 ) ) >> 12 ) − ( phaseY << 2 ) deltaY	(H-8)

| VertPhasePositionAdjustFlag | CrossLayerPhaseAlignmentFlag | VertPhasePositionFlag | phaseX | deltaX | phaseY | deltaY |
| --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 0 | 1 | 0 | 2 | 8 | 2 | 8 |
| 1 | 0 | 0 | 0 | 0 | 0 | 0 |
| 1 | 0 | 1 | 0 | 0 | 0 | 8 |
| 1 | 1 | 0 | 2 | 8 | 2 | 4 |
| 1 | 1 | 1 | 2 | 8 | 2 | 12 |

Table 1. Derivation of phaseX, deltaX, phaseY, and deltaY when cIdx = 0.

| VertPhasePositionAdjustFlag | CrossLayerPhaseAlignmentFlag | VertPhasePositionFlag | phaseX | deltaX | phaseY | deltaY |
| --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 | 1 | 4 |
| 0 | 1 | 0 | 1 | 4 | 2 | 8 |
| 1 | 0 | 0 | 0 | 0 | 1 | 2 |
| 1 | 0 | 1 | 0 | 0 | 1 | 10 |
| 1 | 1 | 0 | 1 | 4 | 2 | 4 |
| 1 | 1 | 1 | 1 | 4 | 2 | 12 |

Table 2. Derivation of phaseX, deltaX, phaseY, and deltaY when cIdx = 1.

# Test results

Using SCE1 5 BT.2020 [2] sequences, a test was arranged in which the enhancement layer would be 4K 10 bits and the base layer was derived from the 1080p of the BT.2020 provided by SCE1 with additional changes of reducing the bit-depth to 8bits and alternative line dropping to create alternating top and bottom fields. The length of sequences was cut to one-fifth of the original sequence to expedite the simulations.

Attached the performance of the proposed phase calculation is compared with the current implementation of phase derivation (which is only works for 2x-vertical scaling), and also the case where no vertical compensation is performed for base layer pictures containing bottom fields.

# Summary

In order to compensate for phase offsets introduced by downsampling or to maintain proper luma/chroma color space positions after upsampling, it is recommended that SHVC allow for upsampling with additional phase offset adjustments.  An approach was described that changes the semantics of the same three offset flags currently used in [1] to accommodate additional phase offsets to include cases such as 1080i to 4k.

# References

[1]  J. Chen, J. Boyce, Y. Ye, M. Hannuksela, G. Sullivan, Y. Wang, “High efficiency video coding (HEVC) scalable extension Draft 5,” JCTVC-P1008_v4, January 2014.

[2] “Description of HEVC Scalable Extensions Core Experiment SCE1: Color Gamut and Bit-Depth Scalability”, JCTVC-P1101, January 2014.

# Patent rights declaration(s)

ARRIS Group, Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).