| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Valencia, ES, 27 March – 4 April 2014 | Document: JCTVC-Q0122-1 |
| --- | --- |

| Title: | Increased precision for deriving the reference layer sample location |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Koohyar Minoo, David Baylon 6450 Sequence Drive San Diego, CA 92121 USA | Tel: Email: | +1 (858) 404-2521 koohyar.minoo@arrisi.com david.baylon@arrisi.com |
| Source: | ARRIS |  |  |

_____________________________

# Abstract

It was observed in [1] that the current precision in computing the reference layer sample location can lead to errors in filter phase selection, especially as the picture size increases.  It is proposed that the precision be increased without changing the maximum bit depth.

# Problem:

Currently in SHVC the division free scaling is achieved by shifting the scaling factor by 16bits, to  reduce the scaling error (if it exists) by a factor of 2-12 (in the 1/16 sub-pixel domain).

There are two problems

- As the pixel position in the EL gets multiplied by this error, the multiplied error should be kept small enough so the filter decision is will not change due to this magnified error become greater than an error margin which yields wrong filter index.

- The possibility of overflowing 32 bit registers exist for picture sizes bigger than 215.

Illustration of the accuracy problem:

In the general case when the shift is N and the rounding error is err  the magnified error is:

magnified_error = err * 2-(N-4) * pixel_position

The maximum error before crossing the 1/2 position starting from the intial phase offset is:

error_margin = 1/2 - initial_phase_offset

First miss classified pixel is where the magnified_error becomes bigger than error_margin.

first_miss_classified_pixel = (error_margin / err) * 2-(N-4)

For example, for the case of 1.5x scalability:

error = 1/3

There are three sub-pixel location at 0, 1/3 and 2/3. The lowest error margin

initial_phase_offset  = 1/3 (which is currently mapped to filter index 5)

error_margin = 1/2 -1/3 = 1/6

first_miss_classified_pixel = ((1/6) / (1/3)) / 2-12 = 211 = 2048

Therefore one every three pixel is affected after 2048 pixel.

The second lowest error margin is associated with 0 phase :

initial_phase_offset  = 0 (which is currently mapped to filter index 0)

error_margin = 1/2 -0 = 1/2

first_miss_classified_pixel = ((1/2) / (1/3)) / 2-12 = 3*211 = 6144

The third lowest error margin is associate with 2/3 phase:

initial_phase_offset  = -1/3 (which is currently mapped to filter index 11)

error_margin = 1/2 –(-1/3) = 5/6

first_miss_classified_pixel = ((5/6) / (1/3)) / 2-12 = 5*211 = 10240

# Proposed increased precision for deriving the reference layer sample location

It is proposed to increase the precision in the reference layer sample location computations in order to provide accurate phase offset calculations.  The precision for calculating the reference layer sample location is increased to take advantage of the maximum bit depth, e.g. 32-bit.  The proposed changes to the semantics in [2] are as follows:

- H.8.1.4	Resampling process for inter-layer reference pictures

The variables shiftX and shiftY are defined as:

shiftX  = 31 – Ceil( Log2 (RefLayerPicWidthInSamplesY));

shiftY  = 31 – Ceil( Log2 (RefLayerPicHeightInSamplesY));

The variables ScaleFactorX and ScaleFactorY are derived as follows:

ScaleFactorX = ( ( RefLayerPicWidthInSamplesY << 16 shiftX ) + ( ScaledRefLayerPicWidthInSamplesY >> 1 ) ) / 
		ScaledRefLayerPicWidthInSamplesY		(H-1)
ScaleFactorY = ( ( RefLayerPicHeightInSamplesY << 16 shiftY ) + ( ScaledRefLayerPicHeightInSamplesY >> 1 ) ) / 
		ScaledRefLayerPicHeightInSamplesY		(H-2)

H.6.1	Derivation process for reference layer sample location

Input to this process is a luma location ( xP, yP ) relative to the top-left luma sample of the current picture.

Output of this process is a luma location ( xRef, yRef ) relative to the top-left luma sample of the reference layer picture.

The variables xRef and yRef are derived as follows:

xRef = ( ( xP − ScaledRefLayerLeftOffset ) * ScaleFactorX + ( 1 << 15 (shiftX – 1) ) ) >> 16 shiftX 	(H-1)
yRef = ( ( yP − ScaledRefLayerTopOffset ) * ScaleFactorY + ( 1 << 15 (shiftY – 1) ) ) >> 16 shiftY 	(H-2)

H.6.2	Derivation process for reference layer sample location used in resampling

The variables phaseX, phaseY, addX and addY are derived as follows:

phaseX = ( cIdx  = =  0 ) ? ( cross_layer_phase_alignment_flag << 1 ) : cross_layer_phase_alignment_flag	(H-3)
phaseY = VertPhasePositionAdjustFlag ? ( VertPhasePositionFlag << 2 ) : 
( ( cIdx  = =  0 ) ? ( cross_layer_phase_alignment_flag << 1 ) : cross_layer_phase_alignment_flag + 1)	(H-4)

addX = ( ScaleFactorX * phaseX + 2 ) >> 2 		(H-5) 
addY = ( ScaleFactorY * phaseY + 2 ) >> 2 		(H-6)

The variables xRef16 and yRef16 are derived as follows:

xRef16 = ( ( ( xP − offsetX ) * ScaleFactorX + addX + ( 1 << 11 (shiftX – 5)  ) ) >> 12 (shiftX – 4) ) − ( phaseX << 2 )	(H-7)
yRef16 = ( ( ( yP − offsetY ) * ScaleFactorY + addY + ( 1 << 11 (shiftY– 5) ) ) >> 12 (shiftY – 4) ) − ( phaseY << 2 )	(H-8)

# Summary

In order to compensate for phase offsets introduced by downsampling or to maintain proper luma/chroma color space positions after upsampling, it is recommended that SHVC allow for upsampling with increased precision in the calculation of the reference layer sample locations.

# Reference

[1]  K. Minoo, D. Baylon, A. Luthra, “On handling re-sampling phase offsets with fixed filters”, JCTVC-P0177, 16th JCT-VC Meeting, San Jose, USA, January 2014.

[2]  J. Chen, J. Boyce, Y. Ye, M. Hannuksela, G. Sullivan, Y. Wang, “High efficiency video coding (HEVC) scalable extension Draft 5,” JCTVC-P1008_v4, January 2014.

# Patent rights declaration(s)

ARRIS Group, Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).