| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Valencia, ES, 27 March – 4 April 2014 | Document: JCTVC-Q0123 |
| --- | --- |

| Title: | SCE1: Crosscheck for Color gamut scalability using gain-offset models (JCTVC-0072) Test 2 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Koohyar Minoo 6450 Sequence Drive San Diego, CA 92121 USA | Tel: Email: | +1 (858) 404-2521 koohyar.minoo@arrisi.com |
| Source: | ARRIS |  |  |

_____________________________

# Abstract

This contribution reports the cross-check comments and results for the Test1.2 of SCE1, titled “Linear matrix mapping (N=1)”.

# Introduction

The core experiment to study the coding of efficiency of different proposals for color gamut scalability was conducted under SCE1. Test1.2, which is described in the SCE1 test description [1], is the subject of this cross-check.

Test1.2 utilizes a 3D linear model with offset to predict samples from a base layer in the process of inter-layer sample prediction. The 12 parameters of this model are signalled in the PPS and can be updated by transmitting a new PPS in the bit-stream.

# Additional comments

It appears that for this coding tool the advantage of smaller foot-print for signalling the transfer function (12 parameters vs. up to 8x8x8x12 or 8x2x2x12 parameters) is not fully exploited. In the examined source code the signalling happens about every one second. The study of a more frequent signalling (e.g. per picture) seem to be of value.

# Experimental results

For generating the anchor the software provided by SCE1 chairs based on SHM5.1 with additional patch to fix different bit-depths for SAO process and the ability to signal sps/pps per configurable interval was used. For the test the software provided by the proponents where used with the suggested compiler directives to conform to the tool described in Test1.2 of [1].

The test results as provided in the attached MS-Excel file match those provided by the proponents of Test1.2 for that test. It was noted that compared to the anchor software, the source code provided for Test1.2 removes the configurable periodic signalling of SPS/PPS in the based layer.

Note that complexity analysis was done by the SCE1 chairs and this contribution does not contain any comments about the complexity analysis of Test1.2 and the proposed metrics used for that analysis.

# Reference

[1] “Description of HEVC Scalable Extensions Core Experiment SCE1: Color Gamut and Bit-Depth Scalability”, JCTVC-P1101, Jan 2014

# Patent rights declaration(s)

ARRIS Group, Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).