| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Valencia, ES, 27 March – 4 April 2014 | Document: JCTVC-Q0124 |
| --- | --- |

| Title: | Improvements on 1D dictionary coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jing Ye, Shan Liu, Shawmin Lei Xianyi Chen, Liping Zhao, Tao Lin 2860 Junction Ave, San Jose, CA 95134, USA 1239 Siping Ave, Shanghai 200092, China | Email: | jing.ye@mediatek.com shan.liu@mediatek.com lintao@tongji.edu.cn |
| Source: | MediaTek Inc. Tongji University |  |  |

_____________________________

# Abstract

This document presents a couple of improvements on 1D dictionary coding technique. Firstly 1D dictionary is implemented in CU level. Secondly the binarization and coding of the matching length is modified for enhancing coding efficiency. The proposed implementation example using HM-13.0+RExt-6.0 as anchor reports -13.1%, -8.0%, and -4.8% BD-rate savings for SC RGB sequences under AI, RA, and LB, respectively.

# Introduction

Dictionary dual-coder is first officially proposed by Tongji University [1] in the 11th JCT-VC meeting. Then several companies are interested in the dictionary dual coder. They proposed improvements to the 1D dictionary tool or integrated it with other tools. In the 16th JCT-VC meeting, in January 2014 in San Jose, Microsoft proposed a 2D dictionary tool [2]. 1D dictionary coding is based on string matching. For the matched strings, the distance from matched pixel position to current pixel position and matched length will be coded.

The current 1D dictionary is implemented in LCU level, and it is observed that 1D dictionary implemented in CU level could bring some benefits, such as flexibility and compatibility. The current 1D dictionary codes length according to the length value, for example, if the length is smaller than 8, it will use 3 bits, and if the length is smaller than 16 and bigger than 8, it will use 3 bits, then the rest will use 8 bits. It is observed that there are some statistic features of the length distributions, which could give coding performance an improvement. Since current 1D dictionary coding codes each frame in interleaved format, there are more lengths are multiple of 3, which could give encoding performance an improvement if the bits for those lengths which are multiples of 3 could be reduced.

To address these issues, in this document, CU level 1D dictionary and a new method for length coding are proposed.

# Proposed method

## Method 1 CU level 1D dictionary

For a CU coded by 1D dictionary mode, a flag “intra_p2m_flag” is used to indicate the dictionary mode. The searching for matching distance and length uses CU as the basic unit. The searching range is determined by the dictionary size, and the dictionary size is specified by Table 1. The searching could across CU or LCU boundary. Horizontal and Vertical scan order could be both supported in current implementation.

| Level | Dictionary size(byte) |
| --- | --- |
| 1 | 4096 |
| 2 | 8192 |
| 3 | 16384 |
| 4 | 65536 |
| 5 | 262144 |
| 6 | 1048576 |
| 7 | 4194304 |

Table1. Dictionary level and size

The SPS syntax is modified as follows:

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| video_parameter_set_id | u(4) |
| <snip ……> |  |
| intra_p2m_enabled_flag | ue(v) |
| log2_max_pic_order_cnt_lsb_minus4 | ue(v) |
| <snip ……> |  |
| rbsp_trailing_bits( ) |  |
| } |  |

The cu syntax is modified as follows:

| coding_unit( x0, y0, log2CbSize ) { | Descriptor |
| --- | --- |
| … |  |
| else { |  |
| if( intra_block_copy_enabled_flag ) |  |
| intra_bc_flag[ x0 ][ y0 ] | ae(v) |
| if( !intra_bc_flag[ x0 ][ y0 ] ) { |  |
| if(intra_p2m_enabled_flag ) |  |
| intra_p2m_flag[ x0 ][ y0 ] | ae(v) |
| if(intra_p2m_flag[ x0 ][ y0 ] ) |  |
| p2m_coder( x0, y0, log2CbSize ) |  |
| } |  |
| if( !intra_p2m_flag [ x0 ][ y0 ] ){ |  |
| if( slice_type != I && !intra_bc_flag[ x0 ][ y0 ]) |  |
| pred_mode_flag | ae(v) |
| if( CuPredMode[ x0 ][ y0 ] != MODE_INTRA ¦ ¦ log2CbSize = = MinCbLog2SizeY) |  |
| part_mode | ae(v) |
| if( CuPredMode[ x0 ][ y0 ] = = MODE_INTRA ) { |  |
| if( PartMode = = PART_2Nx2N && pcm_enabled_flag && !intra_bc_flag log2CbSize >= Log2MinIpcmCbSizeY && log2CbSize <= Log2MaxIpcmCbSizeY ) |  |
| pcm_flag[ x0 ][ y0 ] | ae(v) |
| … |  |
| if( !pcm_flag[ x0 ][ y0 ] ) { |  |
| if( CuPredMode[ x0 ][ y0 ] != MODE_INTRA && !( PartMode = = PART_2Nx2N && merge_flag[ x0 ][ y0 ] ) ¦ ¦ CuPredMode[ x0 ][ y0 ] = = MODE_INTRA && intra_bc_flag[ x0 ][ y0 ] ) |  |
| rqt_root_cbf | ae(v) |
| if( rqt_root_cbf ) { |  |
| … |  |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |

p2m_coder( x0, y0, log2CbSize ) {

decoded_sample_count = 0

NumSamplesInCU = (3 << (log2CbSize << 1))

while(decoded_sample_count < NumSamplesInCU) {

matching_string_flag

if(matching_string_flag = = 1) {

unmatchable_sample_residual_abs

unmatchable_sample_residual_sign

decoded_sample_count += 1

}

else {

matching_string_distance_use_recent_8_flag

if(matching_string_distance_use_recent_8_flag = = 1) {

matching_string_distance_recent_8_idx

matching_string_distance_minus1 = Recent8Distances[matching_string_distance_recent_8_idx]

for( i = matching_string_distance_recent_8_idx; i > 0; i-- )

Recent8Distances[i] = Recent8Distances[i-1]

Recent8Distances[0] = matching_string_distance_minus1

}

else {

matching_string_distance_minus1

for( i = 7; i > 0; i-- )

Recent8Distances[i] = Recent8Distances[i-1]

Recent8Distances[0] = matching_string_distance_minus1

}

matching_string_length_minus2

decoded_sample_count += (matching_string_length_minus2 + 2)

}

}

}

## Method 2 modified length coding techique

To parse a CU coded by 1D dictionary mode, the decoder will first parse matching_string_flag to determine if there is a match. If matching_string_flag =1, the distance and length will be parsed. For a matched length, smaller_than_27_and_divided_by3_flag indicates the length is a multiple of 3 and also located in range [3, 24] and bigger_than_251_and_divided_by3_flag indicates the length is a multiple of 3 and located in the range [252, 273]. Then the decoder will parse 3 bits to get the length value. If smaller_than_27_and_divided_by3_flag =0 and bigger_than_251_and_divided_by3_flag =0, the decoder will parse 8 bits to get the length value. The following pseudo-code summarizes the decoding length process.

Decode smaller_than_27_and_divided_by3_flag

if ( smaller_than_27_and_divided_by3_flag==0)

{

decode 3 bits length;

length=length*3+3;

}

else

{

Decode bigger_than_251_and_divided_by3_flag

if (bigger_than_251_and_divided_by3_flag==0)

{

decode 3 bits length;

length=length*3+252;

}

else

{

decode 8 bits length;

}

}

uDecByteCnt += length;

# Simulation results

As an implementation example, the proposed methods were tested using HM-13.0+RExt-6.0 as anchor. The encoder will evaluate two tools of 1D dictionary mode coding: CU level dictionary and CU level dictionary with modified length coding. All the results were generated under the common test conditions and software reference configurations defined in for RExt [5] with SC CFP sequences. The encoding and decoding runtime is not reliable.

## CU level 1D dictionary using the length coding method in[1]

Lossy and lossless coding results of the proposed method using HM-13.0+RExt-6.0 as the anchor are shown in Table 2, where -11.5%, -6.8%, and -3.8% BD-rate savings are shown for SC RGB sequences under AI, RA, and LB, respectively.

Lossy coding results with clipped PSNR of the proposed method using HM -13.0+RExt-6.0 as anchor are also provided. The results shown in Table 2 shows a little BD variation compared with the non-clipped one.

## CU level 1D dictionary using new length coding method

Lossy and lossless coding results of the proposed method using HM-13.0+RExt-6.0 as the anchor are shown in Table 3, where -13.1%, -8.0%, and -4.8% BD-rate savings are shown for SC RGB sequences under AI, RA, and LB, respectively.

Lossy coding results with clipped PSNR of the proposed method using HM -13.0+RExt-6.0 as anchor are also provided. The results shown in Table 3 shows a little BD variation compared with the non-clipped one.

## New length coding method gain on top of CU level 1D dictionary

Lossy and lossless coding results of the proposed method using the result from section 3.1 as the anchor are shown in Table 4, where -2.5%, -1.3%, and -0.8% BD-rate savings are shown for SC YUV sequences under AI, RA, and LB, respectively.

Lossy coding results with clipped PSNR of the proposed method using HM -13.0+RExt-6.0 as anchor are also provided. The results shown in Table 4 shows a little BD variation compared with the non-clipped one.

# Conclusions

This document presents CU level dictionary with new length encoding method. Results on HM-13.0+RExt-6.0 as the anchor demonstrate -13.1%, -8.0%, and -4.8% BD-rate savings for SC RGB sequences under AI-Main-Tier, RA-Main-Tier, and LB-Main-Tier, respectively.

# References

[1]. T. Lin, S. Wang, P. Zhang, K. Zhou, “AHG7: Full-chroma (YUV444) dictionary+hybrid dual-coder extension of HEVC”, JCT-VC Document, JCTVC-K0133, Shanghai, China, October 2012.

[2]Bin Li, Jizheng Xu, Feng Wu, “Screen content coding using dictionary based mode”, JCT-VC Document, JCTVC-P0214, San José, US, 9–17 Jan. 2014.

[3]. Tao Lin, Shuhui Wang, Peijun Zhang, Kailun Zhou, “AHG8: P2M based dual-coder extension of HEVC”, JCT-VC Document, JCTVC-L0303, Geneva, Switzerland,14–23 Jan. 2013.

[4]. D. Flynn, J. Sole, T. Suzuki, “High Efficiency Video Coding (HEVC) Range Extension text specification: Draft 4”, JCTVC-N1005, Vienna, AT, July 2013.

[5]. Chris Rosewarne, Karl Sharman, David Flynn, “Common test conditions and software reference configurations for HEVC range extensions”,  JCT-VC Document, JCTVC-P0214, San José, US, 9–17 Jan. 2014.

Table 2. CU level dictionary
Anchor: HM-13.0+RExt-6.0

Lossy

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| RGB, text & graphics with motion, 1080p | -11.5% | -12.4% | -11.8% |
| RGB, text & graphics with motion,720p | -5.6% | -5.4% | -5.6% |
| RGB, mixed content, 1440p | -0.8% | -0.6% | -0.6% |
| RGB, mixed content, 1080p | -0.6% | -0.5% | -0.5% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -7.9% | -9.2% | -9.0% |
| YUV, text & graphics with motion,720p | -2.5% | -4.5% | -5.6% |
| YUV, mixed content, 1440p | -0.1% | -0.4% | -0.2% |
| YUV, mixed content, 1080p | -0.3% | -0.2% | -0.1% |
| YUV, Animation, 720p | 0.1% | 0.0% | 0.0% |
| Enc Time[%] | 109% |  |  |
| Dec Time[%] | 109% |  |  |
|  | Random Access |  |  |
|  | Y | U | V |
| RGB, text & graphics with motion, 1080p | -6.6% | -7.3% | -7.0% |
| RGB, text & graphics with motion,720p | -6.8% | -6.4% | -6.6% |
| RGB, mixed content, 1440p | -1.8% | -1.5% | -1.6% |
| RGB, mixed content, 1080p | -0.6% | -0.6% | -0.6% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -5.6% | -8.3% | -8.1% |
| YUV, text & graphics with motion,720p | -3.3% | -6.6% | -8.1% |
| YUV, mixed content, 1440p | -0.5% | -1.2% | -1.0% |
| YUV, mixed content, 1080p | -0.4% | -0.7% | -0.5% |
| YUV, Animation, 720p | 0.2% | 0.0% | -0.1% |
| Enc Time[%] | 108% |  |  |
| Dec Time[%] | 131% |  |  |
|  | Low delay B |  |  |
|  | Y | U | V |
| RGB, text & graphics with motion, 1080p | -3.6% | -4.2% | -3.9% |
| RGB, text & graphics with motion,720p | -3.8% | -3.4% | -3.6% |
| RGB, mixed content, 1440p | -1.5% | -1.6% | -1.4% |
| RGB, mixed content, 1080p | -0.4% | -0.3% | -0.2% |
| RGB, Animation, 720p | 0.1% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -2.5% | -4.8% | -4.6% |
| YUV, text & graphics with motion,720p | -1.5% | -3.5% | -4.9% |
| YUV, mixed content, 1440p | -0.5% | -1.4% | -0.8% |
| YUV, mixed content, 1080p | -0.1% | -0.1% | -0.1% |
| YUV, Animation, 720p | 0.2% | 0.0% | 0.0% |
| Enc Time[%] | 106% |  |  |
| Dec Time[%] | 134% |  |  |

Clipped CU level dictionary
Anchor: HM-13.0+RExt-6.0

Lossy

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| RGB, text & graphics with motion, 1080p | -11.5% | -12.4% | -11.8% |
| RGB, text & graphics with motion,720p | -5.6% | -5.4% | -5.6% |
| RGB, mixed content, 1440p | -0.8% | -0.6% | -0.6% |
| RGB, mixed content, 1080p | -0.6% | -0.5% | -0.5% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -7.9% | -9.2% | -9.0% |
| YUV, text & graphics with motion,720p | -2.5% | -4.5% | -5.6% |
| YUV, mixed content, 1440p | -0.1% | -0.4% | -0.2% |
| YUV, mixed content, 1080p | -0.3% | -0.2% | -0.1% |
| YUV, Animation, 720p | 0.1% | 0.0% | 0.0% |
| Enc Time[%] | 108% |  |  |
| Dec Time[%] | 109% |  |  |
|  | Random Access |  |  |
|  | Y | U | V |
| RGB, text & graphics with motion, 1080p | -6.6% | -7.3% | -7.0% |
| RGB, text & graphics with motion,720p | -6.8% | -6.4% | -6.6% |
| RGB, mixed content, 1440p | -1.8% | -1.5% | -1.6% |
| RGB, mixed content, 1080p | -0.6% | -0.6% | -0.6% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -5.6% | -8.3% | -8.1% |
| YUV, text & graphics with motion,720p | -3.3% | -6.7% | -8.1% |
| YUV, mixed content, 1440p | -0.5% | -1.2% | -1.0% |
| YUV, mixed content, 1080p | -0.4% | -0.7% | -0.5% |
| YUV, Animation, 720p | 0.2% | 0.0% | -0.1% |
| Enc Time[%] | 108% |  |  |
| Dec Time[%] | 133% |  |  |
|  | Low delay B |  |  |
|  | Y | U | V |
| RGB, text & graphics with motion, 1080p | -3.6% | -4.2% | -3.9% |
| RGB, text & graphics with motion,720p | -3.8% | -3.4% | -3.6% |
| RGB, mixed content, 1440p | -1.5% | -1.6% | -1.4% |
| RGB, mixed content, 1080p | -0.4% | -0.3% | -0.2% |
| RGB, Animation, 720p | 0.1% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -2.5% | -4.8% | -4.6% |
| YUV, text & graphics with motion,720p | -1.5% | -3.5% | -4.9% |
| YUV, mixed content, 1440p | -0.5% | -1.4% | -0.8% |
| YUV, mixed content, 1080p | -0.1% | -0.1% | -0.1% |
| YUV, Animation, 720p | 0.2% | 0.0% | 0.0% |
| Enc Time[%] | 106% |  |  |
| Dec Time[%] | 136% |  |  |

Lossless

Table 3.  CU level dictionary with new length encoding method
Anchor: HM-13.0+RExt-6.0

Lossy

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| RGB, text & graphics with motion, 1080p | -13.1% | -14.1% | -13.5% |
| RGB, text & graphics with motion,720p | -6.6% | -6.5% | -6.7% |
| RGB, mixed content, 1440p | -0.9% | -0.7% | -0.7% |
| RGB, mixed content, 1080p | -0.7% | -0.6% | -0.6% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -10.1% | -11.3% | -11.2% |
| YUV, text & graphics with motion,720p | -3.3% | -6.0% | -7.3% |
| YUV, mixed content, 1440p | -0.2% | -0.5% | -0.3% |
| YUV, mixed content, 1080p | -0.4% | -0.3% | -0.2% |
| YUV, Animation, 720p | 0.1% | 0.0% | 0.0% |
| Enc Time[%] | 109% |  |  |
| Dec Time[%] | 108% |  |  |
|  | Random Access |  |  |
|  | Y | U | V |
| RGB, text & graphics with motion, 1080p | -7.6% | -8.4% | -8.2% |
| RGB, text & graphics with motion,720p | -8.0% | -7.5% | -7.8% |
| RGB, mixed content, 1440p | -2.0% | -1.6% | -1.7% |
| RGB, mixed content, 1080p | -0.7% | -0.7% | -0.6% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -6.8% | -10.0% | -9.7% |
| YUV, text & graphics with motion,720p | -4.1% | -7.8% | -9.3% |
| YUV, mixed content, 1440p | -0.7% | -1.5% | -1.1% |
| YUV, mixed content, 1080p | -0.6% | -0.8% | -0.6% |
| YUV, Animation, 720p | 0.2% | 0.0% | -0.1% |
| Enc Time[%] | 108% |  |  |
| Dec Time[%] | 131% |  |  |
|  | Low delay B |  |  |
|  | Y | U | V |
| RGB, text & graphics with motion, 1080p | -4.5% | -5.3% | -4.9% |
| RGB, text & graphics with motion,720p | -4.8% | -4.3% | -4.5% |
| RGB, mixed content, 1440p | -1.8% | -1.6% | -1.5% |
| RGB, mixed content, 1080p | -0.3% | -0.3% | -0.2% |
| RGB, Animation, 720p | 0.1% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -3.3% | -6.1% | -5.8% |
| YUV, text & graphics with motion,720p | -1.8% | -4.6% | -6.0% |
| YUV, mixed content, 1440p | -0.6% | -1.5% | -1.0% |
| YUV, mixed content, 1080p | -0.2% | -0.3% | -0.2% |
| YUV, Animation, 720p | 0.2% | 0.0% | 0.0% |
| Enc Time[%] | 106% |  |  |
| Dec Time[%] | 132% |  |  |

Clipped CU level dictionary with new length encoding method
Anchor: HM-13.0+RExt-6.0

Lossy

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| RGB, text & graphics with motion, 1080p | -13.1% | -14.1% | -13.5% |
| RGB, text & graphics with motion,720p | -6.6% | -6.5% | -6.7% |
| RGB, mixed content, 1440p | -0.9% | -0.7% | -0.7% |
| RGB, mixed content, 1080p | -0.7% | -0.6% | -0.6% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -10.1% | -11.3% | -11.2% |
| YUV, text & graphics with motion,720p | -3.3% | -6.0% | -7.3% |
| YUV, mixed content, 1440p | -0.2% | -0.5% | -0.3% |
| YUV, mixed content, 1080p | -0.4% | -0.3% | -0.2% |
| YUV, Animation, 720p | 0.1% | 0.0% | 0.0% |
| Enc Time[%] | 108% |  |  |
| Dec Time[%] | 108% |  |  |
|  | Random Access |  |  |
|  | Y | U | V |
| RGB, text & graphics with motion, 1080p | -7.6% | -8.4% | -8.2% |
| RGB, text & graphics with motion,720p | -8.0% | -7.5% | -7.8% |
| RGB, mixed content, 1440p | -2.0% | -1.6% | -1.7% |
| RGB, mixed content, 1080p | -0.7% | -0.7% | -0.6% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -6.8% | -10.0% | -9.7% |
| YUV, text & graphics with motion,720p | -4.1% | -7.8% | -9.3% |
| YUV, mixed content, 1440p | -0.7% | -1.5% | -1.1% |
| YUV, mixed content, 1080p | -0.6% | -0.8% | -0.6% |
| YUV, Animation, 720p | 0.2% | 0.0% | -0.1% |
| Enc Time[%] | 108% |  |  |
| Dec Time[%] | 131% |  |  |
|  | Low delay B |  |  |
|  | Y | U | V |
| RGB, text & graphics with motion, 1080p | -4.5% | -5.3% | -4.9% |
| RGB, text & graphics with motion,720p | -4.7% | -4.3% | -4.5% |
| RGB, mixed content, 1440p | -1.8% | -1.6% | -1.5% |
| RGB, mixed content, 1080p | -0.3% | -0.3% | -0.2% |
| RGB, Animation, 720p | 0.1% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -3.3% | -6.1% | -5.8% |
| YUV, text & graphics with motion,720p | -1.8% | -4.6% | -6.0% |
| YUV, mixed content, 1440p | -0.6% | -1.5% | -1.0% |
| YUV, mixed content, 1080p | -0.2% | -0.3% | -0.2% |
| YUV, Animation, 720p | 0.2% | 0.0% | 0.0% |
| Enc Time[%] | 106% |  |  |
| Dec Time[%] | 132% |  |  |

lossless

Table 4.  CU level dictionary with new length encoding method
Anchor: CU level dictionary

Lossy

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| RGB, text & graphics with motion, 1080p | -1.7% | -1.8% | -1.8% |
| RGB, text & graphics with motion,720p | -1.2% | -1.3% | -1.3% |
| RGB, mixed content, 1440p | -0.1% | -0.1% | -0.1% |
| RGB, mixed content, 1080p | -0.1% | -0.1% | -0.1% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -2.5% | -2.4% | -2.4% |
| YUV, text & graphics with motion,720p | -1.0% | -1.8% | -1.9% |
| YUV, mixed content, 1440p | -0.1% | -0.1% | -0.1% |
| YUV, mixed content, 1080p | -0.1% | -0.1% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 99% |  |  |
|  | Random Access |  |  |
|  | Y | U | V |
| RGB, text & graphics with motion, 1080p | -1.0% | -1.2% | -1.1% |
| RGB, text & graphics with motion,720p | -1.3% | -1.2% | -1.3% |
| RGB, mixed content, 1440p | -0.2% | -0.1% | -0.1% |
| RGB, mixed content, 1080p | -0.1% | -0.1% | -0.1% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -1.3% | -1.7% | -1.6% |
| YUV, text & graphics with motion,720p | -1.0% | -1.5% | -1.5% |
| YUV, mixed content, 1440p | -0.2% | -0.3% | -0.2% |
| YUV, mixed content, 1080p | -0.2% | -0.2% | -0.1% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 100% |  |  |
|  | Low delay B |  |  |
|  | Y | U | V |
| RGB, text & graphics with motion, 1080p | -0.9% | -1.0% | -1.0% |
| RGB, text & graphics with motion,720p | -1.0% | -1.0% | -1.0% |
| RGB, mixed content, 1440p | -0.2% | 0.0% | -0.1% |
| RGB, mixed content, 1080p | 0.1% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -0.8% | -1.2% | -1.3% |
| YUV, text & graphics with motion,720p | -0.4% | -1.4% | -1.3% |
| YUV, mixed content, 1440p | -0.1% | -0.1% | -0.3% |
| YUV, mixed content, 1080p | -0.1% | -0.2% | -0.2% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 98% |  |  |

Clipped CU level dictionary with new length encoding method
Anchor: CU level dictionary

Lossy

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| RGB, text & graphics with motion, 1080p | -1.7% | -1.8% | -1.8% |
| RGB, text & graphics with motion,720p | -1.2% | -1.3% | -1.3% |
| RGB, mixed content, 1440p | -0.1% | -0.1% | -0.1% |
| RGB, mixed content, 1080p | -0.1% | -0.1% | -0.1% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -2.5% | -2.4% | -2.4% |
| YUV, text & graphics with motion,720p | -1.0% | -1.8% | -1.9% |
| YUV, mixed content, 1440p | -0.1% | -0.1% | -0.1% |
| YUV, mixed content, 1080p | -0.1% | -0.1% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 99% |  |  |
|  | Random Access |  |  |
|  | Y | U | V |
| RGB, text & graphics with motion, 1080p | -1.0% | -1.2% | -1.1% |
| RGB, text & graphics with motion,720p | -1.3% | -1.2% | -1.3% |
| RGB, mixed content, 1440p | -0.2% | -0.1% | -0.1% |
| RGB, mixed content, 1080p | -0.1% | -0.1% | -0.1% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -1.3% | -1.7% | -1.6% |
| YUV, text & graphics with motion,720p | -1.0% | -1.5% | -1.5% |
| YUV, mixed content, 1440p | -0.2% | -0.3% | -0.2% |
| YUV, mixed content, 1080p | -0.2% | -0.2% | -0.1% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 98% |  |  |
|  | Low delay B |  |  |
|  | Y | U | V |
| RGB, text & graphics with motion, 1080p | -0.9% | -1.0% | -1.0% |
| RGB, text & graphics with motion,720p | -1.0% | -1.0% | -1.0% |
| RGB, mixed content, 1440p | -0.2% | 0.0% | -0.1% |
| RGB, mixed content, 1080p | 0.1% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -0.8% | -1.2% | -1.3% |
| YUV, text & graphics with motion,720p | -0.4% | -1.3% | -1.3% |
| YUV, mixed content, 1440p | -0.1% | -0.1% | -0.3% |
| YUV, mixed content, 1080p | -0.1% | -0.2% | -0.2% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 97% |  |  |

Lossless

# Patent rights declaration(s)

MediaTek Inc. Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Tongji University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).