| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Valencia, ES, 27 March – 4 April 2014 | Document: JCTVC-Q0125_r2 |
| --- | --- |

| Title: | Crosscheck for JCTVC-Q0124, Improvements on 1D dictionary coding mode |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Robert Cohen 201 Broadway, 8th Floor Cambridge, MA 02139 USA | Tel: Email: | +1-617-621-7503 cohen@merl.com |
| Source: | Mitsubishi Electric Research Laboratories |  |  |

_____________________________

# Abstract

This document provides the results of a crosscheck of JCTVC-Q0124, specifically the “method 1 + method 2” results as compared to the HM-13.0+RExt-6.0 anchor. The coding performance results for both lossy and lossless coding conditions match the corresponding results in JCTVC-Q0124. The software was examined, and some issues were found, including that “-Werror” had to be disabled to get the software to compile on Linux.

# Cross-check overview

The JCTVC-Q0124 proposal integrated software based on the Pseudo-2D Matching method of JCTVC-L0303, also known as a dictionary coder or as string matching, into the HM-13.0+RExt-6.0 software. The proposal then includes additional improvements to the binarization and coding of the string-matching length. The cross-check performed in this document is for the CU-level 1D dictionary using the new length coding method; currently described in Section 3.2 in JCTVC-Q0124.

# Examination of software

The cross-check was performed on a Linux-based system. The software would not compile unless the “-Werror” option to the gcc compiler was disabled. There also appears to be some operations performed on mixed data types in TComDictCoder.cpp, such as assigning values directly between 64-bit and 32-bit data types, and performing operations across similar data types such as size_t and unsigned int. If this software is going to be used for further study, a rewriting of the software may be needed. Note that these comments are addressing the software associated with the primary implementation of the 1D dictionary, not with the CU-level change or new length coding method.

# Coding performance

## Lossy coding conditions

The performance of the CU-Level dictionary with new length encoding method, with per-picture PSNRs clipped, is as follows:

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| RGB, text & graphics with motion, 1080p | -13.1% | -14.1% | -13.5% |
| RGB, text & graphics with motion,720p | -6.6% | -6.5% | -6.7% |
| RGB, mixed content, 1440p | -0.9% | -0.7% | -0.7% |
| RGB, mixed content, 1080p | -0.7% | -0.6% | -0.6% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -10.1% | -11.3% | -11.2% |
| YUV, text & graphics with motion,720p | -3.3% | -6.0% | -7.3% |
| YUV, mixed content, 1440p | -0.2% | -0.5% | -0.3% |
| YUV, mixed content, 1080p | -0.4% | -0.3% | -0.2% |
| YUV, Animation, 720p | 0.1% | 0.0% | 0.0% |
| Enc Time[%] | 106% |  |  |
| Dec Time[%] | 116% |  |  |
|  | Random Access |  |  |
|  | Y | U | V |
| RGB, text & graphics with motion, 1080p | -7.6% | -8.4% | -8.2% |
| RGB, text & graphics with motion,720p | -8.0% | -7.5% | -7.8% |
| RGB, mixed content, 1440p | -2.0% | -1.6% | -1.7% |
| RGB, mixed content, 1080p | -0.7% | -0.7% | -0.6% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -6.8% | -10.0% | -9.7% |
| YUV, text & graphics with motion,720p | -4.1% | -7.8% | -9.3% |
| YUV, mixed content, 1440p | -0.7% | -1.5% | -1.1% |
| YUV, mixed content, 1080p | -0.6% | -0.8% | -0.6% |
| YUV, Animation, 720p | 0.2% | 0.0% | -0.1% |
| Enc Time[%] | 106% |  |  |
| Dec Time[%] | 156% |  |  |
|  | Low delay B |  |  |
|  | Y | U | V |
| RGB, text & graphics with motion, 1080p | -4.5% | -5.3% | -4.9% |
| RGB, text & graphics with motion,720p | -4.7% | -4.3% | -4.5% |
| RGB, mixed content, 1440p | -1.8% | -1.6% | -1.5% |
| RGB, mixed content, 1080p | -0.3% | -0.3% | -0.2% |
| RGB, Animation, 720p | 0.1% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -3.3% | -6.1% | -5.8% |
| YUV, text & graphics with motion,720p | -1.8% | -4.6% | -6.0% |
| YUV, mixed content, 1440p | -0.6% | -1.5% | -1.0% |
| YUV, mixed content, 1080p | -0.2% | -0.3% | -0.2% |
| YUV, Animation, 720p | 0.2% | 0.0% | 0.0% |
| Enc Time[%] | 105% |  |  |
| Dec Time[%] | 170% |  |  |

These lossy results match the corresponding results in JCTVC-Q0124.

## Lossless coding conditions

The performance of the CU-Level dictionary with new length encoding method for lossless coding conditions is as follows:

|  | AI |  |  |  |
| --- | --- | --- | --- | --- |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 24.2% | 25.9% | 17.6% | 37.5% |
| RGB, text & graphics with motion,720p | 13.9% | 13.7% | 0.6% | 23.0% |
| RGB, mixed content, 1440p | 3.3% | 3.7% | 2.1% | 5.4% |
| RGB, mixed content, 1080p | 8.7% | 7.5% | 4.4% | 10.7% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 27.7% | 29.6% | 17.6% | 44.2% |
| YUV, text & graphics with motion,720p | 13.5% | 14.9% | 0.5% | 31.5% |
| YUV, mixed content, 1440p | 3.5% | 3.9% | 1.4% | 6.5% |
| YUV, mixed content, 1080p | 6.4% | 5.8% | 4.3% | 7.2% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] |  |  |  |  |
| Dec Time[%] |  |  |  |  |
|  | RA |  |  |  |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 13.2% | 17.2% | 12.4% | 26.4% |
| RGB, text & graphics with motion,720p | 2.5% | 8.0% | 0.4% | 22.4% |
| RGB, mixed content, 1440p | 0.5% | 0.5% | 0.5% | 0.5% |
| RGB, mixed content, 1080p | 5.1% | 3.1% | 0.8% | 5.4% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 13.1% | 20.6% | 12.2% | 32.5% |
| YUV, text & graphics with motion,720p | 2.4% | 9.7% | 0.5% | 30.6% |
| YUV, mixed content, 1440p | 0.5% | 0.5% | 0.4% | 0.6% |
| YUV, mixed content, 1080p | 2.9% | 1.9% | 0.8% | 3.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] |  |  |  |  |
| Dec Time[%] |  |  |  |  |
|  | LB |  |  |  |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 12.7% | 13.9% | 10.9% | 18.1% |
| RGB, text & graphics with motion,720p | 1.7% | 6.8% | 0.2% | 20.5% |
| RGB, mixed content, 1440p | 0.2% | 0.2% | 0.1% | 0.3% |
| RGB, mixed content, 1080p | 4.8% | 2.6% | 0.2% | 5.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 12.2% | 16.7% | 11.7% | 23.1% |
| YUV, text & graphics with motion,720p | 1.5% | 8.4% | 0.2% | 28.1% |
| YUV, mixed content, 1440p | 0.2% | 0.2% | 0.2% | 0.3% |
| YUV, mixed content, 1080p | 2.6% | 1.5% | 0.3% | 2.7% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] |  |  |  |  |
| Dec Time[%] |  |  |  |  |

These lossless results match the corresponding results in JCTVC-Q0124.