| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: San Jose, US, 9th – 17th, Jan. 2014 | Document: JCTVC-Q0127 |
| --- | --- |

| Title: | Simplification on block vector prediction for intra block copy |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xiaozhong Xu , Shan Liu, Shawmin Lei MediaTek USA 2860 Junction Ave San Jose, CA 95134, USA | Tel: Email: | Shawmin Lei +886-3-5670766 ext. 25555 { xiaozhong.xu, liu.shan, shawmin.lei} @mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

In this contribution, the block vector prediction for PU based intra block copy (IntraBC) is proposed to change from using the block vector (BV) of last coded IntraBC PU to using the BV of last coded IntraBC CU. The proposed change can remove the dependency of BV derivation between PUs when one CU consists of multiple PUs-. The simulation results show coding performance changes relative to RExt-6.0 anchor are in the range of -0.1% (gain) to 0.2% (loss) for mandatory sequences, and -0.6% (gain) to 0.2% (loss) for optional sequences.

# Introduction

In the current HEVC range extensions draft specification [1], PU level intra block copying (IntraBC) was adopted. In the block vector (BV) prediction, the concept of using BV of last IntraBC coded CU is extended to be using BV of last IntraBC coded PU. When PU level IntraBC is used, more than one PU is involved in the current CU. To decode the BV for the current PU, the BV of last PU should be available therefore decoding dependency exists in thisprocess.

# Proposed method

In this contribution, it is proposed to maintain the original concept of using BV of last IntraBC coded CU as predictor for all PUs in the current IntraBC coded CU. Therefore, all the BVs for the current CU will have a common BV predictor, which comes from the BV of last IntraBC coded CU in the same CTU. Take NxN PU partition as an example, for the current WD:

    - BV[0] = BVD[0] + BV_last;

    - BV[1] = BVD[1] + BV[0];

    - BV[2] = BVD[2] + BV[1];

    - BV[3] = BVD[3] + BV[2];

For the proposed method, the above will be changed into:

    - BV[0] = BVD[0] + BV_last;

    - BV[1] = BVD[1] + BV_last;

    - BV[2] = BVD[2] + BV_last;

    - BV[3] = BVD[3] + BV_last;

where the BV[i] is the block vector for the partition i, BVD[i] is the block vector difference for the partition i, BV_last indicates the block vector of last IntraBC coded CU. i=0, 1, 2, 3.

# Changes in specification text

The following sentence is added into the draft text with highlighted yellow color.

### 8.4.4 Derivation process for block vector components in intra block copying prediction mode

Inputs to this process are:

- a luma location ( xCb, yCb ) of the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture,

- a variable log2CbSize specifying the size of the current luma coding block.

Output of this process is the (nCbS)x(nCbX) array of block vectors bvIntra.

……..

- ….

- ….

- …

- …

- …

- The value of BvpIntra[ compIdx ] is updated to be equal to bvIntra[ xPb ][ yPb ][ compIdx ], when blkIdx equals to numPartitions − 1.

........

# Experimental results

The proposed method is integrated on HM-12.1+RExt-6.0 [2].	The simulations are performed under the updated CTC [3]. Table 1 and Table 2 show the results for losses and lossy respectively.

Table 1. Lossless results

Table 2. Lossy results

# Conclusion

In this proposal, intra block copy BV prediction is changed from using BV of last IntraBC coded PU to using BV of last IntraBC coded CU. By doing this, and the parsing of BVs inside the same CU is simplified without having dependency on previously coded BV. Simulation results show that the proposed change has negligible impact on coding performance.

# Patent rights declaration (s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- D. Flynn, M. Naccari, K. Sharman, C. Rosewarne, J. Sole, G. J. Sullivan, T. Suzuki , “HEVC Range Extensions Draft 6,” Document of Joint Collaborative Team on Video Coding, JCTVC-P1005, Jan. 2014.

- HM-13.0+RExt-6.0, https://hevc.hhi.fraunhofer.de/svn/svn_HEVCSoftware/tags/HM-13.0+RExt-6.0

- Common test conditions and software reference configurations for HEVC range extensions, “C. Rosewarne, K. Sharman, D. Flynn”, JCTVC-P1006, Jan. 2014.