| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Valencia, ES, 27 March – 4 April 2014 | Document: JCTVC-Q0128_r1 |
| --- | --- |

| Title: | Fix for Strong Intra Smoothing in RExt |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jun Xu, Ali Tabatabai 1730 N. First Street San Jose, CA 95112, USA Ohji Nakagami, Teruhiko Suzuki Sony City Osaki. 2-10-1 Osaki Shinagawa-ku , Tokyo, 141-8610 JAPAN | Tel: Email: | +1-408-352-4086 jun.xu@am.sony.com ali.tabatabai@am.sony.com +81-50-3750-2746 ohji.nakagami@jp.sony.com teruhikos@jp.sony.com |
| Source: | Sony Electronics Inc. Sony Corporation |  |  |

_____________________________

# Abstract

This contribution identifies the mismatch between RExt text draft and RExt software on Strong Intra Smoothing. Solution is proposed to resolve this issue. Simulation results demonstrate that proposed solution can maintain coding efficiency of current reference software.

In the revision1, the proposed text is updated since the initial text was the change for the general intra filtering process control, which was not intended solution. Also, the result xls sheets are slightly updated due to the incorrect level syntax value setting on encoding. Thanks to the cross-checker (JCTVC-Q0209) for reporting the issue.

# Problem Statement

In RExt text draft JCTVC-P1005_v1.doc[1], Strong Intra Smoothing (SIS) is allowed to use for chroma components when chroma format is 4:4:4 in Section 8.4.5.2.1 as quoted below.

Depending on the value of predModeIntra, the following ordered steps apply:

- When intra_smoothing_disabled_flag is equal to 0 and either cIdx is equal to 0 or ChromaArrayType is equal to 3, the filtering process of neighbouring samples specified in subclause  is invoked with the sample array p and the transform block size nTbS as inputs, and the output is reassigned to the sample array p.

In the filtering process of neighboring samples specified in subclause 8.4.4.2.38.4.5.2.3, SIS is decided to be applied (biIntFlag is set to 1) if all of the following conditions are true.

- filterFlag is equal to 1 (predModeIntra is not equal to INTRA_DC and nTbS is not equal 4)

- strong_intra_smoothing_enabled_flag is equal to 1

- nTbS is equal to 32

- Abs( p[ −1 ][ −1 ] + p[ nTbS * 2 − 1 ][ −1 ] – 2 * p[ nTbS − 1 ][ −1 ] ) < ( 1  <<  ( BitDepthY − 5 ) )

- Abs( p[ −1 ][ −1 ] + p[ −1 ][ nTbS * 2 − 1 ] – 2 * p[ −1 ][ nTbS − 1 ] ) < ( 1  <<  ( BitDepthY − 5 ) )

Here, it is pointed that cIdx is not checked in the conditions. Thus, if ChromaArrayType is equal to 3, biIntFlag can be set to 1 even if cIdx is equal to 1 or 2.

However, RExt reference software HM13.0+RExt6.0 only allows SIS for luma in the function initAdiPatternChType() of TComPattern.cpp as quoted below.

Bool useStrongIntraSmoothing = isLuma(chType) && pcCU->getSlice()->getSPS()->getUseStrongIntraSmoothing();

Those highlighted parts in text draft and reference software conflict with each other.

# Solution

To resolve the mismatch issue on SIS, it is proposed to change the text draft to align with RExt software. The simulation results have demonstrate that prohibiting SIS for chroma has better coding performance and less complexity than allowing SIS for chroma.

## Text changes

Proposed text changes are applied to Section 8.4.5.2.1 of JCTVC-P1005_v1.doc. The detailed changes are shown in red and highlighted as follows.

Note: Since the initial proposed text affects not only SIS but also other intra filtering processes for chroma component, the proposed text is updated.

8.4.5.2.3 Filtering process of neighbouring samples

…(snip)…

When filterFlag is equal to 1, the following applies:

–	The variable biIntFlag is derived as follows:

–	If all of the following conditions are true, biIntFlag is set equal to 1:

- strong_intra_smoothing_enabled_flag is equal to 1

- nTbS is equal to 32

- cIdx is equal to 0

- Abs( p[ −1 ][ −1 ] + p[ nTbS * 2 − 1 ][ −1 ] – 2 * p[ nTbS − 1 ][ −1 ] ) < ( 1  <<  ( BitDepthY − 5 ) )

- Abs( p[ −1 ][ −1 ] + p[ −1 ][ nTbS * 2 − 1 ] – 2 * p[ −1 ][ nTbS − 1 ] ) < ( 1  <<  ( BitDepthY − 5 ) )

–	Otherwise, biIntFlag is set equal to 0.

# Simulation results

In this section, comparisons between SIS ON and OFF for chroma are presented based on both AHG5 and AHG8 test sequences. The reference software used is HM13.0+RExt6.0.

More specifically,

- Tested: SIS ON means that SIS is allowed for chroma when chroma format is 4:4:4 and SIS is applied because intra_smoothing_disabled_flag is set to 0 (false) in default settings.

- Anchor: SIS OFF is exactly the HM13.0+RExt6.0.

## AHG5 test sequences

In this simulation, AHG5 test sequences and conditions apply. Because the simulation is run in a cluster, encoding and decoding time is not accurate. The results are summarized in Table 1. Results show that allowing SIS for chroma when chroma format is 4:4:4 will degrade the coding performance of current reference software under common test conditions.

Table 1 Summary of results for AHG5 conditions

## AHG8 test sequences

In this simulation, AHG8 test sequences and conditions apply. Because the simulation is run in a cluster, encoding and decoding time is not accurate. The results are summarized in Table 2. Results show that allowing SIS for chroma when chroma format is 4:4:4 will degrade the coding performance of current reference software under common test conditions.

Table 2 Summary of results for AHG8 conditions

# Conclusion

This contribution proposed a solution to fix the mismatch between text draft and reference software on SIS for RExt. Proposed solution maintained the coding efficiency of current reference software. It is recommended to adopt proposed changes to the next version of text draft of RExt.

# Reference

- D. Flynn, M. Naccari, K. Sharman, C. Rosewarne, J. Sole, G. J. Sullivan, T. Suzuki,” HEVC Range Extensions Draft 6”, JCTVC-P1005, San Jose, US, Jan 2014.

- C. Pang, “Cross-check of JCTVC-Q0128 on Fix for Strong Intra Smoothing in RExt”, JCTVC-Q0209, Valencia, ES, Mar 2014.

# Patent rights declaration(s)

Sony Electronics Inc. and Sony Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).