| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Valencia, ES, 27 March – 4 April 2014 | Document: JCTVC-Q0129 |
| --- | --- |

| Title: | Non-SCE1: Reduction of the number of color space regions in SCE1 test 1.1 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Cheung Auyeung 1730 N. First Street San Jose, CA 95112, USA | Tel: Email: | +1-408-352-4086 Cheung.Auyeung@am.sony.com |
|  | Kazushi Sato Sony City Osaki 9F 2-10-1 Osaki, Shinagawa, 141-8610 Tokyo, Japan |  | +81-50-3750-2746 Kazushi.Sato@jp.sony.com |
| Source: | Sony Electronics Inc. and Sony Corporation |  |  |

_____________________________

# Abstract

In SCE1 test 1.1, Nokia proposed to divide the YUV color space uniformly into NYxNCbxNCr=8x8x8 regions, and for each region, a YUV matrix based mapping is used to derive the mapped pixel values. Nokia reported that the luma BDR of AI_1x, AI_2x, RA_1x, and RA_2x were -8.1%, -8.5%, -9.7%, -6.4% respectively. This contribution proposes to reduce the number of regions by 75% by dividing the regions non-uniformly into 8x4x4 regions to reduce the memory requirements and the computation for regression analyses. It resulted in corresponding BDR of -8.6%, -8.8%, -9.9%, -6.5%. In addition, if the color space is divided into the proposed 4x4x4 non-uniform regions, the BDR became -9.1%, -8.8%, -9.5%, -5.8% correspondingly.

# Introduction

SCE 1 [1] evaluates the performance of several prediction tools of color differences between the base layer (BL) and the enhancement layer (EL) of a scalable video encoder as shown in Figure 1.

Figure 1: Color Space Scalable Encoder.

In SCE1 [1] test 1.1, Nokia proposed to divide the YUV color space of the resampled BL reference picture uniformly into NYxNCbxNCr=8x8x8 regions. And for each region, a YUV matrix based mapping is used to derive the mapped pixel values  from the bit-shifted and resampled BL reference picture values by

.

To encode and store the prediction parameters , a lookup tables with 8x8x8x12 coefficients are needed to store the prediction parameters, and 8x8x8 regression analyses are also needed to compute the coefficients.

# Proposal

To reduce the size of the lookup table and to reduce the number of regression analyses, this contribution propose to reduce the number of regions from 8x8x8 to 8x4x4 by dividing the YUV color space non-uniformly as shown in Figure 2. Consequently, the size of the lookup table and the number of regression analyses are reduced by 75%.

This contribution also proposes to reduction the number of regions from 8x8x8 uniform partitions into 4x4x4 non-uniform partitions as shown in Figure 3. The size of the lookup table and the number of regression analyses are reduced by 87.5%.

Figure 2: Reduction of color space regions from 8x8x8 uniform regions to 8x4x4 non-uniform regions.

Figure 3: Reduction of color space regions from 8x8x8 uniform regions to 4x4x4 non-uniform regions.

# Summary

Summary information about the complexity characteristics and coding efficiency of each configuration can be found below:

|  | SCE1 Test 1.1 8x8x8 uniform regions | SCE1 Test 1.1 but with 8x4x4 uniform regions | Proposed 8x4x4 non-uniform regions | Proposed 4x4x4 non-uniform regions |
| --- | --- | --- | --- | --- |
| Number of multipliers (worst case) 2 to 1 scalability ratio | 76.125 | 76.125 | 76.125 | 76.125 |
| Size of table | 8x8x8 | 8x4x4 | 8x4x4 | 4x4x4 |
| Cross-colour dependency in region selection | Yes | Yes | Yes | Yes |
| Cross-colour dependency in mapping process | Yes | Yes | Yes | Yes |
| Other | Multiplier of each pixel depends on the value of the pixel | Multiplier of each pixel depends on the value of the pixel | Multiplier of each pixel depends on the value of the pixel | Multiplier of each pixel depends on the value of the pixel |
| BD-rateY (AI_1x) | -8.1% | -8.5% | -8.6% | -9.1% |
| BD-rateY (AI_2x) | -8.5% | -8.5% | -8.8% | -8.8% |
| BD-rateY (RA_1x) | -9.7% | -9.1% | -9.9% | -9.5% |
| BD-rateY (RA_2x) | -6.4% | -6.0% | -6.5% | -5.8% |

Complexity analyses for all configurations are the same as the SCE1 test 1.1.

# Experimental Results

Since timings are not accurate, they are not reported here.

## SCE1 test 1.1 with 8x4x4 uniform partitions

|  | AI HEVC 1x 10-bit base |  |  | AI HEVC 2x 8-bit base |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A+ | -8.5% | -14.3% | -17.3% | -8.5% | -11.6% | -15.4% |
| Overall (Test vs Ref) | -8.5% | -14.3% | -17.3% | -8.5% | -11.6% | -15.4% |
| Overall (Test vs single layer) | 20.9% | 12.9% | -7.1% | 10.8% | 10.0% | 1.4% |
| Overall (Ref vs single layer) | 32.3% | 31.8% | 11.7% | 21.2% | 24.7% | 19.4% |
| EL only (Test vs Ref) | -32.8% | -36.3% | -39.4% | -16.6% | -19.3% | -23.1% |
| Overall (Test EL+BL vs single EL+BL) | -35.8% | -40.6% | -51.5% | -27.5% | -27.6% | -34.6% |
| BL Match | Matched |  |  | Matched |  |  |
|  | RA HEVC 1x 10-bit base |  |  | RA HEVC 2x 8-bit base |  |  |
|  | Y | U | V | Y | U | V |
| Class A+ | -9.1% | -14.2% | -17.9% | -6.0% | -9.2% | -12.9% |
| Overall (Test vs Ref) | -9.1% | -14.2% | -17.9% | -6.0% | -9.2% | -12.9% |
| Overall (Test vs single layer) | 31.6% | 27.3% | 4.1% | 21.5% | 21.2% | 11.0% |
| Overall (Ref vs single layer) | 45.0% | 48.8% | 26.2% | 29.3% | 33.3% | 27.6% |
| EL only (Test vs Ref) | -27.3% | -31.0% | -34.0% | -11.3% | -14.1% | -17.7% |
| Overall (Test EL+BL vs single EL+BL) | -28.6% | -30.8% | -43.2% | -19.1% | -17.2% | -25.7% |
| BL Match | Matched |  |  | Matched |  |  |

## Proposed 8x4x4 non-uniform partitions

|  | AI HEVC 1x 10-bit base |  |  | AI HEVC 2x 8-bit base |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A+ | -8.6% | -15.7% | -18.5% | -8.8% | -12.6% | -16.7% |
| Overall (Test vs Ref) | -8.6% | -15.7% | -18.5% | -8.8% | -12.6% | -16.7% |
| Overall (Test vs single layer) | 20.6% | 11.1% | -8.5% | 10.3% | 8.8% | -0.2% |
| Overall (Ref vs single layer) | 32.3% | 31.8% | 11.7% | 21.2% | 24.7% | 19.4% |
| EL only (Test vs Ref) | -32.5% | -37.1% | -40.1% | -17.3% | -20.5% | -24.7% |
| Overall (Test EL+BL vs single EL+BL) | -35.7% | -41.2% | -51.9% | -27.7% | -28.3% | -35.6% |
| BL Match | Matched |  |  | Matched |  |  |
|  | RA HEVC 1x 10-bit base |  |  | RA HEVC 2x 8-bit base |  |  |
|  | Y | U | V | Y | U | V |
| Class A+ | -9.9% | -15.9% | -19.3% | -6.5% | -10.2% | -14.1% |
| Overall (Test vs Ref) | -9.9% | -15.9% | -19.3% | -6.5% | -10.2% | -14.1% |
| Overall (Test vs single layer) | 30.4% | 24.6% | 2.3% | 20.9% | 19.8% | 9.6% |
| Overall (Ref vs single layer) | 45.0% | 48.8% | 26.2% | 29.3% | 33.3% | 27.6% |
| EL only (Test vs Ref) | -29.5% | -33.8% | -36.4% | -12.0% | -15.4% | -19.2% |
| Overall (Test EL+BL vs single EL+BL) | -29.0% | -31.8% | -43.8% | -19.4% | -18.0% | -26.5% |
| BL Match | Matched |  |  | Matched |  |  |

## Proposed 4x4x4 non-uniform partitions

|  | AI HEVC 1x 10-bit base |  |  | AI HEVC 2x 8-bit base |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A+ | -9.1% | -15.5% | -18.1% | -8.8% | -12.1% | -15.9% |
| Overall (Test vs Ref) | -9.1% | -15.5% | -18.1% | -8.8% | -12.1% | -15.9% |
| Overall (Test vs single layer) | 20.1% | 11.3% | -8.0% | 10.4% | 9.4% | 0.8% |
| Overall (Ref vs single layer) | 32.3% | 31.8% | 11.7% | 21.2% | 24.7% | 19.4% |
| EL only (Test vs Ref) | -36.1% | -39.8% | -42.3% | -17.4% | -20.2% | -24.0% |
| Overall (Test EL+BL vs single EL+BL) | -36.2% | -41.4% | -52.1% | -27.8% | -28.2% | -35.3% |
| BL Match | Matched |  |  | Matched |  |  |
|  | RA HEVC 1x 10-bit base |  |  | RA HEVC 2x 8-bit base |  |  |
|  | Y | U | V | Y | U | V |
| Class A+ | -9.5% | -15.2% | -18.3% | -5.8% | -9.1% | -12.6% |
| Overall (Test vs Ref) | -9.5% | -15.2% | -18.3% | -5.8% | -9.1% | -12.6% |
| Overall (Test vs single layer) | 30.9% | 25.7% | 3.5% | 21.9% | 21.3% | 11.4% |
| Overall (Ref vs single layer) | 45.0% | 48.8% | 26.2% | 29.3% | 33.3% | 27.6% |
| EL only (Test vs Ref) | -28.9% | -33.0% | -35.4% | -11.0% | -14.0% | -17.4% |
| Overall (Test EL+BL vs single EL+BL) | -28.8% | -31.5% | -43.6% | -18.8% | -17.3% | -25.6% |
| BL Match | Matched |  |  | Matched |  |  |

# References

- A. Duenas, P. Andrivon, Y. Ye, E. Alshina, X. Li, K. Ugur, K. Sato, S.H. Kim, “Description of HEVC Scalable Extensions Core Experiment SCE1: Color Gamut and Bit-Depth Scalability,” JCTVC-P1101, San José, US, 9–17 Jan. 2014.

# Patent rights declaration(s)

Sony Electronics Inc. and Sony Group may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).