| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Valencia, ES, 27 March – 4 April 2014 | Document: JCTVC-Q0130 |
| --- | --- |

| Title: | AHG12: Mismatch of SHVC draft 5 and SHM-5.0 software in bit-depth scalability with 1x scalability |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Cheung Auyeung 1730 N. First Street San Jose, CA 95112, USA | Tel: Email: | +1-408-352-4086 Cheung.Auyeung@am.sony.com |
| Source: | Sony Electronics Inc. |  |  |

_____________________________

# Abstract

This contribution reports that when bit-depth scalability is enabled with 1x scalability, SHM-5.0 software does not match the description of SHVC working draft 5. This contribution proposes two alternate fixes to the SHM-5.0 software for bit-depth and 1x scalability. Both bug fixes resulted in the same luma BDR of AI_1x and RA_1x of -28% and -17.9%, respectively, with the SCE1 BT.709 8 bits and 10 bits test sequences as inputs.

# Introduction

This contributions reports that when bit-depth scalability is enabled with 1x spatial scalability, SHM-5.0 software does not match the specification of the resampling process in SHVC working draft 5.

Figure 1: Mismatch in SHVC draft 5 and SHM-5.0 software.

As shown in Figure 1, SHVC draft 5 section H.8.1.4 specified that when the base layer and enhancement layer have the same picture sizes, same internal bit-depths, and zero scaled reference layer offsets, the inter-layer reference picture ilRefPic is set equal to the decoded reference layer picture rlPic.

However, the SHM-5.0 does not check if the BL and EL have the same bit-depths. In SHM-5.0, when the base layer and enhancement layer have the same picture sizes and zero scaled reference layer offset, the inter-layer reference picture ilRefPic is set equal to the decoded reference layer picture rlPic without checking the bit-depth difference of BL and EL. Consequently, SHM-5.0 does not support bit-depth scalability with 1x scalability in the inter-layer prediction process.

# Proposed bug fix to SHM software

This contribution proposes two alternate implementations to fix the bit-depth and 1x scalability bug in SHM-5.0 depicted in Figure 2. To fix the bug, this contribution proposes to modify the joint resampling and bit-shift process and the corresponding bypass decision in SHM-5.0.

Figure 2: Bit-depth scalability with 1x scalability is not supported in SHM-5.0.

## Bug fix implementation 1

As shown in Figure 3, the first bug fix implementation adds bit-shift for the same picture size with BL and EL to the joint resampling and bit-shift process. It also adds the bit-depth check back to the bypass decision to align with the SHVC draft 5.

Figure 3: Bug fix to SHM-5.0 aligned with SHEVC draft text.

## Bug fix implementation 2

As shown in Figure 4, the second bug fix implementation adds bit-shift for the same picture size with BL and EL to the joint resampling and bit-shift process. In contrast to the first implementation, it removes the bypass decision, since the bypass decision in the working draft is not necessary to specify the behaviour of the decoder.

Figure 4: Alternate bug fix to SHM-5.0.

# Experimental Results

The proposed bug fix was implemented in the SCE1 anchor and tested with the QPs in Table 1from SHM SNR scalability common test conditions [2] using the SCE1 [3] test sequences in Table 2. The BDR results for the two fixes are the same and they are in Table 3.

Table 1: SHM common test conditions QPs for 1x scalability.

|  | Base layer | Enhancement layer |
| --- | --- | --- |
| Test cases | Base layer | Enhancement layer |
| 1x: RA and AI | 1080p 8-bit BT.709 QP_BL={26, 30 ,34, 38} | 1080p 10-bit BT.709 QP_EL1={20, 24, 28, 32} QP_EL2={22, 26, 30, 34} |

Table 2: SCE1 BT709 test sequences.

| Test Sequences | size | fps | Duration | Bit-depth |
| --- | --- | --- | --- | --- |
| BT709_Birthday_1920x1080_60_zerophase_0.9pi.yuv | 1920 x 1080 | 60 | 300 | 8 |
| BT709_BirthdayFlashPart1_1920x1080_60_zerophase_0.9pi.yuv | 1920 x 1080 | 60 | 300 | 8 |
| BT709_BirthdayFlashPart2_1920x1080_60_zerophase_0.9pi.yuv | 1920 x 1080 | 60 | 300 | 8 |
| BT709_Parakeets_1920x1080_50_zerophase_0.9pi.yuv | 1920 x 1080 | 50 | 250 | 8 |
| BT709_TableCar_1920x1080_60_zerophase_0.9pi.yuv | 1920 x 1080 | 60 | 300 | 8 |
| BT709_Birthday_1920x1080_60_10bit_zerophase_0.9pi.yuv | 1920 x 1080 | 60 | 300 | 10 |
| BT709_BirthdayFlashPart1_1920x1080_60_10bit_zerophase_0.9pi.yuv | 1920 x 1080 | 60 | 300 | 10 |
| BT709_BirthdayFlashPart2_1920x1080_60_10bit_zerophase_0.9pi.yuv | 1920 x 1080 | 60 | 300 | 10 |
| BT709_Parakeets_1920x1080_50_10bit_zerophase_0.9pi.yuv | 1920 x 1080 | 50 | 250 | 10 |
| BT709_TableCar_1920x1080_60_10bit_zerophase_0.9pi.yuv | 1920 x 1080 | 60 | 300 | 10 |

Table 3: Anchor is SCE1 anchor, and test is SCE1 anchor with bug fix.

|  | AI HEVC 1x 8-bit base |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A+ | -28.0% | -27.4% | -27.9% |
| Overall (Test vs Ref) | -28.0% | -27.4% | -27.9% |
| Overall (Test vs single layer) | 13.5% | 13.9% | 13.2% |
| Overall (Ref vs single layer) | 57.8% | 57.3% | 57.4% |
| EL only (Test vs Ref) | -50.6% | -50.5% | -50.9% |
| Overall (Test EL+BL vs single EL+BL) | -27.9% | -28.7% | -28.6% |
| BL Match | Matched |  |  |
|  | RA HEVC 1x 8-bit base |  |  |
|  | Y | U | V |
| Class A+ | -17.9% | -15.1% | -16.2% |
| Overall (Test vs Ref) | -17.9% | -15.1% | -16.2% |
| Overall (Test vs single layer) | 24.2% | 27.3% | 25.6% |
| Overall (Ref vs single layer) | 51.5% | 50.1% | 50.0% |
| EL only (Test vs Ref) | -30.6% | -28.0% | -28.9% |
| Overall (Test EL+BL vs single EL+BL) | -18.9% | -17.3% | -17.7% |
| BL Match | Matched |  |  |

# References

- J. Chen, J. Boyce, Y. Ye, M.M. Hannuksela, G. Sullivan, Y.K. Wang, “High efficiency video coding (HEVC) scalable extensions Draft 5,” JCTVC-P1008_v4, San José, US, 9–17 Jan. 2014.

- V. Seregin and Y. He, “Common SHM test conditions and software reference configurations,” JCTC-P1009, San José, US, 9–17 Jan. 2014.

- A. Duenas, P. Andrivon, Y. Ye, E. Alshina, X. Li, K. Ugur, K. Sato, S.H. Kim, “Description of HEVC Scalable Extensions Core Experiment SCE1: Color Gamut and Bit-Depth Scalability,” JCTVC-P1101, San José, US, 9–17 Jan. 2014.

# Patent rights declaration(s)

Sony Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).