| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: San Jose, US, 9th – 17th, Jan. 2014 | Document: JCTVC-Q0132 |
| --- | --- |

| Title: | On unification of intra block copy and inter-picture motion compensation |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xiaozhong Xu , Shan Liu, Shawmin Lei MediaTek USA 2860 Junction Ave San Jose, CA 95134, USA | Tel: Email: | Shawmin Lei +886-3-5670766 ext. 25555 { xiaozhong.xu, liu.shan, shawmin.lei} @mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

In this contribution, the intra block copy operation (IntraBC) for a prediction unit is modified from its current form (in RExt6) to align with a motion compensated PU in inter slice. AMVP is used for PU level BV prediction and inter PU syntax is used for IntraBC signaling. The simulation results using AMVP for IntraBC reported average -0.3%, 0.0% and -0.3% BD-rate change (YCbCr444SC main-tier) compared with RExt6 anchor. The simulation results using inter PU signaling reported average 1.9%, 2.1% and 2.0% BD-rate change (YCbCr444SC main-tier) compared with RExt6 anchor. The IntraBC related syntax is simplified.

# Introduction

In the current HEVC range extensions draft specification [1], intra block copying (IntraBC) was adopted. From coding tool point of view, it is very similar to the inter motion compensation as a block based predictive coding tool. Currently, the major differences between the two are the vector prediction mechanism and the precision of displacement vector. In this contribution, the investigation is conducted with the attempt to use all the tools and syntax representations in inter motion compensation to perform intra block copy.

# Aligned intra block copy with inter motion compensation

In this contribution, the following changes are made:

- The spatial AMVP is used for BV prediction for intra block copy.

- Use prediction_unit() syntax to signal the usage of IntraBC.

  - The partition mode coding of IntraBC is unified with inter partition modes.

  - The vector prediction of IntraBC is unified with spatial AMVP for inter PU vector prediction.

  - The mvd coding of IntraBC is unified with inter mvd coding.

  - IntraBC mode is treated as an inter mode with inter_pred_idc equals to 0 (L0). In this mode, pred_mode_flag = 0 in intra slice; pred_mode_flag = 0 and ref_idx_l0 = num_ref_idx_l0_active_minus1 + 1 in B and P slices.

In addition, we tested by setting BV precision to be quarter-pel accuracy, same as for current inter prediction. This is done by two left shifts at encoder side when encoding BV difference and at decoder side two right shifts after decoding the BV difference.

# Syntax and semantic changes

With the described unifications, the CU syntax is as follows. The texts which are highlighted in green were previously introduced for signaling the use of IntraBC, which can now be removed.

#### 7.3.8.5 Coding unit syntax

| coding_unit( x0, y0, log2CbSize ) { | Descriptor |
| --- | --- |
| if( transquant_bypass_enabled_flag ) |  |
| cu_transquant_bypass_flag | ae(v) |
| if( slice_type != I ) |  |
| cu_skip_flag[ x0 ][ y0 ] | ae(v) |
| nCbS = ( 1 << log2CbSize ) |  |
| if( cu_skip_flag[ x0 ][ y0 ] ) |  |
| prediction_unit( x0, y0, nCbS, nCbS ) |  |
| else { |  |
| if( intra_block_copy_enabled_flag ) |  |
| intra_bc_flag[ x0 ][ y0 ] | ae(v) |
| if( slice_type != I && !intra_bc_flag[ x0 ][ y0 ] ¦¦ intra_block_copy_enabled_flag ) |  |
| pred_mode_flag | ae(v) |
| if( CuPredMode[ x0 ][ y0 ] != MODE_INTRA ¦ ¦ intra_bc_flag[ x0 ][ y0 ] ¦ ¦ log2CbSize = = MinCbLog2SizeY ) |  |
| part_mode | ae(v) |
| if( CuPredMode[ x0 ][ y0 ] = = MODE_INTRA ) { |  |
| if( PartMode = = PART_2Nx2N && pcm_enabled_flag && !intra_bc_flag[ x0 ][ y0 ] && log2CbSize >= Log2MinIpcmCbSizeY && log2CbSize <= Log2MaxIpcmCbSizeY ) |  |
| pcm_flag[ x0 ][ y0 ] | ae(v) |
| if( pcm_flag[ x0 ][ y0 ] ) { |  |
| while( !byte_aligned( ) ) |  |
| pcm_alignment_zero_bit | f(1) |
| pcm_sample( x0, y0, log2CbSize ) |  |
| } else if( intra_bc_flag[ x0 ][ y0 ] ) { |  |
| mvd_coding( x0, y0, 2) |  |
| if( PartMode = = PART_2NxN ) |  |
| mvd_coding( x0, y0 + ( nCbS / 2 ), 2) |  |
| else if( PartMode = = PART_Nx2N ) |  |
| mvd_coding( x0 + ( nCbS / 2 ), y0, 2) |  |
| else if( PartMode = = PART_NxN ) { |  |
| mvd_coding( x0 + ( nCbS / 2 ), y0, 2) |  |
| mvd_coding( x0, y0 + ( nCbS / 2 ), 2) |  |
| mvd_coding( x0 + ( nCbS / 2 ), y0 + ( nCbS / 2 ), 2) |  |
| } |  |
| } else { |  |
| pbOffset = ( PartMode = = PART_NxN ) ? ( nCbS / 2 ) : nCbS |  |
| for( j = 0; j < nCbS; j = j + pbOffset ) |  |
| for( i = 0; i < nCbS; i = i + pbOffset ) |  |
| prev_intra_luma_pred_flag[ x0 + i ][ y0 + j ] | ae(v) |
| for( j = 0; j < nCbS; j = j + pbOffset ) |  |
| for( i = 0; i < nCbS; i = i + pbOffset ) |  |
| if( prev_intra_luma_pred_flag[ x0 + i ][ y0 + j ] ) |  |
| mpm_idx[ x0 + i ][ y0 + j ] | ae(v) |
| else |  |
| rem_intra_luma_pred_mode[ x0 + i ][ y0 + j ] | ae(v) |
| if( ChromaArrayType = = 3 ) |  |
| for( j = 0; j < nCbS; j = j + pbOffset ) |  |
| for( i = 0; i < nCbS; i = i + pbOffset ) |  |
| intra_chroma_pred_mode[ x0 + i ][ y0 + j ] | ae(v) |
| else if( ChromaArrayType != 0 ) |  |
| intra_chroma_pred_mode[ x0 ][ y0 ] | ae(v) |
| } |  |
| } else { |  |
| if( PartMode = = PART_2Nx2N ) |  |
| prediction_unit( x0, y0, nCbS, nCbS ) |  |
| else if( PartMode = = PART_2NxN ) { |  |
| prediction_unit( x0, y0, nCbS, nCbS / 2 ) |  |
| prediction_unit( x0, y0 + ( nCbS / 2 ), nCbS, nCbS / 2 ) |  |
| } else if( PartMode = = PART_Nx2N ) { |  |
| prediction_unit( x0, y0, nCbS / 2, nCbS ) |  |
| prediction_unit( x0 + ( nCbS / 2 ), y0, nCbS / 2, nCbS ) |  |
| } else if( PartMode = = PART_2NxnU ) { |  |
| prediction_unit( x0, y0, nCbS, nCbS / 4 ) |  |
| prediction_unit( x0, y0 + ( nCbS / 4 ), nCbS, nCbS * 3 / 4 ) |  |
| } else if( PartMode = = PART_2NxnD ) { |  |
| prediction_unit( x0, y0, nCbS, nCbS * 3 / 4 ) |  |
| prediction_unit( x0, y0 + ( nCbS * 3 / 4 ), nCbS, nCbS / 4 ) |  |
| } else if( PartMode = = PART_nLx2N ) { |  |
| prediction_unit( x0, y0, nCbS / 4, nCbS ) |  |
| prediction_unit( x0 + ( nCbS / 4 ), y0, nCbS * 3 / 4, nCbS ) |  |
| } else if( PartMode = = PART_nRx2N ) { |  |
| prediction_unit( x0, y0, nCbS * 3 / 4, nCbS ) |  |
| prediction_unit( x0 + ( nCbS * 3 / 4 ), y0, nCbS / 4, nCbS ) |  |
| } else { /* PART_NxN */ |  |
| prediction_unit( x0, y0, nCbS / 2, nCbS / 2 ) |  |
| prediction_unit( x0 + ( nCbS / 2 ), y0, nCbS / 2, nCbS / 2 ) |  |
| prediction_unit( x0, y0 + ( nCbS / 2 ), nCbS / 2, nCbS / 2 ) |  |
| prediction_unit( x0 + ( nCbS / 2 ), y0 + ( nCbS / 2 ), nCbS / 2, nCbS / 2 ) |  |
| } |  |
| } |  |
| if( !pcm_flag[ x0 ][ y0 ] ) { |  |
| if( CuPredMode[ x0 ][ y0 ] != MODE_INTRA && !( PartMode = = PART_2Nx2N && merge_flag[ x0 ][ y0 ] ) ¦ ¦ ( CuPredMode[ x0 ][ y0 ] = = MODE_INTRA && intra_bc_flag[ x0 ][ y0 ] ) ) |  |
| rqt_root_cbf | ae(v) |
| if( rqt_root_cbf ) { |  |
| MaxTrafoDepth = ( CuPredMode[ x0 ][ y0 ] = = MODE_INTRA ? ( max_transform_hierarchy_depth_intra + IntraSplitFlag ) : max_transform_hierarchy_depth_inter ) |  |
| transform_tree( x0, y0, x0, y0, log2CbSize, 0, 0 ) |  |
| } |  |
| } |  |
| } |  |
| } |  |

#### 7.4.9.5 Coding unit semantics

- If intra_bc_flag[ x0 ][ y0 ] is equal to 1, CuPredMode[ x ][ y ] is inferred to be equal to MODE_INTRA.

- Otherwise, if slice_type is equal to I and intra_block_copy_enabled_flag is equal to 0, CuPredMode[ x ][ y ] is inferred to be equal to MODE_INTRA.

# Experimental results

The proposed method is integrated on HM-12.1+RExt-6.0 [2].	The simulations are performed under the updated CTC [3]. Note that some simulations are still running and more results will be updated upon completion. Note that for now the results in Simulation A and B are full length results; simulations C and D are 1-second duration.

## Simulation A: use AMVP for BV prediction

In this simulation, the  spatial AMVP is used as the BV predictor.

Table 1 and Table 2 show the results for losses and lossy respectively.

Table 1. Simulation A lossless results

Table 2. Simulation A lossy results

## Simulation B: IntraBC using PU syntax with integer MV accuracy

In this simulation, InraBC uses inter PU syntax while the BV accuracy for IntraBC is still integer.

Table 3 and Table 4 show the results for losses and lossy respectively.

Table 3. Simulation B lossless results

Table 4. Simulation B lossy results

## Simulation C: AMVP + quarter-pel MV accuracy

In this simulation, spatial AMVP for vector prediction and fractional-pel accuracy MV precision are adopted.

Table 5 and Table 6 show the results for losses and lossy respectively.

Table 5. Simulation C lossless results

Table 6. Simulation C lossy results

## Simulation D: IntraBC using PU syntax with fractional-pel MV accuracy

In this simulation, InraBC uses inter PU syntax while the mv accuracy for IntraBC is fractional.

Table 7 and Table 8 show the results for losses and lossy respectively.

Table 7. Simulation D lossless results

Table 8. Simulation D lossy results

# Conclusion

In this proposal, intra block copy is modified to be align with inter motion compensation with regard to syntax alignment and coding tools used. Simulation results of such alignments are reported.

# Patent rights declaration (s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- D. Flynn, M. Naccari, K. Sharman, C. Rosewarne, J. Sole, G. J. Sullivan, T. Suzuki , “HEVC Range Extensions Draft 6,” Document of Joint Collaborative Team on Video Coding, JCTVC-P1005, Jan. 2014.

- HM-13.0+RExt-6.0, https://hevc.hhi.fraunhofer.de/svn/svn_HEVCSoftware/tags/HM-13.0+RExt-6.0

- Common test conditions and software reference configurations for HEVC range extensions, “C. Rosewarne, K. Sharman, D. Flynn”, JCTVC-P1006, Jan. 2014.