| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Valencia, ES, 27 March – 4 April 2014 | Document: JCTVC-Q0133 |
| --- | --- |

| Title: | Comments on HEVC 4:4:4 8 bit profiles |  |  |
| --- | --- | --- | --- |
| Status: | Input |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Alexandros Tourapis David Singer 1 Infinite Loop Cupertino, CA 95014 USA | Tel: Email: | atourapis@apple.com singer@apple.com |
| Source: | Apple Inc. |  |  |

_____________________________

# Abstract

This contribution identifies certain possibilities for the creation of 4:4:4 8 bit profiles as an outcome of the Screen Content CfP and expresses an opinion on which avenue is preferred to be followed depending on this outcome.

# Proposal

There has been considerable debate on whether the upcoming Amd1 of the HEVC specification should define explicitly a 4:4:4 8 bit profile.  Such debate has arisen from the current ongoing investigation activity on Screen Content coding, where the goal is to determine whether there exist new coding tools that can provide considerable coding advantage, mainly for screen content, versus the available tools that are already specified in the HEVC specification.

Since a 4:4:4 8 bit profile is mainly seen being used within the context of Screen Content coding, it has been felt by some that, if evidence is shown, there could be a redundancy in profiles if a profile was specified now given the existing coding tools in the HEVC specification and if a second profile, explicitly targeting screen content, was specified later.

On the other hand, others [1] have expressed the concern that without a defined 4:4:4 8 bit profile by the end of the 17th JCT-VC meeting, this may affect several other ongoing activities, outside MPEG, that relate to next generation wireless display products and where the need of a 4:4:4 8 bit profile is imminent.  Therefore, they would recommend that a 4:4:4 8 bit profile is defined now, without having to wait for the definition of an explicit, screen content profile. It is, however, also noted that some tools that have previously been identified as screen content coding tools and have been already included in the specification, should also be included in the proposed 4:4:4 8 bit profile.

Although we also share the opinion in [1] that an explicit 4:4:4 8 bit profile should be specified at the 17th JCT-VC meeting, we are also of the opinion that its definition should be based on the outcome of the Screen Content CfP. This CfP is expected to conclude at this meeting. In particular, and depending on this outcome, we perceive the following three possibilities:

- A single 4:4:4 8 bit profile is defined at the 17th meeting. No further work on any future 4:4:4 8 bit profile is anticipated.

- An initial 4:4:4 8 bit profile is defined at the 17th meeting with, however, the expectation that an “advanced 4:4:4 8 bit” profile using new, i.e. screen content, tools is specified later. However, in this scenario, it would be important that we make sure that these two profiles form a sensible family

- No profile is defined now, with the expectation that an “advanced 4:4:4 8 bit” profile, using new tools, is specified at a later date.

From these options, we feel that option (c) would not be appropriate and, as was also identified in [1], may have considerable repercussions to several ongoing activities and products. Option (a) would be a likely outcome if the screen content CfP comes up empty, whereas (b) should be the preferred option if it does. However, for this option, we should make sure that when defining the two profiles, we do not fall into a non-nested profile situation.

It is therefore our recommendation that the outcome of the Screen Content CfP is studied carefully, and that the committee defines now either:

- the only expected 4:4:4 8-bit profile (outcome (a)) or

- the initial member of a pair of expected profiles which have a sensible, nested, relationship (outcome (b)).

# References

- G. Martin-Cocher, P. Onno, C. Rosewarne, A. Fudseth, R. Sjöberg, A. Duenas, M. Karczewicz, and M. Budagavi, “Request for an HEVC 4:4:4 8 bit profile”, JCTVC-Q0113, 17th Meeting: Valencia, ES, 27 March – 4 April 2014

# Patent rights declaration(s)

Apple Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

This implies that the advanced profile should be a superset of the first profile that would be defined. We would essentially wish to avoid a scenario where any newly identified coding tools in the advanced profile replace a tool that may have already been included in this hypothetical, first profile.