| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Valencia, ES, 27 March – 4 April 2014 | Document: JCTVC-Q0135 |
| --- | --- |

| Title: | AMP for Intra BC prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Lihua Zhu Jizheng Xu YongJun Wu Gary J. Sullivan Sridhar Sankuratri B. Anil Kumar Microsoft Corp. 1 Microsoft Way Redmond, WA 98052 USA | Email: | lizhu@microsoft.com jzxu@microsoft.com yongjunw@microsoft.com garysull@microsoft.com |
| Source: | Microsoft Corp. |  |  |

_____________________________

# Abstract

This document reports the results of applying asymmetric motion partitioning (AMP) for Intra block copy (BC) prediction. The addition of the AMP mode is asserted to improve harmonization of the Intra BC design with what is used for inter-picture prediction, and it is noted that since AMP uses only two partitions instead of four, it may have lower decoding complexity than the NxN case. Two sets of results with different motion estimation algorithms to speed up AMP in the Intra BC mode are provided. The first one uses the existing fast motion estimation for the Intra BC mode. The second one uses a modified hybrid motion estimation for the Intra BC mode.

# Introduction

## Asymmetric motion partition for inter prediction

JCTVC-A124 [1] proposed the Asymmetric motion partitioning (AMP), which uses rectangular shape PU splitting of a block for inter prediction. The motion partitions can be 2NxnU, 2NxnD, nLx2N, and nRx2N. Figure 1 shows a use case for the AMP when the block size is 16x16. It has been well established that AMP can improve the coding efficiency of inter prediction.

Figure 1 Example of asymmetric motion partitions

## Asymmetric motion partition for intraBC prediction with NxN

We applied the AMP for the intra BC prediction, since AMP takes irregular image patterns which will be good for the image and text mixing content such as web pages and ppt.

## Asymmetric motion partition for intraBC prediction without NxN

We also investigated using the same prediction architecture as the Inter without NxN (AMP + 2NxN/Nx2N + 2Nx2N). This could potentially bring benefits for hardware reuse, and it would reduce the number of partitions relative to the NxN case. In our test, this alternative resulted in an average 1.4% loss for YCbCr 4:4:4 SC test sequences.

# Results and analysis

The experimental results of the proposed method are presented for the existing fast motion estimation in the HEVC reference software and with a modified motion estimation, respectively. The method is tested on the top of HM13.0_Rext6.0.

- Snapshot of  the default fast motion estimation result (Rext6.0 + AMP)

- Snapshot of the improved fast motion estimation result (Rext6.0 + AMP)

- Snapshot of the improved fast motion estimation result (Rext6.0 without NxN + AMP )

# References

- K. McCann (Zetacast / Samsung), W.-J. Han, I.-K. Kim, J.-H. Min, E. Alshina, A. Alshin, T. Lee, J. Chen, V. Seregin, S. Lee, Y.-M. Hong, M.-S. Cheon, N. Shlyakhov (Samsung), “Samsung’s Response to the Call for Proposals on Video Compression Technology”, document JCTVC-A124, First JCT-VC Meeting, Dresden, DE, April, 2010.

# Patent rights declaration(s)

Microsoft Corp. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).