| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Valencia, ES, 27 March – 4 April 2014 | Document: JCTVC-Q0136 |
| --- | --- |

| Title: | AHG14: On methods and test conditions for color gamut scalability |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Koohyar Minoo David Baylon Ajay Luthra 6450 Sequence Drive San Diego, CA 92121 USA | Tel: Email: | +1 (858) 404-2521 koohyar.minoo@arrisi.com david.baylon@arrisi.com ajay.luthra@arrisi.com |
| Source: | ARRIS |  |  |

_____________________________

# Abstract

This contribution proposes the following modifications to the test conditions of future core experiments regarding color gamut scalability:

- Include test sequences for which the color grading is performed at the lowest spatial resolution.

- Include a test condition where the base layer is in the same color gamut as enhanced layer.

Regarding the proposed methods in SCE1 this contribution asserts that none of the proposed methods captures the local nature of color transform function for a class of use cases in which tone-mapping and color grading are performed based on local characteristics of the scene. Therefore to qualify proposed scheme for such use cases, it is also proposed to:

- Include test sequences representing global and local tone mapping and color grading scenarios.

# Order of spatial scaling and color-mapping

There are use cases where the tone mapping is performed first and then down-sampling is performed second. The current SCE1 covers test sequences for this use case. However due to the non-linear nature of color grading and spatial scaling it is conceivable that to best preserve the “artistic intent” for the content, it would be preferable to perform tone mapping and color grading after spatial scaling is performed. Currently SCE1 does not cover test sequences for the later use case. Thus it is proposed to:

- Include test sequences for which the color grading is performed at the lowest spatial resolution.  This means:

  - For the case of 1x spatial scalability, the un-decimated sequences should also be tested. e.g. 4K BT.2020 and 4K BT.709

  - For the case of 2x spatial scalability, the color grading should be applied at lower resolution, e.g. the 1080 BT.709 sequence should be derived from the 1080 BT.2020.

Note that depending on the order in which the resizing and color grading is performed to generate the base layer, it might be preferable for the decoder to perform the color-transform and resizing in certain order. Adding the proposed test condition will help to verify different proposals with different order of resizing and color mapping on the decoder side.

# Performance of CGS tools on non-CGS content

A desired property for a CGS tool is to maintain the coding efficiency for scalable coding of a wider variety of sequences in different color gamut spaces, (other than BT.709 and BT2020 included in current test conditions for SCE1 [1]). Also since the color grading is an artistic process, the strength of color adjustment (which can be thought of as the deviation of color transfer function from an Identity transform) can drastically vary.

Although it may not be possible to test sequences which represent all possible color correction choices, at the minimum the proposed approaches should be tested with test conditions where the base layer is generated from the enhancement without any color correction done.

# Local vs. global color grading

For use cases where the tone mapping and color grading are performed based on local content of a scene, it might be preferable to signal different color transform parameters for different spatially local regions of coded pictures. It has been observed that none of the proposed tools in SCE1 accommodates this flexibility. To learn how efficient such global color correction schemes are, it is proposed that test sequences include cases with local color tone mapping and color grading.

# Reference

[1] “Description of HEVC Scalable Extensions Core Experiment SCE1: Color Gamut and Bit-Depth Scalability”, JCTVC-P1101, Jan 2014

# Patent rights declaration(s)

ARRIS Group, Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).