| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Valencia, ES, 27 March – 4 April 2014 | Document: JCTVC-Q0137 M32875 |
| --- | --- |

| Title: | AhG 8: Adaptive Scaling of SAO offset |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Woo-Shik Kim, Joel Sole, Marta Karczewicz 5775 Morehouse Drive San Diego, CA, USA 92121 | Tel: Email: | 1-858-845-8893 wooshikk@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

The scale factor for the offset in sample adaptive offset (SAO) is signaled in the picture parameter set (PPS). In this contribution an adaptive offset scaling scheme is proposed, in which the signaling level is changed to the slice header, and the range of the scaling factor is modified. The reported coding gain is 0.3/0.6% for luma/chroma, respectively in low delay B main-tier.

# Introduction

In the last meeting, the proposal to signal the offset scale factor in sample adaptive offset (SAO) [1] is adopted into the HEVC Range Extension [2]. Each scale factor for luma and chroma channel is signaled in the picture parameter set (PPS), respectively, and the range of the scale factor is set as [0, Max( 0, BitDepth − 10 )].

In HM, SAO usage is checked for each slice and a flag to disable SAO is signaled in the slice header. This provides flexibility to control SAO at the encoder side. In the same way it is proposed to move the signaling of offset scale factor from PPS to the slice header, so that the adaptation of this scale factor can be easily made during encoding. It is also proposed to modify the offset scale factor range so that the proposed scheme can be applied for screen contents coding.

# Proposed Methods

First, the signaling of offset scale factor is moved to the slice header. When SAO is disabled in the slice, the signaling of offset scale factor is skipped. At the encoder side the statistics to calculate the non-quantized offset value is performed once as in the current HM. Then, for each possible offset scale factor, RD cost is calculated to select the optimal offset scale factor.

Secondly, the offset scale factor range is set to [0, BitDepth − 5]. This allows the adaptation can be applied for all bit-depths. This change is necessary especially for screen contents coding of which the bit-depth is typically not greater than 10 bit.

The same changes are applied both for luma and chroma offset scale factors, and a flag is signaled in the sequence parameter set (SPS) to enable or disable this scheme, when SAO is enabled in SPS.

# Experimental Results

To evaluate the coding efficiency of the proposed method, the common test conditions [3] are used, and HEVC_HM13.0_RExt6.0 is used as the anchor. Below are the results.

|  | All Intra HE Main-tier |  |  | All Intra HE High-tier |  |  | AI HE Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| RGB 4:4:4 | -0.1% | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YCbCr 4:4:4 | 0.0% | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YCbCr 4:2:2 | 0.0% | -0.3% | -0.5% | 0.0% | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% |
| Overall | 0.0% | -0.1% | -0.2% | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | -0.2% | -0.2% | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 99% |  |  | 100% |  |  | 98% |  |  |
| Dec Time[%] | 98% |  |  | 99% |  |  | 98% |  |  |
|  | RA HE Main-tier |  |  | RA HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| RGB 4:4:4 | -0.2% | -0.4% | -0.3% | -0.1% | -0.2% | -0.2% |  |  |  |
| YCbCr 4:4:4 | -0.1% | -0.4% | -0.2% | -0.1% | -0.2% | -0.1% |  |  |  |
| YCbCr 4:2:2 | -0.1% | -0.6% | -0.6% | 0.0% | -0.2% | -0.2% |  |  |  |
| Overall | -0.1% | -0.5% | -0.4% | -0.1% | -0.2% | -0.1% |  |  |  |
|  | -0.1% | -0.6% | -0.4% | -0.1% | 0.2% | -0.1% |  |  |  |
| Enc Time[%] | 97% |  |  | 98% |  |  |  |  |  |
| Dec Time[%] | 101% |  |  | 102% |  |  |  |  |  |
|  | Low delay B HE Main-tier |  |  | Low delay B HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| RGB 4:4:4 | -0.5% | -0.6% | -0.6% | -0.2% | -0.2% | -0.3% |  |  |  |
| YCbCr 4:4:4 | -0.2% | -0.3% | -0.3% | -0.1% | -0.1% | -0.2% |  |  |  |
| YCbCr 4:2:2 | -0.2% | -0.5% | -0.8% | -0.1% | -0.2% | -0.3% |  |  |  |
| Overall | -0.3% | -0.5% | -0.6% | -0.1% | -0.2% | -0.3% |  |  |  |
|  | -0.3% | -0.4% | -0.7% | -0.2% | -0.1% | -0.2% |  |  |  |
| Enc Time[%] | 99% |  |  | 98% |  |  |  |  |  |
| Dec Time[%] | 101% |  |  | 101% |  |  |  |  |  |

In addition, the performance is evaluated to code the screen contents under AhG8 test condition. The results are shown below.

|  | All Intra Main-tier |  |  | All Intra High-tier |  |  | AI Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class F | 0.0% | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB 4:4:4 SC | -0.1% | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB 4:4:4 Animation | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YCbCr 4:4:4 SC | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YCbCr 4:4:4 Animation | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RangeExt | 0.0% | -0.1% | -0.3% | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% |
| RGB 4:4:4 SC (Optional) | -0.2% | -0.1% | -0.2% | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% |
| YCbCr 4:4:4 SC (Optional) | -0.2% | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 97% |  |  | 97% |  |  | 97% |  |  |
| Dec Time[%] | 98% |  |  | 98% |  |  | 98% |  |  |
|  | RA Main-tier |  |  | RA High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| RGB 4:4:4 SC | -0.2% | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% |  |  |  |
| RGB 4:4:4 Animation | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| YCbCr 4:4:4 SC | 0.0% | 0.1% | 0.1% | 0.0% | 0.1% | 0.0% |  |  |  |
| YCbCr 4:4:4 Animation | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| RangeExt | -0.2% | -0.5% | -0.5% | -0.1% | -0.2% | -0.2% |  |  |  |
| RGB 4:4:4 SC (Optional) | -0.3% | -0.3% | -0.4% | -0.2% | -0.2% | -0.2% |  |  |  |
| YCbCr 4:4:4 SC (Optional) | -0.2% | 0.0% | 0.1% | -0.1% | 0.0% | 0.1% |  |  |  |
| Enc Time[%] | 100% |  |  | 100% |  |  |  |  |  |
| Dec Time[%] | 101% |  |  | 101% |  |  |  |  |  |
|  | Low delay B Main-tier |  |  | Low delay B High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | -0.4% | 0.0% | -0.3% | -0.1% | 0.2% | 0.0% |  |  |  |
| Class B | 0.0% | -0.2% | -0.1% | 0.0% | -0.1% | 0.0% |  |  |  |
| RGB 4:4:4 SC | 0.0% | -0.1% | -0.1% | 0.1% | 0.1% | 0.1% |  |  |  |
| RGB 4:4:4 Animation | -0.1% | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% |  |  |  |
| YCbCr 4:4:4 SC | -0.5% | -0.3% | -0.1% | -0.2% | -0.1% | -0.1% |  |  |  |
| YCbCr 4:4:4 Animation | -0.1% | -0.2% | -0.1% | -0.1% | -0.1% | 0.0% |  |  |  |
| RangeExt | -0.2% | -0.5% | -0.5% | -0.1% | -0.2% | -0.2% |  |  |  |
| RGB 4:4:4 SC (Optional) | -0.3% | -0.5% | -0.6% | -0.2% | -0.2% | -0.2% |  |  |  |
| YCbCr 4:4:4 SC (Optional) | 0.1% | 0.3% | 0.4% | 0.7% | 0.8% | 0.8% |  |  |  |
| Enc Time[%] | 97% |  |  | 97% |  |  |  |  |  |
| Dec Time[%] | 101% |  |  | 100% |  |  |  |  |  |

# Conclusions

In this contribution the adaptive SAO offset scaling scheme is proposed, where the offset scale factor is signaled in the slice header. The range of the offset scale factor is changed to expand its application.

# References

- E. Alshina, A. Alshin, W.-S. Kim, J. Sole, M. Karczewicz, A. M. Tourapis, Y.-W. Huang, and S.-T. Hsiang, “AhG 18: On SAO performance at high bit-depth and high bit-rate,” ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Document JCTVC-P0222, Jan. 2014.

- D. Flynn, M. Naccari, C. Rosewarne, J. Sole, G. Sullivan, and T. Suzuki, “High Efficiency Video Coding (HEVC) Range Extensions text specification: Draft 6,” ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Document JCTVC-P1005, Jan. 2014.

- C. Rosewarne, K. Sharman, and D. Flynn, “Common test conditions and software reference configurations for HEVC range extensions,” ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Document JCTVC-P1006, Jan. 2014.

# Proposed Text Changes

#### 7.3.2.2 Sequence parameter set RBSP syntax

| if( sps_extension_present_flag ) { |  |
| --- | --- |
| … |  |
| if( sample_adaptive_offset_enabled_flag ) |  |
| sao_offset_scale_enabled_flag | u(1) |

#### 7.3.2.3 Picture parameter set RBSP syntax

| log2_sao_offset_scale_luma | ue(v) |
| --- | --- |
| log2_sao_offset_scale_chroma | ue(v) |

#### 7.3.6.1 General slice segment header syntax

| if( sao_offset_scale_enabled_flag ) { |  |
| --- | --- |
| if( slice_sao_luma_flag ) |  |
| log2_sao_offset_scale_luma | ue(v) |
| if( slice_sao_chroma_flag ) |  |
| log2_sao_offset_scale_chroma | ue(v) |
| } |  |

#### 7.4.3.3 Sequence parameter set RBSP semantics

sao_offset_scale_enabled_flag equal to 1 specifies that log2_sao_offset_scale_luma and log2_sao_offset_scale_chroma are present in the slice segment header. sao_offset_scale_enabled_flag equal to 0 specifies that log2_sao_offset_scale_luma and log2_sao_offset_scale_chroma are not present in the slice segment header.

#### 7.4.3.3 Picture parameter set RBSP semantics

log2_sao_offset_scale_luma is the base 2 logarithm of the scaling parameter that is used to scale SAO offset values for luma samples. The value of log2_sao_offset_scale_luma shall be in the range of 0 to Max( 0, BitDepthY − 10 ), inclusive. When not present, the value of log2_sao_offset_scale_luma is inferred to be equal to 0.

log2_sao_offset_scale_chromais the base 2 logarithm of the scaling parameter that is used to scale SAO offset values for chroma samples. The value of log2_sao_offset_scale_chroma shall be in the range of 0 to Max( 0, BitDepthC − 10 ), inclusive. When not present, the value of log2_sao_offset_scale_chroma is inferred to be equal to 0.

#### 7.4.7.1 General slice segment header semantics

log2_sao_offset_scale_luma is the base 2 logarithm of the scaling parameter that is used to scale SAO offset values for luma samples. The value of log2_sao_offset_scale_luma shall be in the range of 0 to BitDepthY − 5, inclusive. When not present, the value of log2_sao_offset_scale_luma is inferred to be equal to 0.

log2_sao_offset_scale_chroma is the base 2 logarithm of the scaling parameter that is used to scale SAO offset values for chroma samples. The value of log2_sao_offset_scale_chroma shall be in the range of 0 to BitDepthC − 5, inclusive. When not present, the value of log2_sao_offset_scale_chroma is inferred to be equal to 0.

# Patent rights declaration(s)

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).