| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Valencia, ES, 27 March – 4 April 2014 | Document: JCTVC-Q0139 |
| --- | --- |

| Title: | Intra block copy with larger search region |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chao Pang, Joel Sole, Ted Hsieh, Marta Karczewicz 5775 Morehouse Drive San Diego, CA 92121-1714 USA | Tel: Email: | + 1-858-658-3779 cpang@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

This contribution provides results of the objective compression performance for Intra block copy with a larger search region than 64 columns on the left of the current CTU. When the search region is the entire frame, BD-rate improvements of 19.1%, 16.5% and 14.8% are reported for YUV 4:4:4 SC sequences in AI-MT, RA-MT and LB-MT, respectively.

# Introduction

In current HEVC Range Extensions, the search region for Intra block copy (Intra BC) consists of: (1) the reconstructed region in current CTU; (2) the reconstructed 64 columns of CTU height on the left of current CTU.

# Proposed

In this proposal, the search region for Intra BC is enlarged to: (1) the entire frame; (2) the neighboring rectangular region of current CTU with the size of 5 LCUWidth x 4 LCUHeight. To reduce the additional computational complexity, hash-based fast encoder search is employed.

## Memory bandwidth evaluation

In the following analysis, bandwidth is calculated as number of reference pixels fetched from the memory based on 4x2 and 8x2 memory blocks as described in [1]. It assumes that memory can be accessed and read based on the memory blocks, the chroma storage is not interleaved and only integer number of blocks can be fetched. Let’s denote a block size to be compensated as M×N, memory block as m×n, and L is the interpolation filter length. Note that no interpolation is needed for Intra BC, then for each pixel within M×N block the number of pixels per fetched from the memory can be calculated as follows.

In Table 1, we have listed the worst case results of memory bandwidth for Intra BC, which is 4x4 PU, and Inter, which is 8x8 bi-predicted blocks, respectively. According to Table 1, the memory bandwidth of Intra BC is much less than that of Inter.

Table 1 Memory bandwidth comparison of Intra BC and Inter for 4:4:4

| Memory configuration | Intra BC | Inter |
| --- | --- | --- |
| 4×2 | 9.0 | 22.0 |
| 8×2 | 18.0 | 30.0 |

## Hardware implementation

Increasing the search area to the entire frame has implications in the potential implementation. There is need to store the reconstructed CTUs and need to cache to fetch and store reconstructed CTUs based on the block vectors. This could be similar to the inter motion compensation cache. Of course, there is additional burden at encoder and decoder, but we still can find the balance between complexity and performance in the codec implementation.

# Results

In this section, experimental results are presented when the search region for Intra BC. The proposed method is implemented on top of HM13.0_RExt6.0, and AHG8 test conditions [2] are used. The performance results are compared to the anchor for lossy and lossless configuration in terms of BD-rates and rate savings, respectively.

- Test 1: Search region is the entire frame, lossy (Table 2) and lossless (Table 3);

- Test 2: search region is the neighboring rectangular, lossy (Table 4) and lossless (Table 5).

- Test 3: Search region is the entire frame in lossy LB case by reducing one reference frame for inter and use it as the frame buffer for Intra BC (Table 6);

Table 2 Lossy results of Test 1

|  | All Intra Main-tier |  |  | All Intra High-tier |  |  | All Intra Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class F | -5.2% | -5.0% | -5.0% | -4.6% | -4.5% | -4.5% | -4.0% | -4.0% | -4.0% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB 4:4:4 SC | -19.0% | -18.8% | -18.9% | -18.4% | -18.5% | -18.4% | -17.8% | -17.7% | -17.7% |
| RGB 4:4:4 Animation | -0.1% | -0.1% | -0.1% | 0.0% | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% |
| YCbCr 4:4:4 SC | -19.1% | -18.8% | -19.1% | -18.9% | -18.9% | -19.0% | -18.5% | -18.4% | -18.5% |
| YCbCr 4:4:4 Animation | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% |
| RangeExt | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB 4:4:4 SC (Optional) | -16.7% | -16.6% | -16.6% | -16.8% | -16.7% | -16.8% | -16.6% | -16.3% | -16.3% |
| YCbCr 4:4:4 SC (Optional) | -20.7% | -20.2% | -20.4% | -21.3% | -20.9% | -21.1% | -21.7% | -21.4% | -21.5% |
| Enc Time[%] | 106% |  |  | 105% |  |  | 103% |  |  |
| Dec Time[%] | 95% |  |  | 95% |  |  | 94% |  |  |
|  | Random Access Main-tier |  |  | Random Access High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | -4.0% | -3.9% | -3.7% | -3.6% | -3.5% | -3.5% |  |  |  |
| Class B | 0.0% | 0.0% | -0.1% | -0.1% | 0.0% | -0.1% |  |  |  |
| RGB 4:4:4 SC | -16.7% | -16.5% | -16.5% | -16.4% | -16.3% | -16.3% |  |  |  |
| RGB 4:4:4 Animation | -0.1% | -0.1% | -0.1% | 0.0% | -0.1% | -0.1% |  |  |  |
| YCbCr 4:4:4 SC | -16.5% | -16.2% | -16.3% | -16.6% | -16.5% | -16.5% |  |  |  |
| YCbCr 4:4:4 Animation | -0.1% | -0.2% | -0.2% | -0.1% | -0.1% | -0.1% |  |  |  |
| RangeExt | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% |  |  |  |
| RGB 4:4:4 SC (Optional) | -13.3% | -13.2% | -13.0% | -13.4% | -13.3% | -13.1% |  |  |  |
| YCbCr 4:4:4 SC (Optional) | -17.6% | -17.0% | -17.1% | -18.3% | -17.7% | -17.9% |  |  |  |
| Enc Time[%] | 98% |  |  | 99% |  |  |  |  |  |
| Dec Time[%] | 94% |  |  | 95% |  |  |  |  |  |
|  | Low delay B Main-tier |  |  | Low delay B High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | -3.0% | -2.6% | -3.3% | -2.6% | -2.5% | -2.6% |  |  |  |
| Class B | 0.0% | -0.1% | 0.0% | 0.0% | -0.1% | 0.0% |  |  |  |
| RGB 4:4:4 SC | -15.2% | -14.7% | -14.7% | -15.3% | -14.9% | -14.9% |  |  |  |
| RGB 4:4:4 Animation | -0.1% | -0.1% | -0.1% | -0.1% | 0.0% | 0.0% |  |  |  |
| YCbCr 4:4:4 SC | -14.8% | -14.4% | -14.4% | -15.5% | -15.1% | -15.0% |  |  |  |
| YCbCr 4:4:4 Animation | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| RangeExt | 0.1% | -0.1% | 0.1% | 0.0% | 0.0% | 0.0% |  |  |  |
| RGB 4:4:4 SC (Optional) | -10.2% | -10.0% | -10.1% | -10.7% | -10.3% | -10.5% |  |  |  |
| YCbCr 4:4:4 SC (Optional) | -14.6% | -14.2% | -14.3% | -15.4% | -15.1% | -15.1% |  |  |  |
| Enc Time[%] | 100% |  |  | 101% |  |  |  |  |  |
| Dec Time[%] | 96% |  |  | 97% |  |  |  |  |  |

Table 3 Lossless results of Test 1

|  | AI | RA | LB |
| --- | --- | --- | --- |
| Class F | -2.7% | -1.5% | -0.8% |
| Class B | 0.0% | 0.0% | 0.0% |
| RGB 4:4:4 SC | -14.2% | -13.2% | -13.1% |
| RGB 4:4:4 Animation | 0.0% | 0.0% | 0.0% |
| YCbCr 4:4:4 SC | -15.4% | -14.5% | -14.3% |
| YCbCr 4:4:4 Animation | 0.0% | 0.0% | 0.0% |
| RangeExt | 0.0% | 0.0% | 0.0% |
| RGB 4:4:4 SC (Optional) | -15.5% | -13.3% | -10.1% |
| YCbCr 4:4:4 SC (Optional) | -20.1% | -17.8% | -16.1% |
| Enc Time[%] | 117% | 103% | 100% |
| Dec Time[%] | 102% | 98% | 95% |

Table 4 Lossy results of Test 2

|  | All Intra Main-tier |  |  | All Intra High-tier |  |  | All Intra Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class F | -3.1% | -2.9% | -2.9% | -2.8% | -2.7% | -2.6% | -2.5% | -2.4% | -2.4% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB 4:4:4 SC | -12.5% | -12.3% | -12.4% | -12.0% | -11.9% | -12.0% | -11.5% | -11.3% | -11.4% |
| RGB 4:4:4 Animation | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YCbCr 4:4:4 SC | -12.7% | -12.4% | -12.4% | -12.5% | -12.5% | -12.3% | -12.1% | -12.0% | -12.0% |
| YCbCr 4:4:4 Animation | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RangeExt | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB 4:4:4 SC (Optional) | -11.4% | -11.2% | -11.2% | -11.4% | -11.3% | -11.3% | -11.0% | -10.6% | -10.5% |
| YCbCr 4:4:4 SC (Optional) | -14.0% | -13.5% | -13.5% | -14.2% | -13.9% | -13.9% | -14.2% | -14.1% | -14.0% |
| Enc Time[%] | 98% |  |  | 97% |  |  | 96% |  |  |
| Dec Time[%] | 93% |  |  | 92% |  |  | 91% |  |  |
|  | Random Access Main-tier |  |  | Random Access High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | -2.5% | -2.2% | -2.2% | -2.2% | -2.1% | -2.1% |  |  |  |
| Class B | 0.0% | -0.1% | 0.0% | -0.1% | -0.1% | 0.0% |  |  |  |
| RGB 4:4:4 SC | -10.7% | -10.5% | -10.5% | -10.3% | -10.2% | -10.2% |  |  |  |
| RGB 4:4:4 Animation | 0.0% | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% |  |  |  |
| YCbCr 4:4:4 SC | -10.6% | -10.2% | -10.2% | -10.5% | -10.3% | -10.2% |  |  |  |
| YCbCr 4:4:4 Animation | 0.0% | -0.1% | -0.1% | 0.0% | -0.1% | 0.0% |  |  |  |
| RangeExt | 0.0% | -0.1% | -0.1% | 0.0% | 0.0% | -0.1% |  |  |  |
| RGB 4:4:4 SC (Optional) | -9.5% | -9.4% | -9.3% | -9.5% | -9.5% | -9.4% |  |  |  |
| YCbCr 4:4:4 SC (Optional) | -12.5% | -12.0% | -12.0% | -12.9% | -12.6% | -12.5% |  |  |  |
| Enc Time[%] | 94% |  |  | 94% |  |  |  |  |  |
| Dec Time[%] | 93% |  |  | 93% |  |  |  |  |  |
|  | Low delay B Main-tier |  |  | Low delay B High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | -1.6% | -1.2% | -1.5% | -1.4% | -1.3% | -1.4% |  |  |  |
| Class B | 0.0% | 0.0% | -0.2% | 0.0% | -0.1% | 0.0% |  |  |  |
| RGB 4:4:4 SC | -9.2% | -8.9% | -9.0% | -9.3% | -9.0% | -9.0% |  |  |  |
| RGB 4:4:4 Animation | -0.1% | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% |  |  |  |
| YCbCr 4:4:4 SC | -9.0% | -8.7% | -8.9% | -9.4% | -9.1% | -9.0% |  |  |  |
| YCbCr 4:4:4 Animation | 0.0% | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% |  |  |  |
| RangeExt | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.1% |  |  |  |
| RGB 4:4:4 SC (Optional) | -6.5% | -6.4% | -6.5% | -6.4% | -6.1% | -6.3% |  |  |  |
| YCbCr 4:4:4 SC (Optional) | -9.9% | -9.5% | -9.5% | -10.2% | -9.8% | -9.9% |  |  |  |
| Enc Time[%] | 95% |  |  | 95% |  |  |  |  |  |
| Dec Time[%] | 93% |  |  | 93% |  |  |  |  |  |

Table 5 Lossless result of Test 2

|  | AI | RA | LB |
| --- | --- | --- | --- |
| Class F | -1.5% | -0.8% | -0.3% |
| Class B | 0.0% | 0.0% | 0.0% |
| RGB 4:4:4 SC | -8.9% | -7.8% | -7.7% |
| RGB 4:4:4 Animation | 0.0% | 0.0% | 0.0% |
| YCbCr 4:4:4 SC | -9.8% | -8.8% | -8.5% |
| YCbCr 4:4:4 Animation | 0.0% | 0.0% | 0.0% |
| RangeExt | 0.0% | 0.0% | 0.0% |
| RGB 4:4:4 SC (Optional) | -8.2% | -7.1% | -5.5% |
| YCbCr 4:4:4 SC (Optional) | -12.7% | -12.0% | -9.2% |
| Enc Time[%] | 108% | 103% | 109% |
| Dec Time[%] | 101% | 99% | 105% |

Table 6 Lossy results of Test 3

|  | Low delay B Main-tier |  |  | Low delay B High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class F | -0.4% | 0.1% | 0.2% | -0.3% | -0.2% | 0.1% |
| Class B | 0.8% | 0.9% | 1.3% | 0.8% | 0.9% | 1.0% |
| RGB 4:4:4 SC | -14.7% | -14.1% | -14.2% | -14.8% | -14.3% | -14.3% |
| RGB 4:4:4 Animation | 1.2% | 1.1% | 1.1% | 1.2% | 1.2% | 1.2% |
| YCbCr 4:4:4 SC | -14.1% | -13.7% | -13.6% | -14.8% | -14.3% | -14.1% |
| YCbCr 4:4:4 Animation | 1.1% | 1.1% | 1.2% | 1.2% | 1.4% | 1.4% |
| RangeExt | 1.5% | 1.5% | 1.6% | 1.7% | 1.4% | 1.7% |
| RGB 4:4:4 SC (Optional) | -9.0% | -8.7% | -8.9% | -9.8% | -9.4% | -9.6% |
| YCbCr 4:4:4 SC (Optional) | -12.0% | -11.0% | -11.2% | -12.7% | -11.8% | -11.9% |
| Enc Time[%] | 89% |  |  | 89% |  |  |
| Dec Time[%] | 97% |  |  | 97% |  |  |

# Draft text changes

When the search region for Intra BC is enlarged to the entire frame, the draft text changes are as follows:

In 8.4.4

….

It is a requirement of bitstream conformance that all of the the following conditions are true:

–	The value of bvIntra[ xPb ][ yPb ][ 0 ] shall be greater than or equal to – ( xPb % CtbSizeY + 64 ).

–	The value of bvIntra[ xPb ][ yPb ][ 1 ] shall be greater than or equal to – ( yPb % CtbSizeY ).

–	When the derivation process for z-scan order block availability as specified in subclause Error: Reference source not found is invoked with ( xCurr, yCurr ) set equal to ( xCb, yCb ) and the neighbouring luma location ( xNbY, yNbY ) set equal to ( xPb + bvIntra[ xPb ][ yPb ][ 0 ], yPb + bvIntra[ xPb ][ yPb ][ 1 ] ) as inputs, the output shall be equal to TRUE.

–	When the derivation process for z-scan order block availability as specified in subclause Error: Reference source not found is invoked with ( xCurr, yCurr ) set equal to ( xCb, yCb ) and the neighbouring luma location ( xNbY, yNbY ) set equal to ( xPb + bvIntra[ xPb ][ yPb ][ 0 ] + nPbSw − 1, yPb + bvIntra[ xPb ][ yPb ][ 1 ] + nPbSh – 1 ) as inputs, the output shall be equal to TRUE.

–	When entropy_coding_sync_enabled_flag is equal to 1, the following condition shall be true

–	The value of xPb % CtbSizeY + bvIntra[ xPb ][ yPb ][ 0 ]  shall be less than or equal to  (CurrCtby – RefCtby) << 1

where CurrCtby =  (yPb/ CtbSizeY) * CtbSizeY

RefCtby = ((yPb + bvIntra[ xPb ][ yPb ][ 1 ])/ CtbSizeY) * CtbSizeY

–	One or both of the following conditions shall be true:

–	bvIntra[ xPb ][ yPb ][ 0 ] + nPbSw  <=  0

–	bvIntra[ xPb ][ yPb ][ 1 ] + nPbSh  <=  0

# References

[1] E. François, A. Tabatabai, and E. Alshina “BoG report: Methodology for evaluating complexity of combined and residual prediction methods in SHVC”, JCTVC-L0440, Geneva, CH, Jan. 2013.

[2] C. Rosewarne, K. Sharman, and D. Flynn, “Common test conditions and software reference configurations for HEVC range extensions”, JCTVC-P1006, San José, US, January 2014.

# Patent rights declaration(s)

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).