| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Valencia, ES, 27 March – 4 April 2014 | Document: JCTVC-Q0140 |
| --- | --- |

| Title: | AhG8: Performance of encoder and parameter only changes for Screen Content Coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Joel Sole, Chao Pang, Li Zhang, Krishna Rapaka, Marta Karczewicz 5775 Morehouse Drive San Diego, CA 92121-1714 USA | Tel: Email: | + 1-858-658-3779 joels@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

This contribution provides results of the objective compression performance of some encoder and parameter modifications of HEVC RExt Profile for screen content coding. BD-rate improvements of 31.3% and 26.7% are reported for the RGB 4:4:4 SC and the YUV 4:4:4 SC classes.

# Description

No new tools are introduced in the HEVC Range Extensions design. The modifications consist of encoder-only improvements and changing parameters for some existing tools. Main changes are the following ones:

- Search area for intra BC is increased with respect to the reference: full frame search and hash-table at the encoder side are used as described in [1].

- Transform skip size is increased with respect to the reference: a maximum transform-skip size of 8×8 is used (instead of 4).

- Refinement inter-frame motion estimation for screen content coding as in [2].

- Use of uniform quantization for RDPCM as in [3].

- Refinement of intra block copying search by considering the chroma components as in [4].

# Performance

In this section, experimental results are presented with the proposed encoder and parameter only changes. The proposed method is implemented on top of HM13.0_RExt6.0, and the performance results are compared in terms BD-rate gains under the following tests.

- Test 1: anchor is HM-13.0+RExt-6.0 with CfP sequences in lossy case (Table 1);

- Test 2: anchor is CfP anchor in lossy case (Table 2);

- Test 3: anchor is AVC High 4:4:4[5] (Table 3);

Table 1 BD-rate gains with respect to HM-13.0+RExt-6.0

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| RGB, text & graphics with motion, 1080p | -31.3% | -27.6% | -27.7% |
| RGB, text & graphics with motion,720p | -22.9% | -16.9% | -17.4% |
| RGB, mixed content, 1440p | -19.6% | -14.2% | -14.4% |
| RGB, mixed content, 1080p | -17.1% | -11.9% | -11.8% |
| RGB, Animation, 720p | -8.8% | 0.5% | 1.2% |
| YUV, text & graphics with motion, 1080p | -26.7% | -27.0% | -26.7% |
| YUV, text & graphics with motion,720p | -15.4% | -15.8% | -16.2% |
| YUV, mixed content, 1440p | -14.7% | -14.8% | -14.8% |
| YUV, mixed content, 1080p | -12.5% | -13.0% | -12.7% |
| YUV, Animation, 720p | -0.1% | -0.8% | -0.6% |
| Enc Time[%] | 117% |  |  |
| Dec Time[%] | 93% |  |  |
|  | Random Access |  |  |
|  | Y | U | V |
| RGB, text & graphics with motion, 1080p | -30.8% | -26.2% | -26.0% |
| RGB, text & graphics with motion,720p | -29.3% | -22.1% | -23.9% |
| RGB, mixed content, 1440p | -28.6% | -22.0% | -22.4% |
| RGB, mixed content, 1080p | -25.6% | -15.1% | -14.5% |
| RGB, Animation, 720p | -14.2% | -6.0% | -2.3% |
| YUV, text & graphics with motion, 1080p | -25.9% | -28.8% | -28.7% |
| YUV, text & graphics with motion,720p | -20.8% | -22.4% | -25.1% |
| YUV, mixed content, 1440p | -19.7% | -23.2% | -23.1% |
| YUV, mixed content, 1080p | -15.6% | -20.6% | -19.7% |
| YUV, Animation, 720p | -1.5% | -10.9% | -13.1% |
| Enc Time[%] | 102% |  |  |
| Dec Time[%] | 96% |  |  |
|  | Low delay B |  |  |
|  | Y | U | V |
| RGB, text & graphics with motion, 1080p | -29.1% | -22.9% | -21.8% |
| RGB, text & graphics with motion,720p | -31.8% | -21.3% | -23.3% |
| RGB, mixed content, 1440p | -41.2% | -32.0% | -32.2% |
| RGB, mixed content, 1080p | -30.6% | -18.2% | -17.0% |
| RGB, Animation, 720p | -22.6% | -5.2% | -3.1% |
| YUV, text & graphics with motion, 1080p | -23.8% | -27.8% | -27.3% |
| YUV, text & graphics with motion,720p | -19.8% | -22.9% | -27.6% |
| YUV, mixed content, 1440p | -29.4% | -36.8% | -37.0% |
| YUV, mixed content, 1080p | -18.5% | -23.9% | -23.3% |
| YUV, Animation, 720p | -7.0% | -14.2% | -18.0% |
| Enc Time[%] | 97% |  |  |
| Dec Time[%] | 90% |  |  |

Table 2 BD-rate gains with respect to CfP anchor

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| RGB, text & graphics with motion, 1080p | -41.0% | -37.6% | -37.6% |
| RGB, text & graphics with motion,720p | -29.2% | -23.5% | -24.0% |
| RGB, mixed content, 1440p | -24.6% | -19.4% | -19.6% |
| RGB, mixed content, 1080p | -23.8% | -19.2% | -19.1% |
| RGB, Animation, 720p | -9.5% | -0.4% | 0.3% |
| YUV, text & graphics with motion, 1080p | -36.5% | -36.7% | -36.4% |
| YUV, text & graphics with motion,720p | -21.6% | -22.2% | -22.3% |
| YUV, mixed content, 1440p | -19.8% | -20.1% | -20.1% |
| YUV, mixed content, 1080p | -19.1% | -19.9% | -19.6% |
| YUV, Animation, 720p | -0.8% | -1.5% | -1.2% |
| Enc Time[%] | 169% |  |  |
| Dec Time[%] | 73% |  |  |
|  | Random Access |  |  |
|  | Y | U | V |
| RGB, text & graphics with motion, 1080p | -36.0% | -31.4% | -31.1% |
| RGB, text & graphics with motion,720p | -33.8% | -26.7% | -28.4% |
| RGB, mixed content, 1440p | -31.3% | -25.0% | -25.4% |
| RGB, mixed content, 1080p | -29.0% | -18.8% | -18.2% |
| RGB, Animation, 720p | -14.9% | -6.7% | -2.9% |
| YUV, text & graphics with motion, 1080p | -30.6% | -33.5% | -33.3% |
| YUV, text & graphics with motion,720p | -25.3% | -27.0% | -29.6% |
| YUV, mixed content, 1440p | -22.8% | -26.7% | -26.8% |
| YUV, mixed content, 1080p | -19.2% | -24.2% | -23.4% |
| YUV, Animation, 720p | -1.9% | -11.3% | -13.3% |
| Enc Time[%] | 109% |  |  |
| Dec Time[%] | 92% |  |  |
|  | Low delay B |  |  |
|  | Y | U | V |
| RGB, text & graphics with motion, 1080p | -31.9% | -25.5% | -24.4% |
| RGB, text & graphics with motion,720p | -34.4% | -23.6% | -25.9% |
| RGB, mixed content, 1440p | -42.7% | -33.6% | -33.8% |
| RGB, mixed content, 1080p | -32.5% | -20.1% | -19.0% |
| RGB, Animation, 720p | -23.2% | -5.6% | -3.5% |
| YUV, text & graphics with motion, 1080p | -26.1% | -30.2% | -29.7% |
| YUV, text & graphics with motion,720p | -22.0% | -25.2% | -29.6% |
| YUV, mixed content, 1440p | -31.0% | -38.5% | -38.9% |
| YUV, mixed content, 1080p | -20.5% | -25.9% | -25.2% |
| YUV, Animation, 720p | -7.2% | -14.1% | -17.8% |
| Enc Time[%] | 101% |  |  |
| Dec Time[%] | 86% |  |  |

Table 3 BD-rate gains with respect to AVC High 4:4:4

|  | All Intra Main-tier |  |  | All Intra High-tier |  |  | All Intra Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class F 4:2:0 SC | -43.2% | -40.6% | -41.7% | -44.7% | -37.8% | -38.3% | -43.8% | -35.5% | -36.2% |
| Class B 4:2:0 Natural | -22.7% | -19.2% | -17.9% | -19.5% | -19.4% | -19.4% | -13.5% | -17.6% | -19.6% |
| RGB 4:4:4 SC | -78.4% | -75.2% | -75.5% | -79.2% | -77.1% | -77.4% | -77.6% | -75.9% | -75.9% |
| RGB 4:4:4 Animation | -47.8% | -38.8% | -37.3% | -43.8% | -37.2% | -36.4% | -38.8% | -32.3% | -31.9% |
| YCbCr 4:4:4 SC | -64.0% | -61.7% | -63.2% | -65.5% | -61.8% | -63.2% | -64.9% | -61.3% | -62.1% |
| YCbCr 4:4:4 Animation | -25.4% | -30.1% | -25.8% | -23.2% | -28.4% | -25.4% | -20.8% | -24.3% | -22.3% |
| RangeExt 4:4:4 Natural | -20.9% | -14.4% | -17.8% | -17.1% | -14.8% | -18.6% | -12.0% | -12.7% | -14.8% |
| RGB 4:4:4 SC (Optional) | -93.1% | -92.9% | -93.1% | -92.7% | -92.7% | -92.8% | -91.7% | -91.3% | -91.3% |
| YCbCr 4:4:4 SC (Optional) | -87.6% | -83.6% | -83.6% | -87.9% | -84.4% | -84.1% | -86.6% | -85.3% | -85.1% |
| Enc Time[%] | #VALUE! |  |  | #VALUE! |  |  | #VALUE! |  |  |
| Dec Time[%] | #VALUE! |  |  | #VALUE! |  |  | #VALUE! |  |  |
|  | Random Access Main-tier |  |  | Random Access High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F 4:2:0 SC | -44.8% | -51.9% | -52.5% | -43.8% | -47.9% | -47.8% |  |  |  |
| Class B 4:2:0 Natural | -38.0% | -40.9% | -44.8% | -32.2% | -35.4% | -43.5% |  |  |  |
| RGB 4:4:4 SC | -76.3% | -71.7% | -72.9% | -75.0% | -71.5% | -72.3% |  |  |  |
| RGB 4:4:4 Animation | -50.6% | -45.0% | -41.4% | -45.6% | -41.0% | -36.8% |  |  |  |
| YCbCr 4:4:4 SC | -63.1% | -63.1% | -65.9% | -63.4% | -62.6% | -65.1% |  |  |  |
| YCbCr 4:4:4 Animation | -38.3% | -49.5% | -44.8% | -30.4% | -45.7% | -40.9% |  |  |  |
| RangeExt 4:4:4 Natural | -29.0% | -23.3% | -19.1% | -24.7% | -31.4% | -26.2% |  |  |  |
| RGB 4:4:4 SC (Optional) | -90.0% | -89.8% | -90.0% | -91.0% | -90.9% | -91.0% |  |  |  |
| YCbCr 4:4:4 SC (Optional) | -81.9% | -82.6% | -82.6% | -84.0% | -83.4% | -83.1% |  |  |  |
| Enc Time[%] | #VALUE! |  |  | #VALUE! |  |  |  |  |  |
| Dec Time[%] | #VALUE! |  |  | #VALUE! |  |  |  |  |  |
|  | Low delay B Main-tier |  |  | Low delay B High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F 4:2:0 SC | -46.1% | -61.3% | -62.3% | -44.2% | -53.7% | -53.5% |  |  |  |
| Class B 4:2:0 Natural | -44.4% | -36.5% | -44.8% | -35.6% | -25.9% | -32.7% |  |  |  |
| RGB 4:4:4 SC | -72.2% | -65.4% | -67.1% | -70.5% | -65.5% | -66.5% |  |  |  |
| RGB 4:4:4 Animation | -55.9% | -48.2% | -44.6% | -50.0% | -42.8% | -38.3% |  |  |  |
| YCbCr 4:4:4 SC | -60.3% | -61.2% | -64.7% | -60.2% | -59.6% | -62.8% |  |  |  |
| YCbCr 4:4:4 Animation | -46.0% | -60.5% | -58.8% | -35.6% | -56.0% | -53.5% |  |  |  |
| RangeExt 4:4:4 Natural | -36.8% | -38.0% | -34.2% | -32.2% | -38.0% | -40.5% |  |  |  |
| RGB 4:4:4 SC (Optional) | -87.7% | -87.1% | -87.2% | -89.9% | -89.5% | -89.6% |  |  |  |
| YCbCr 4:4:4 SC (Optional) | -78.3% | -80.4% | -80.2% | -81.6% | -81.7% | -80.9% |  |  |  |
| Enc Time[%] | #VALUE! |  |  | #VALUE! |  |  |  |  |  |
| Dec Time[%] | #VALUE! |  |  | #VALUE! |  |  |  |  |  |

# Conclusions

Additional large coding performance gains can be obtained with the current HEVC Range Extensions design for screen content coding without introducing new tools.

# References

- “Description of screen content coding technology proposal by Qualcomm”, JCTVC-Q0031, Valenica, ES, March 2014.

- K. Rapaka, J. Sole, L. Zhang, M. Karczewicz, “AhG8: On fast inter search method for screen content coding”, JCTVC-Q0147, Valenica, ES, March 2014.

- F. Zou, R. Joshi, J. Sole, C. Pang, M. Karczewicz, “Quantization rounding for RDPCM”, JCTVC-Q0148, Valenica, ES, March 2014.

- C. Pang, J. Sole, M. Karczewicz, “Intra block copy with encoder search using chroma component”, JCTVC-Q0175, Valenica, ES, March 2014.

- B. Li, J. Xu, G. Sullivan, “Comparison of Compression Performance of HEVC Range Extensions Draft 4 with AVC High 4:4:4 Predictive profile,” JCTVC-O0184, Geneva, CH, Nov. 2013.

# Patent rights declaration(s)

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).