| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Valencia, ES, 27 March – 4 April 2014 | Document: JCTVC-Q0141 |
| --- | --- |

| Title: | Non-SCE1: Coding of color gamut prediction coefficients |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kiran Misra, Seung-Hwan Kim, Andrew Segall 5705 NW Pacific Rim Blvd Camas WA 98607 USA | Tel: Email: | +1 360 817 8451 misrak@sharplabs.com kimse@sharplabs.com asegall@sharplabs.com |
| Source: | SHARP Laboratories of America |  |  |

_____________________________

# Abstract

This contribution proposes an alternate technique for coding of color gamut prediction coefficients presented in JCTVC-Q0072. The performance of the proposed approach was evaluated with SCE1 test conditions as specified in JCTVC-P1101. The average luma BD rate improvement of the proposed approach is reported to be:

Configuration 1, JCTVC-Q0072: 9.0% (All intra 1x), 9.1% (All intra 2x), 9.8% (Random access 1x) and 6.5% (Random access 2x)

Configuration 3, JCTVC-Q0072: 5.7% (All intra 1x), 5.5% (All intra 2x), 6.9% (Random access 1x) and 4.3% (Random access 2x).

# Introduction

JCTVC-Q0072 proposed four techniques on inter-layer prediction for color gamut scalability: (i) 8x8x8 region based matrix mapping, (ii) linear matrix mapping, (iii) 8x8x8 region based gain-offset mapping, and (iv) polynomial matrix mapping.  Of these four approaches, configuration 1 and 3 have the most number of regions: 8x8x8 and therefore the most number of color prediction coefficient sets to be signalled. The large numbers of color prediction coefficient sets have a non-negligible overhead.

This contribution proposes to modify the color prediction coefficient signaling method in JCTVC-Q0072. The asserted benefit is increased coding efficiency.  In this contribution, the color prediction coefficients are proposed to be decomposed into two parts: absolute value and sign information. The absolute value is signalled as Quotient and Remainder based on a predetermined divisor (1<<7). The sign information is signaled separately as a flag and only if the absolute value is non-zero.

The value of the color prediction coefficients are derived as follows:

Coefficient = (Quotient << 7) + Remainder

Coefficient = (Sign==0) ? Coefficient : −Coefficient

Note that no prediction technique is applied between coefficients.

# Experimental Results

Performance of the proposed signaling was determined using settings described in JCTVC-P1101 [1]. Summary results for the proposed modified signaling are listed below for configurations 1 and 3 of JCTVC-Q0072. Detailed results are in the attached excel sheet.

## Configuration 1: 8x8x8, matrix mapping

|  | AI HEVC 1x 10-bit base |  |  | AI HEVC 2x 8-bit base |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A+ | -9.0% | -16.1% | -19.2% | -9.1% | -12.9% | -17.3% |
| Overall (Test vs Ref) | -9.0% | -16.1% | -19.2% | -9.1% | -12.9% | -17.3% |
| Overall (Test vs single layer) | 20.1% | 10.5% | -9.3% | 9.9% | 8.4% | -0.9% |
| Overall (Ref vs single layer) | 32.3% | 31.8% | 11.7% | 21.1% | 24.7% | 19.4% |
| EL only (Test vs Ref) | -34.1% | -38.5% | -41.6% | -17.9% | -21.1% | -25.6% |
| Overall (Test EL+BL vs single EL+BL) | -36.0% | -41.5% | -52.4% | -28.0% | -28.6% | -36.1% |
|  | RA HEVC 1x 10-bit base |  |  | RA HEVC 2x 8-bit base |  |  |
|  | Y | U | V | Y | U | V |
| Class A+ | -9.8% | -16.0% | -19.5% | -6.5% | -10.2% | -14.3% |
| Overall (Test vs Ref) | -9.8% | -16.0% | -19.5% | -6.5% | -10.2% | -14.3% |
| Overall (Test vs single layer) | 30.4% | 24.4% | 2.1% | 20.9% | 19.8% | 9.3% |
| Overall (Ref vs single layer) | 45.0% | 48.8% | 26.2% | 29.3% | 33.2% | 27.6% |
| EL only (Test vs Ref) | -29.4% | -33.9% | -36.5% | -12.1% | -15.5% | -19.5% |
| Overall (Test EL+BL vs single EL+BL) | -29.0% | -31.9% | -43.9% | -19.4% | -18.1% | -26.7% |

## Configuration 3: 8x8x8, gain-offset mapping

|  | AI HEVC 1x 10-bit base |  |  | AI HEVC 2x 8-bit base |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A+ | -5.7% | -6.6% | -12.5% | -5.5% | -4.8% | -10.2% |
| Overall (Test vs Ref) | -5.7% | -6.6% | -12.5% | -5.5% | -4.8% | -10.2% |
| Overall (Test vs single layer) | 24.7% | 23.1% | -1.9% | 14.5% | 18.9% | 7.5% |
| Overall (Ref vs single layer) | 32.3% | 31.8% | 11.7% | 21.1% | 24.7% | 19.4% |
| EL only (Test vs Ref) | -24.1% | -24.2% | -29.5% | -11.4% | -10.2% | -15.7% |
| Overall (Test EL+BL vs single EL+BL) | -33.8% | -35.1% | -48.8% | -24.9% | -22.1% | -30.5% |
|  | RA HEVC 1x 10-bit base |  |  | RA HEVC 2x 8-bit base |  |  |
|  | Y | U | V | Y | U | V |
| Class A+ | -6.9% | -8.4% | -13.9% | -4.3% | -4.8% | -9.5% |
| Overall (Test vs Ref) | -6.9% | -8.4% | -13.9% | -4.3% | -4.8% | -9.5% |
| Overall (Test vs single layer) | 34.7% | 36.1% | 8.9% | 23.7% | 27.1% | 15.4% |
| Overall (Ref vs single layer) | 45.0% | 48.8% | 26.2% | 29.3% | 33.2% | 27.6% |
| EL only (Test vs Ref) | -21.5% | -22.3% | -27.0% | -8.4% | -8.5% | -13.2% |
| Overall (Test EL+BL vs single EL+BL) | -26.9% | -26.4% | -40.8% | -17.7% | -13.9% | -22.8% |

# Conclusion

This contribution proposes a method to reduce the overhead of signaling color gamut prediction coefficients in JCTVC-Q0072.  The performance of this approach was evaluated for JCTVC-Q0072 configuration 1 and 3 with SCE1 test conditions as specified in JCTVC-P1101. The average luma BD rate improvement of the proposed approach was found to be:

Configuration 1: 9.0% (All intra 1x), 9.1% (All intra 2x), 9.8% (Random access 1x) and 6.5% (Random access 2x)

Configuration 3: 5.7% (All intra 1x), 5.5% (All intra 2x), 6.9% (Random access 1x) and 4.3% (Random access 2x).

We propose that the modified coefficient signaling be considered for adoption along with JCTVC-Q0072 configuration 1 and 3.

# References

[1] A. Duenas, P. Andrivon, E. Alshina, Y. Ye, K. Ugur, X. Li, “HEVC Scalable Extensions Core Experiment SCE1 description: Colour Gamut and Bit Depth Scalability,” JCTVC-P1101, San Jose, USA, Jan 2014.

[2] A. Aminlou, K. Ugur, M. M. Hannuksela, “SCE1: Color gamut scalability using gain-offset models,” JCTVC-Q0072, Valencia, Spain, March-April, 2014.

SHARP may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Specification text

| … |  |
| --- | --- |
| if( nuh_layer_id > 0 && NumActiveRefLayerPics > 0 ) { |  |
| cgs_matrix( ) |  |
| … |  |

Configuration 1

| cgs_matrix( ) { |  |
| --- | --- |
| for( n = 0; n < numRefLayerPics; n++ ) { |  |
| cgs_enabled_flag[ n ] | u(1) |
| if( cgs_enabled_flag ) { |  |
| cgs_coeff_precision[ n ] | ue(v) |
| for( i = 0; i < 8; i++ ) { |  |
| for( j = 0; j < 8; j++ ) { |  |
| for( k = 0; k < 8; k++ ) { |  |
| cgs_present_flag[ i ][ j ][ k ][ n ] | u(1) |
| if( cgs_present_flag[ i ][ j ][ k ][ n ] ) { |  |
| for( cIdx = 0; cIdx < 3; cIdx++ ) { |  |
| for( d = 0; d < 4;d++ ) { |  |
| cgs_coeff_q[ cIdx ][ d ][ i ][ j ][ k ][ n ] | ue(v) |
| cgs_coeff_r[ cIdx ][ d ][ i ][ j ][ k ][ n ] | u(7) |
| if ( cgs_coeff_q[ cIdx ][ d ][ i ][ j ][ k ][ n ] ¦ ¦ cgs_coeff_r[ cIdx ][ d ][ i ][ j ][ k ][ n ] ) |  |
| cgs_coeff_s[ cIdx ][ d ][ i ][ j ][ k ][ n ] | u(1) |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |

cgs_enabled_flag[ n ] equal to 1 specifies that color gamut mapping is applied to reference layer picture to derive the inter-layer reference picture. cgs_enabled_flag[ n ] equal to 0 specifies that color gamut mapping is not applied to reference layer picture to derive the inter-layer reference picture.

cgs_coeff_precision[ n ] specifies the precision for the color gamut mapping coefficients used in color gamut mapping process.

cgs_present_flag[ i ][ j ][ k ][ n ] equal to 1 specifies that syntax elements cgs_coeff[ cIdx ][ d ][ i ][ j ][ k ] are present for the region specified with [ i ][ j ][ k ]. cgs_present_flag[ i ][ j ][ k ] equal to 0 specifies that syntax elements cgs_coeff[ cIdx  ][ d ][ i ][ j ][ k ] are not present and the corresponding default values are used.

cgs_coeff_q[ cIdx ][ d ][ i ][ j ][ k ][ n ] specifies the quotient of the color gamut mapping coefficients for component indicated by cIdx

cgs_coeff_r[ cIdx ][ d ][ i ][ j ][ k ][ n ] specifies the remainder of the color gamut mapping coefficients for component indicated by cIdx

cgs_coeff_s[ cIdx ][ d ][ i ][ j ][ k ][ n ] specifies the sign of the color gamut mapping coefficients for component indicated by cIdx. When not present the value of cgs_coeff_s[ cIdx ][ d ][ i ][ j ][ k ][ n ] is inferred to be equal to 0.

The value of cgs_coeff[ cIdx ][ d ][ i ][ j ][ k ][ n ] is derived as follows:

cgs_coeff[ cIdx ][ d ][ i ][ j ][ k ][ n ] = (cgs_coeff_q[ cIdx ][ d ][ i ][ j ][ k ][ n ] << 7) + 
								cgs_coeff_r[ cIdx ][ d ][ i ][ j ][ k ][ n ]

cgs_coeff[ cIdx ][ d ][ i ][ j ][ k ][ n ]= (cgs_coeff_s[ cIdx ][ d ][ i ][ j ][ k ][ n ]==0) ? 
					cgs_coeff[ cIdx ][ d ][ i ][ j ][ k ][ n ] : −cgs_coeff[ cIdx ][ d ][ i ][ j ][ k ][ n ]

Configuration 3:

| cgs_matrix( ) { |  |
| --- | --- |
| for( n = 0; n < numRefLayerPics; n++ ) { |  |
| cgs_enabled_flag[ n ] | u(1) |
| if( cgs_enabled_flag ) { |  |
| cgs_coeff_precision[ n ] | ue(v) |
| for( i = 0; i < 8; i++ ) { |  |
| for( j = 0; j < 8; j++ ) { |  |
| for( k = 0; k < 8; k++ ) { |  |
| cgs_present_flag[ i ][ j ][ k ][ n ] | u(1) |
| if( cgs_present_flag[ i ][ j ][ k ][ n ] ) { |  |
| for( cIdx = 0; cIdx < 3; cIdx++ ) { |  |
| for( d = 0; d < 2;d++ ) { |  |
| cgs_coeff_q[ cIdx ][ d ][ i ][ j ][ k ][ n ] | ue(v) |
| cgs_coeff_r[ cIdx ][ d ][ i ][ j ][ k ][ n ] | u(7) |
| if ( cgs_coeff_q[ cIdx ][ d ][ i ][ j ][ k ][ n ] ¦ ¦ cgs_coeff_r[ cIdx ][ d ][ i ][ j ][ k ][ n ] ) |  |
| cgs_coeff_s[ cIdx ][ d ][ i ][ j ][ k ][ n ] | u(1) |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |

cgs_enabled_flag[ n ] equal to 1 specifies that color gamut mapping is applied to reference layer picture to derive the inter-layer reference picture. cgs_enabled_flag[ n ] equal to 0 specifies that color gamut mapping is not applied to reference layer picture to derive the inter-layer reference picture.

cgs_coeff_precision[ n ] specifies the precision for the color gamut mapping coefficients used in color gamut mapping process.

cgs_present_flag[ i ][ j ][ k ][ n ] equal to 1 specifies that syntax elements cgs_coeff[ cIdx ][ d ][ i ][ j ][ k ] are present for the region specified with [ i ][ j ][ k ]. cgs_present_flag[ i ][ j ][ k ] equal to 0 specifies that syntax elements cgs_coeff[ cIdx  ][ d ][ i ][ j ][ k ] are not present and the corresponding default values are used.

cgs_coeff_q[ cIdx ][ d ][ i ][ j ][ k ][ n ] specifies the quotient of the color gamut mapping coefficients for component indicated by cIdx

cgs_coeff_r[ cIdx ][ d ][ i ][ j ][ k ][ n ] specifies the remainder of the color gamut mapping coefficients for component indicated by cIdx

cgs_coeff_s[ cIdx ][ d ][ i ][ j ][ k ][ n ] specifies the sign of the color gamut mapping coefficients for component indicated by cIdx. When not present the value of cgs_coeff_s[ cIdx ][ d ][ i ][ j ][ k ][ n ] is inferred to be equal to 0.

The value of cgs_coeff[ cIdx ][ d ][ i ][ j ][ k ][ n ] is derived as follows:

cgs_coeff[ cIdx ][ d ][ i ][ j ][ k ][ n ] = (cgs_coeff_q[ cIdx ][ d ][ i ][ j ][ k ][ n ] << 7) + 
								cgs_coeff_r[ cIdx ][ d ][ i ][ j ][ k ][ n ]

cgs_coeff[ cIdx ][ d ][ i ][ j ][ k ][ n ]= (cgs_coeff_s[ cIdx ][ d ][ i ][ j ][ k ][ n ]==0) ? 
					cgs_coeff[ cIdx ][ d ][ i ][ j ][ k ][ n ] : −cgs_coeff[ cIdx ][ d ][ i ][ j ][ k ][ n ]