| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Valencia, ES, 27 March – 4 April 2014 | Document: JCTVC-Q0142_r1 |
| --- | --- |

| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 8th Meeting: Valencia, ES, 29 March – 4 April 2014 | Document: JCT3V-H0044_r1 |
| --- | --- |

| Title: | MV-HEVC/SHVC HLS: On extraction of independent non-base layer |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC and JCT-3V |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hendry Adarsh K. Ramasubramonian Ye-Kui Wang 5775 Morehouse Drive San Diego, CA 92121-1714 USA | Tel: Email: | 1-858-651-4722 fhendry@qti.qualcomm.com 1-858-658-5804 aramasub@qti.qualcomm.com 1-858-651-8345 yekuiw@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution asserts the presence of two problems related to the semantics of poc_lsb_not_present_flag[ i ] and the use of long-term reference pictures. The asserted problems can be summarized as follow:

- When poc_lsb_not_present_flag[ i ] is equal to 1 and the IDR picture in layer i is in the same access unit with a base layer picture that has slice_pic_order_cnt_lsb not equal to 0, the bitstream may become non-conformant as the derived picture order count (POC) for that IDR may be different from other pictures in the same access unit.

- When an independent non-base layer is extracted for input for HEVC decoder, if pictures in layer uses long-term reference picture (LTRP) and the original bitstream has non-cross layer aligned IDR, a problem may occur such that HEVC decoder cannot locate the correct LTRP.

The contribution also asserts that the proposed constraints to the semantics of poc_lsb_not_present_flag[ i ] and base_layer_parameter_set_compatibility_flag[ i ] would solve the problems above.

In the first revision of this contribution, to address the suggestions after review in BoG, text modification to both proposed solutions are provided. Further, it is suggested that the proposed constraint to solve problem 1 could be expressed as a note. A new sub-section 2.3 (i.e., text modification for proposal 1 and 2) was added to reflects what was agreed during the review of this contribution.

# Problem statements

## On poc_lsb_not_present_flag[ i ]

The syntax element poc_lsb_not_present_flag[ i ] allows not signalling of POC LSB information in slice segment headers of slices that belong to IDR picture of an independent non-base layer. This feature was asserted to ease the extraction of independent enhancement layers from a multi-layer bitstream such that an extracted layer can be decoded by an HEVC decoder without modifying the slice segment headers of the pictures in the layer and by only changing the nuh_layer_id in the NAL unit headers of these slice segments.

Currently, based on the syntax and semantics of poc_lsb_not_present_flag[ i ], the only explicit restriction on poc_lsb_not_present_flag[ i ] is that the value of poc_lsb_not_present_flag[ i ] can be set to 1 for a non-base layer only when the layer is an independent layer. However, such requirement alone does not demonstrate all the constraints required for the flag. For example, when IDR pictures are not cross-layer aligned and the poc_lsb_not_present_flag[ i ] for a non-base layer picture is set to 1, the bitstream may be non-conformant as the derived POC values of pictures may not be cross-layer aligned within an access unit; such a bitstream is illustrated in Figure 1. Since POC LSB of the IDR picture in the Layer 1 is not signalled, it is inferred to be equal to 0. Consequently, the POC of this Layer 1 IDR picture is derived to be 0 since the POC MSB for that picture is reset as indicated by poc_reset_idc equal to 1; the POC of the Layer 0 picture in the same AU as the Layer 1 IDR picture (in Figure 1) is derived to be 4. This would result in the POC of the pictures in the access unit to be unequal, which violates the constraint of cross-layer alignment of POC.

Figure 1: Problem of poc_lsb_not_present_flag[ i ] in non-cross layer aligned IDR scenario

## On long-term reference pictures

For extraction of independent non-base layers and enabling it to be decoded by HEVC version 1 decoders without changing slice segment headers, one should not only pay attention to whether or not POC LSB is signalled in the IDR picture in that layer (as explained in the earlier section) but also whether long-term reference pictures are used for prediction of pictures and whether pictures that reset both POC LSB and POC MSB are present in the layer. These additional two parameters are also needed to determine whether it is possible to extract the independent non-base layer without having to change slice segment headers of pictures in the layer. Note that when a particular independent layer is extracted, it is assumed that the corresponding non-VCL NAL units that are needed for decoding the independent layer are also included in the extracted bitstream.

Consider the scenario illustrated in Figure 2. Layer 1 of a two-layer bitstream is independent of the base layer Layer 0, and the IDR pictures in the bitstream are not cross-layer aligned and long-term reference pictures are used for coding of pictures of Layer 1 as illustrated. When a long-term reference picture is indicated in the RPS of a picture in Layer 1 that follows a POC resetting picture with poc_reset_idc equal to 2 (i.e. reset both POC LSB and POC MSB), and this Layer 1 is extracted, an HEVC version 1 decoder could interpret the bitstream as non-conformant if the slice segment header contents are not changed. The long-term reference picture is indicated in the RPS, say for example by using its POC LSB. An HEVC version 1 decoder would not perform POC resetting, and hence would not decrement the POC of any picture. Therefore, when Layer 1 is extracted and fed in to this decoder without changing slice segment header of pictures in the layer, the HEVC version 1 decoder may fail to find the indicated long-term reference picture since the indicated POC LSB in RPS may not match with any existing long-term reference pictures.

Figure 2: Problem of extraction of independent EL

# Proposals

To solve the above describes problems, the following modifications are proposed.

## Proposal 1

It is suggested to add a note at the end of the semantic of poc_lsb_not_present_flag[ i ] to point out that when poc_lsb_not_present_flag[ i ] is equal to 1, and an access unit contains IDR picture with nuh_layer_id equal to layer_id_in_nuh[ i ], the value of slice_pic_order_cnt_lsb have to be equal to 0 for all pictures in the access unit unless poc_reset_idc is equal to 2 for all pictures in the access unit.

## Proposal 2

The following restriction is added to the end of the semantic of poc_lsb_not_present_flag[ i ]:

–	For any picture picA that has nuh_layer_id equal to layer_id_in_nuh[ i ] and refers to the VPS, the following applies:

–	When slice_pic_order_cnt_lsb is greater than 0, poc_reset_idc shall not be equal to 2.

–	When full_poc_reset_flag is equal to 1, poc_lsb_val shall be equal to 0.

## Text modification for proposal 1 and 2

poc_lsb_not_present_flag[ i ] equal to 1 specifies that the slice_pic_order_cnt_lsb syntax element is not present in the slice headers of IDR pictures with nuh_layer_id equal to layer_id_in_nuh[ i ] in the CVS. poc_lsb_not_present_flag[ i ] equal to 0 specifies that slice_pic_order_cnt_lsb syntax element may or may not be present in the slice headers of IDR pictures with nuh_layer_id equal to layer_id_in_nuh[ i ] in the CVS. When not present, poc_lsb_not_present_flag[ i ] is inferred to be equal to 0.

It is a requirement of bitstream conformance that when poc_lsb_not_present_flag[ i ] is equal to 1, for any picture picA that has nuh_layer_id equal to layer_id_in_nuh[ i ] and refers to the VPS, the following applies:

–	When slice_pic_order_cnt_lsb is greater than 0, poc_reset_idc shall not be equal to 2.

–	When full_poc_reset_flag is equal to 1, poc_lsb_val shall be equal to 0.

NOTE xx – When poc_lsb_not_present_flag[ i ] is equal to 1, within all CVSs that refer to the VPS, when an access unit contains an IDR picture with nuh_layer_id equal to layer_id_in_nuh[ i ], the value of slice_pic_order_cnt_lsb have to be equal to 0 for all pictures in the access unit unless poc_reset_idc is equal to 2 for all pictures in the access unit.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).