| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Valencia, ES, 27 March – 4 April 2014 | Document: JCTVC-Q0143 |
| --- | --- |

| Title: | Crosscheck report of SCE1 test color gamut scalability using 8x8x8 regions and matrix mapping (JCTVC-Q0072) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Kiran Misra 5750 NW Pacific Rim Blvd Camas WA 98607 USA | Tel: Email: | +1 360 817 8451 misrak@sharplabs.com |
| Source: | SHARP Labs. Of America, Inc. |  |  |

_____________________________

# Abstract

This documents reports the crosscheck results for SCE1 test on color gamut scalability using 8x8x8 regions and matrix mapping (JCTVC-Q0072). The source code provided by the proponent was verified to be consistent with the description in JCTVC-Q0072. The rate-distortion performance was evaluated for SCE1 test conditions JCTVC-P1101 and matches the one provided in JCTVC-Q0072. Further the worst-case complexity analysis (dynamic range and multipliers-per-pixel for 2x case) results provided by proponent are confirmed.

# Introduction

JCTVC-Q0072 [1] describes a technique where the YUV region is partitioned into 8x8x8 regions and color prediction parameters are signaled for each region. These color prediction parameters are then used for gamut-conversion using the following equation:

YCbCr2020=Gain(Y,Cb,Cr).YCbCr709+Offset(Y,Cb,Cr)

Gain(Y, Cb, Cr) is a 3x3 matrix and Offset(Y, Cb, Cr) is a 1x3 matrix. More details of this technique can be found in JCTVC-Q0072.

# Experimental results:

The performance of color gamut scalability using 8x8x8 regions and matrix mapping is evaluated according to SCE1 test conditions [2] and is summarized below. The detailed results can be found in the attached excel.

|  | AI HEVC 1x 10-bit base |  |  | AI HEVC 2x 8-bit base |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A+ | -8.1% | -15.3% | -18.4% | -8.5% | -12.3% | -16.7% |
| Overall (Test vs Ref) | -8.1% | -15.3% | -18.4% | -8.5% | -12.3% | -16.7% |
| Overall (Test vs single layer) | 21.2% | 11.6% | -8.3% | 10.6% | 9.1% | -0.1% |
| Overall (Ref vs single layer) | 32.3% | 31.8% | 11.7% | 21.2% | 24.7% | 19.4% |
| EL only (Test vs Ref) | -29.4% | -34.5% | -37.7% | -16.7% | -19.9% | -24.4% |
| Overall (Test EL+BL vs single EL+BL) | -35.3% | -40.9% | -51.8% | -27.4% | -28.0% | -35.5% |
|  | RA HEVC 1x 10-bit base |  |  | RA HEVC 2x 8-bit base |  |  |
|  | Y | U | V | Y | U | V |
| Class A+ | -9.7% | -15.9% | -19.4% | -6.4% | -10.2% | -14.2% |
| Overall (Test vs Ref) | -9.7% | -15.9% | -19.4% | -6.4% | -10.2% | -14.2% |
| Overall (Test vs single layer) | 30.6% | 24.6% | 2.2% | 21.0% | 19.9% | 9.4% |
| Overall (Ref vs single layer) | 45.0% | 48.8% | 26.2% | 29.3% | 33.3% | 27.6% |
| EL only (Test vs Ref) | -29.0% | -33.5% | -36.1% | -12.0% | -15.3% | -19.3% |
| Overall (Test EL+BL vs single EL+BL) | -28.9% | -31.8% | -43.8% | -19.4% | -18.0% | -26.6% |

(continued on next page)

# Worst-case complexity analysis results:

The worst-case complexity analysis provided by the proponents was verified:

Multipliers per enhancement layer pixel:

|  |  |  |  |  | x2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | 8x8 | HEVC ingle |  | SHVC 2 layers |  |
|  |  | bi-pred | EL resolution | re-sampling | 1) re-sampling 2)CM | 1) CM 2)re-sampling |
| Hor FIR | N bits (signalled) * 7 bits (predetermined) | 37.0 | 37.0 | 46.3 | 46.3 | 46.3 |
| Vert FIR | 15 bits (signalled) * 7 bits (predetermined) | 20.0 | 20.0 | 25.0 | 25.0 | 25.0 |
| WP | 16 bits (signalled) * 8 bits (signalled) | 3.0 | 3.0 | 3.8 | 3.8 | 3.8 |
| ColorMap | N bits (signalled) * (N-P) bits (signalled) | 9.0 | 0.0 | 0.0 | 9.0 | 2.3 |
|  |  |  | 57.0 | 71.3 | 71.3 | 71.3 |
|  |  |  | 3.0 | 3.8 | 12.8 | 6.0 |
|  | Test Resampling Versus anchor resampling |  | 100% | 125% | 125% | 125% |
|  | Test (Colormap+WP) Versus anchor WP |  | 100% | 125% | 425% | 200% |
|  | Test (Colormap+WP+Resampling) Versus anchor (WP+Resampling) |  | 100% | 125% | 140% | 129% |

Dynamic range analysis:

| Color Mapping |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| # |  | BD0 |  |  | BD1 |  |  | after* |  |  | after>>clip? |  |  |
| H*W*3 |  | N |  |  | 11 |  |  | N+11 |  |  | N |  |  |
| Horizontal interpolation |  |  |  |  |  |  |  |  |  |  |  |  |  |
| # |  |  | BD00 |  |  | BD01 |  |  | after* |  |  | after >> |  |
| (H+T-1)*W*T |  |  | N |  |  | 7 |  |  | N+7 |  |  | N+7-(N-8)=15 |  |
|  | Vertical interpolation |  |  |  |  |  |  |  |  |  |  |  |  |
| # | BD10 |  |  | BD11 |  |  | after* |  |  | after >>+clip |  |  |  |
| H*W*T | 15 |  |  | 7 |  |  | (15+7)=22 |  |  | N |  |  |  |

# Conclusion

It is confirmed that all the rate-distortion data generated for the cross-check match the results provided by the proponents in JCTVC-Q0072. It is also confirmed that source code matches the description in JCTVC-Q0072. The worst-case complexity analysis (dynamic range and multipliers-per-pixel for 2x case) results were also confirmed.

# References

[1] A. Aminlou, K. Ugur, M. M. Hannuksela, “SCE1: Color gamut scalability using gain-offset models,” JCTVC-Q0072, Valencia, Spain, March-April, 2014.

[2] A. Duenas, P. Andrivon, E. Alshina, Y. Ye, K. Ugur, X. Li, “HEVC Scalable Extensions Core Experiment SCE1 description: Colour Gamut and Bit Depth Scalability,” JCTVC-P1101, San Jose, USA, Jan 2014.