| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Valencia, ES, 27 March – 4 April 2014 | Document: JCTVC-Q0144 |
| --- | --- |

| Title: | Crosscheck report of SCE1 test on asymmetric 3D-LUT with phase alignment filter disabled |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Kiran Misra 5750 NW Pacific Rim Blvd Camas WA 98607 USA | Tel: Email: | +1 360 817 8451 misrak@sharplabs.com |
| Source: | SHARP Labs. Of America, Inc. |  |  |

_____________________________

# Abstract

This documents reports the crosscheck results for SCE1 test on asymmetric three dimensional lookup table (3D-LUT) with phase alignment filter disabled (JCTVC-Q0048). The source code provided by the proponents was verified to be consistent with the description in JCTVC-Q0048. The rate-distortion performance was evaluated for SCE1 test conditions JCTVC-P1101 and matches the one provided in JCTVC-Q0048. Further the worst-case complexity analysis (dynamic range and multipliers-per-pixel for 2x case) results provided by proponents are confirmed.

# Introduction

JCTVC-Q0048 [1] describes a technique where an asymmetric three dimensional lookup table (3-D LUT) is used to map the color gamut of the reference layer picture to the color gamut of the current layer. The proposed technique takes as input the input bit depth (luma and chroma), the output bit depth (luma and chroma), a 3-D LUT. In JCTVC-Q0048 the input bit depth is set to base layer bit depth and the output bit depth is set to enhancement layer bit depth. These input parameters along with the reference layer decoded pixel value (luma and chroma) are mapped to output pixel values of a picture. The output picture is then optionally up-sampled to generate an inter-layer reference picture. More details of this technique can be found in JCTVC-Q0048.

# Experimental results:

The performance of the asymmetric 3D-LUT with phase alignment filter disabled is evaluated according to SCE1 test conditions [2] and is summarized below. The detailed results can be found in the attached excel.

|  | AI HEVC 1x 10-bit base |  |  | AI HEVC 2x 8-bit base |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A+ | -8.9% | -15.6% | -18.5% | -7.6% | -10.0% | -12.8% |
| Overall (Test vs Ref) | -8.9% | -15.6% | -18.5% | -7.6% | -10.0% | -12.8% |
| Overall (Test vs single layer) | 20.4% | 11.2% | -8.5% | 11.9% | 12.1% | 4.4% |
| Overall (Ref vs single layer) | 32.3% | 31.8% | 11.7% | 21.2% | 24.7% | 19.4% |
| EL only (Test vs Ref) | -37.6% | -41.6% | -43.8% | -15.2% | -17.1% | -20.0% |
| Overall (Test EL+BL vs single EL+BL) | -36.0% | -41.1% | -52.2% | -26.7% | -26.8% | -32.8% |
| Overall (Ref EL+BL vs single EL+BL) | -29.7% | -30.1% | -40.7% | -20.7% | -18.4% | -22.1% |
|  | RA HEVC 1x 10-bit base |  |  | RA HEVC 2x 8-bit base |  |  |
|  | Y | U | V | Y | U | V |
| Class A+ | -9.9% | -16.2% | -20.8% | -5.7% | -7.6% | -11.2% |
| Overall (Test vs Ref) | -9.9% | -16.2% | -20.8% | -5.7% | -7.6% | -11.2% |
| Overall (Test vs single layer) | 30.4% | 24.3% | 0.4% | 22.0% | 23.3% | 13.2% |
| Overall (Ref vs single layer) | 45.0% | 48.8% | 26.2% | 29.3% | 33.3% | 27.6% |
| EL only (Test vs Ref) | -29.5% | -34.0% | -37.7% | -10.9% | -12.3% | -15.9% |
| Overall (Test EL+BL vs single EL+BL) | -29.3% | -32.1% | -45.5% | -18.9% | -17.1% | -24.6% |
| Overall (Ref EL+BL vs single EL+BL) | -21.8% | -19.7% | -31.3% | -14.1% | -10.3% | -14.0% |

Note: The base layer of test and anchor do not match since the base layer test setup sends PPS’s and SPS’s more often than the anchor base layer.

(continued on next page)

# Worst-case complexity analysis results:

The worst-case complexity analysis provided by the proponents was verified:

Multipliers per enhancement layer pixel:

|  |  |  |  |  | x2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | 8x8 | HEVC ingle |  | SHVC 2 layers |  |
|  |  | bi-pred | EL resolution | re-sampling | 1) re-sampling 2)CM | 1) CM 2)re-sampling |
| Hor FIR | N bits (signalled) * 7 bits (predetermined) | 37.0 | 37.0 | 46.3 | 46.3 | 46.3 |
| Vert FIR | 15 bits (signalled) * 7 bits (predetermined) | 20.0 | 20.0 | 25.0 | 25.0 | 25.0 |
| WP | 16 bits (signalled) * 8 bits (signalled) | 3.0 | 3.0 | 3.8 | 3.8 | 3.8 |
| ColorMap | N bits (signalled) * (N-P) bits (signalled) | 4.5 | 0.0 | 0.0 | 4.5 | 1.1 |
|  |  |  | 57.0 | 71.3 | 71.3 | 71.3 |
|  |  |  | 3.0 | 3.8 | 8.3 | 4.9 |
|  | Test Resampling Versus anchor resampling |  | 100% | 125% | 125% | 125% |
|  | Test (Colormap+WP) Versus anchor WP |  | 100% | 125% | 275% | 163% |
|  | Test (Colormap+WP+Resampling) Versus anchor (WP+Resampling) |  | 100% | 125% | 133% | 127% |

Dynamic range analysis:

| Color Mapping |  |  |  |  |
| --- | --- | --- | --- | --- |
| # | BD0 | BD1 | after* | after>>clip? |
| H*W*3 | N | (N-P) | 2*N-P | N |

| Horizontal interpolation |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| # |  | BD00 |  | BD01 |  | after* |  | after >> |
| (H+T-1)*W*T |  | N |  | 7 |  | N+7 |  | N+7-(N-8)=15 |
|  | Vertical interpolation |  |  |  |  |  |  |  |
| # | BD10 |  | BD11 |  | after* |  | after >>+clip |  |
| H*W*T | 15 |  | 7 |  | (15+7)=22 |  | N |  |

# Conclusion

It is confirmed that all the rate-distortion data generated for the cross-check match the results with phase alignment filter disabled provided by the proponents in JCTVC-Q0048. It is also confirmed that source code matches the description in JCTVC-Q0048. The worst-case complexity analysis (dynamic range and multipliers-per-pixel for 2x case) results were also confirmed.

# References

[1] X. Li, J. Chen, M. Karczewicz, Y. He, Y. Ye, J. Dong, P. Bordes, P. Andrivon, E. Francois, F. Hiron, “SCE1.2: Color gamut scalability with asymmetric 3D LUT,” JCTVC-Q0048, Valencia, Spain, March-April, 2014.

[2] A. Duenas, P. Andrivon, E. Alshina, Y. Ye, K. Ugur, X. Li, “HEVC Scalable Extensions Core Experiment SCE1 description: Colour Gamut and Bit Depth Scalability,” JCTVC-P1101, San Jose, USA, Jan 2014.