| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Valencia, ES, 27 March – 4 April 2014 | Document: JCTVC-Q0145 |
| --- | --- |

| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 8th Meeting: Valencia, ES, 29 March – 4 April 2014 | Document: JCT3V-H0043 |
| --- | --- |

| Title: | MV-HEVC/SHVC HLS: On level definitions |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC and JCT-3V |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ye-Kui Wang Krishna Rapaka Jianle Chen Hendry Adarsh K. Ramasubramonian Qualcomm Incorporated 5775 Morehouse Drive San Diego, CA 92121, USA | Tel: Email: | 1-858-651-8345 yekuiw@qti.qualcomm.com krapaka@qti.qualcomm.com cjianle@qti.qualcomm.com fhendry@qti.qualcomm.com aramasub@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This document reports and discusses asserted issues associated with the current level definitions in SHVC, and proposes a different approach to define levels and decoder capabilities as well as other changes to solve the above issues. It is asserted that the proposed approach for level definitions also applies to MV-HEVC and that the level limits are specified in way that is scalable to the number of layers in bitstreams.

As a part of the proposed level definitions, a decoder capability is specified in a way where the level is associated with a scale, which is needed together with the level to express the decoder capability. Two categories of conforming decoders are specified. Category I decoders can decode some bitstreams conforming to a higher level of the decoder but with the number of layers being smaller than the scale associated with the level of the decoder. Category II decoders can only decode bitstreams conforming to the same or a lower level. Example Category I decoders are those that are implemented using one existing single-layer HEVC decoder by reusing the hardware core as is without changes or implemented without reusing the hardware cores of existing single-layer HEVC decoders. Example Category II decoders are those that are implemented using more than one existing single-layer HEVC decoder by reusing the hardware cores as they are without change.

# SHVC and MV-HEVC level definitions

The current level definitions for SHVC can be summarized as follows:

- All the general limits in subclause A.4.1 are specified to be layer specific.

- Most of the profile specific limits in subclause A.4.2 are specified to be layer specific, and 3 of them are specified to be bitstream specific.

- The sum of PicSizeInSamplesY for all the layers in a sub-bitstream of an output layer set shall be less than twice of MaxLumaPs, which is the maximum number of luma picture size in samples allowed by the level of the sub-bitstream.

The current level definitions for MV-HEVC can be summarized as follows:

- All the general limits in subclause A.4.1 apply directly with no change, but with an editor's note saying that some update to item d on the maximum DPB size is needed.

- All the profile specific limits in subclause A.4.2 apply directly with a minor editorial change to item b, which is on DpbOutputInterval[ n ].

# Problems

The current level definition of SHVC has the following issues (note that the attached Excel sheets contain an analysis that would help understand some of the following issues):

- If more than 4 SNR layers of 720p resolution (or equivalent number of luma pixels e.g. more layers with combined spatial and SNR layers) are needed, then Level 5 or above would have to be used. Consequently, the value of CtbSizeY shall be equal to 32 or 64 (per item e of the general limits). For resolution like 720p or lower, this restriction disallows the use of smaller coding tree block sizes such as 16x16 and can therefore result in sub-optimal coding efficiency.

- If an SHVC decoder is implemented in a way that consists of 4 cores of HEVC Level 3.1 decoders, and is able to decode 4 SNR layers of 720p, per the current level definition, it would have to be said to be conforming to Level 4 or above. Consequently, the decoder needs to be able to decode any Level 4 bitstreams. However, with only high-level syntax changes (i.e. no changes to the hardware cores), such a decoder would not be able to decode an SHVC Level 4 bitstream with 2 SNR layers of 1080p resolution.

- If an SHVC decoder is not implemented by reusing multiple existing HEVC decoder cores but is implemented in a way that can decode both a single-layer HEVC bitstream of 1080p and a two-layer SHVC bitstream of 720p, according to the current level definition it would labeled as a Level 3.1. However, then the expression of the other capability is missing.

- If an SHVC decoder is implemented in a way that consists of 4 cores of HEVC 3.1 decoders to be able to decode 4 SNR layers of 720p, per the current level definition, it would be said to be conforming to Level 4 or above. Consequently, each enhancement-layer picture can have more than 3 tile rows and more than 3 tile columns, e.g. 5 tile rows and 5 tile columns, each tile with width of 256 luma samples and height of 144 luma samples. However, this would go beyond the Level 3.1 limits and consequently the decoder cores for decoding the enhancement layers would have problems.

- All items in subclause A.4.1 of HEVC text are currently specified to be applied to each layer. However, some of these items are not directly applicable to each layer.

  - For item d on DPB size, the SPS syntax element is not applicable for enhancement layers. Also, the DPB in the latest SHVC text is a shared-sub-DPB design, thus item d cannot be directly applied to each layer.

  - For items h and i on CPB size, for bitstream-specific CPB operations, the parameter cannot be applied to each layer.

- Bitstream-specific restrictions on CPB size (by items h and i in subclause A.4.1 of HEVC text) are needed. However, the items h and i in subclause A.4.1 of HEVC text cannot be directly applied on bitstream level, because if directly applied, the same CPB size limit for single-layer bitstreams would also be the limit for multi-layer bitstreams. This is not scalable to the number of layers and would only allow for low picture quality when there are many layers.

- The restrictions by items b, c, d, g, h, i, and j in subclause A.4.2 of HEVC text are currently specified to be layer-specific only. However, bitstream-specific restriction by these items should be specified, regardless of whether their layer-specific counterparts are specified.

# Proposal

This contribution proposes a different approach to define levels and decoder capabilities as well as other changes to solve the above issues. Though in the above the problems are described based on the current SHVC level definitions, some of the problems also apply to MV-HEVC, and the proposed approach and most other changes also apply to MV-HEVC.

The proposal is summarized as follows:

- A decoder capability is specified in a way where the level is associated with a scale, which is needed together with the level to express the decoder capability.

- Two categories of conforming decoders are specified. Category I decoders can decode some bitstreams conforming to a higher level but with the number of layers being smaller than the scale associated with the level of the decoder, while Category II decoders can only decode bitstreams conforming to the same or a lower level (provided that the profile and tier of the bitstream are the same as that of the decoder, respectively). Example Category I decoders are those that are implemented using one existing single-layer HEVC decoder by reusing the hardware core as is without changes or implemented without reusing the hardware cores of existing single-layer HEVC decoders. Example Category II decoders are those that are implemented using more than one existing single-layer HEVC decoder by reusing the hardware cores as they are without change.

- Two DPB size-related restrictions are specified, one for each layer, and one for each sub-DPB, specified using the DPB related parameters in the VPS.

- Bitstream-specific restrictions on CPB size are specified in a way that is scalable to the number of layers and allow for high picture quality when there are many layers.

- Bitstream-specific restrictions corresponding to items b, c, d, g, h, i, and j in subclause A.4.2 of HEVC text are specified in a way that is that is scalable to the number of layers.

The text changes in relative to the latest SHVC and MV-HEVC draft texts are provided in the attachments. The attached slides show some example scenarios for illustrating of some of the above constraints.

# Discussions

For both general and profile-specific constraints, except for those are clearly layer-specific, such as luma picture size, in the proposal they are all specified to be bitstream specific. Should some of them be specified to be layer-specific in addition?

The value of scaleFactor is equal to the number of layers in the sub-bitstream for layers with the same resolution. When spatial scalability or picture rate scalability across layers exists, using a smaller value of scaleFactor could be more optimal. This aspect should be further studied.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).