| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Valencia, ES, 27 March – 4 April 2014 | Document: JCTVC-Q0146 |
| --- | --- |

| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 8th Meeting: Valencia, ES, 29 March – 4 April 2014 | Document: JCT3V-H0042 |
| --- | --- |

| Title: | MV-HEVC/SHVC HLS: On picture order count and related |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC and JCT-3V |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Adarsh K. Ramasubramonian Hendry Ye-Kui Wang 5775 Morehouse Drive San Diego, CA 92121, USA | Tel: Email: | 1-858-658-5804 aramasub@qti.qualcomm.com 1-858-651-4722 fhendry@qti.qualcomm.com 1-858-651-8345 yekuiw@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This document proposes several changes to the semantics of syntax elements related to POC, and also proposes a constraint to disallow an enhancement layer RASL picture of a CRA picture to refer to pictures that precede, in decoding order, a POC resetting picture that precedes the associated CRA picture in decoding order and that is in the same layer as the CRA picture.

# Proposals

## Inferred value of poc_msb_val_present_flag

The semantics of vps_poc_lsb_aligned_flag and poc_msb_val_present_flag are given as follows.

vps_poc_lsb_aligned_flag equal to 0 specifies that the value of slice_pic_order_cnt_lsb may or may not be the same in different pictures of an access unit. vps_poc_lsb_aligned_flag equal to 1 specifies that the value of slice_pic_order_cnt_lsb is the same in all pictures of an access unit. Additionally, the value of vps_poc_lsb_aligned_flag affects the decoding process for picture order count in subclause F.8.3.1. When not present, vps_poc_lsb_aligned_flag is inferred to be equal to 0.

poc_msb_val_present_flag equal to 1 specifies that poc_msb_val is present. When poc_msb_val_present_flag is equal to 0 and PocMsbValRequiredFlag is equal to 0, poc_msb_val is not present. When not present, the value of poc_msb_val_present_flag is inferred as follows:

–	If PocMsbValRequiredFlag is equal to 1, the value of poc_msb_val_present_flag is inferred to be equal to 1.

–	Otherwise, the value of poc_msb_val_present_flag is inferred to be equal to 0.

By inferring vps_poc_lsb_aligned_flag to be equal to 0 when it is not present, a problem may occur when a legacy HEVC bitstream (with only one layer) is decoded by an SHVC decoder. In the HEVC bitstream, the VPS extension would be absent and hence the value of vps_poc_lsb_aligned_flag is inferred to be equal to 0. When the value of vps_poc_lsb_aligned_flag is equal to 0, and the current picture is a CRA or a BLA picture, the value of PocMsbValRequiredFlag is derived to be equal to 1 (as given in Eq. F-28). The problem is that, since the slice header extension is absent in this bitstream, according to the above semantics, the value of poc_msb_val_present_flag is inferred to be equal to 1 while the poc_msb_val is not signalled in this bitstream.

Even if the inference rule of vps_poc_lsb_aligned_flag is changed such that when not present it is inferred to be equal to 1, the problem still remains as the value of poc_msb_val_present_flag is still derived to be equal to 1.

### Solution

The semantics of the poc_msb_val_present_flag is proposed to be altered such that a value of 0 is inferred for the syntax element when the slice_segment_header_extension_length is equal to zero.

poc_msb_val_present_flag equal to 1 specifies that poc_msb_val is present. When poc_msb_val_present_flag is equal to 0 and PocMsbValRequiredFlag is equal to 0, poc_msb_val is not present. When not present, the value of poc_msb_val_present_flag is inferred as follows:

–	If slice_segment_header_extension_length is equal to 0, the value of poc_msb_val_present_flag is inferred to be equal to 0.

–	Otherwise, if PocMsbValRequiredFlag is equal to 1, the value of poc_msb_val_present_flag is inferred to be equal to 1.

–	Otherwise, the value of poc_msb_val_present_flag is inferred to be equal to 0.

## Reference pictures of RASL pictures across POC resetting periods

Consider a bitstream btsA where IRAP pictures are not cross-layer aligned. Figure 1 shows an example of such a bitstream, and the POC resetting pictures are marked as A, B and C in the figure. Assume that RASL picture X uses the picture Y as a long-term reference picture, and no other picture is used for reference for picture X. The RPS subset RefPicSetLtFoll of the CRA picture R will include picture Y because RASL picture X uses it for reference; pictures A, B, and C will also have picture Y in their respective RPS.

Further consider the case where a middle-box subjects the bitstream btsA to a down-switch followed by an up-switch, e.g. to adapt to a lower available bandwidth. The resultant bitstream is shown in Figure 2. The long-term picture Y is available in the sub-DPB corresponding to the enhancement layer, but due to the down-switch and up-switch, the POC of the long-term reference picture is not decremented; the amount by which the POC is to be decremented in the EL is "lost" because the POC resetting pictures A, B and C were removed during the down-switch and up-switch. For instance, assume that the POC LSB length is 8 bits and let CRA picture R include picture Y in its RPS using the POC LSB of Y (equal to 100) in the original bitstream (Figure 1). That POC LSB is no longer valid after the up-switch and down-switch.

In the given example, when the RASL picture X only uses picture Y for reference, it would be expected that there is no need to mark the CRA picture R as a BLA picture or change the value of its flag HandleCraAsBlaFlag equal to 1 because the RASL picture is correctly decodable. If the NAL unit type of picture R is not changed to one of the NAL unit types of a BLA picture, or the HandleCraAsBlaFlag corresponding to picture R is not set equal to 1, when the decoder tries to decode RASL picture X, the constraint that no picture shall have an entry equal to "no reference picture" in RefPicSetStCurrBefore, RefPicSetStCurrAfter, or RefPicSetLtCurr would be violated, and picture X cannot be correctly decoded even both the picture itself and its reference picture is received. In theory, when the number of access units between two switching events is not high, it is also possible that the reference picture is a short-term reference picture.

Figure 1: Original structure before down-switching and up-switching

Figure 2: Problem that occurs after down-switching and up-switching.

### Solution

In order to avoid the problem described above, it is proposed that RASL pictures of a CRA picture be mandated not to refer to pictures that precede a POC reset picture in the same layer that precedes, in decoding order, the associated CRA picture of the RASL picture. This can be implemented by including a constraint on the RPS for CRA pictures, as follows.

It is a requirement of bitstream conformance that any picture in the RPS of a CRA picture that has nuh_layer_id greater than 0 shall not precede, in decoding order, any POC resetting picture that precedes, in decoding order, the CRA picture and that has the same nuh_layer_id as the CRA picture.

## Semantics of slice_segment_header_extension_data_bit

Currently, the semantics of the syntax element slice_segment_header_extension_data_bit in slice segment header extension are missing. The syntax element name has been changed from _byte to _bit. The following semantics are proposed as an editorial fix:

slice_segment_header_extension_data_bit may have any value. Decoders shall ignore the value of slice_segment_header_extension_data_bit. Its value does not affect decoder conformance to profiles specified in this version of this Specification.

## Semantics of poc_reset_info_present_flag

When the syntax element poc_reset_info_present_flag is not present, there is no value that it takes. It is proposed to infer the value to be equal to 0.

### Solution

The highlighted text below show the changes proposed.

poc_reset_info_present_flag equal to 0 specifies that the syntax element poc_reset_idc is not present in the slice segment headers of the slices referring to the PPS. poc_reset_info_present_flag equal to 1 specifies that the syntax element poc_reset_idc is present in the slice segment headers of the slices referring to the PPS. When not present, the value of poc_reset_info_present_flag is inferred to be equal to 0.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).