| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Valencia, ES, 27 March – 4 April 2014 | Document: JCTVC-Q0147r2 |
| --- | --- |

| Title: | AhG8: On fast inter search method for screen content coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Krishna Rapaka Joel Sole Li Zhang Marta Karczewicz 5775 Morehouse Drive San Diego, CA 92121-1714 USA | Tel: Email: | +1-858-658-2042 krapaka@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution proposes a fast inter search method for screen content coding to better capture the spatial and temporal characteristics exhibited by screen content sequences like sharp edges, high spatial frequencies and non-monotonic error surface. It is asserted that the current HM’s inter prediction search is poorly suited for predicting such content and that substantial improvement may be made purely by changing the search algorithm. The following three modifications to inter search are proposed a) Multi stage approximated error cost computation with early exit at each stage b) Modified initial search method with a fixed diamond search traversing uniformly across the initial search space c) Modified early skip detection based on residual signals merge MV only. The proposed modifications are encoder only and do not affect decoder operation. For the lossy coding, the proposed inter search under SCC CfP test conditions show BD-rate gain of 9.8% and 12.2% for 1080p text and graphic sequences under random access and low delay configurations respectively.

In this revised version, the test results under AHG8 common test conditions are added to the document and a software patch is provided over HM-RExt 6.0 of the proposed inter search method.

# Introduction

The current HM’s fast inter search method performs well for natural camera-captured sequences which are smoother due to the optical system that attenuates higher frequencies and noisier due to the noise introduced by lens distortion. However, it is asserted that this inter search does not perform well for the non-natural content (screen content, compound images etc.). These screen content sequences often created by capturing computer desktop screens have sharp edges, no noise, different spectral activity & spatial frequencies (e.g text and graphics) and a fast motion estimation algorithm needs carefully account these characteristics for better efficiency and lower computational complexity. Also, it is obvious that full search is extremely expensive and may not be feasible in most applications.

# Proposal

The current HM’s fast inter search is modified to better capture the spatial and temporal characteristics in screen-content sequences. These sequences exhibit non-monotonic error surface and high spatial frequency characteristics contrary to the assumptions of inter search algorithms designed for natural camera-captured sequences. The proposed modifications are

- Multi stage approximated error cost computation with early exit at each stage.

- Modified initial search method with a fixed diamond search traversing uniformly across the initial search space.

- Modified early skip detection based on residual signals merge MV only.

## Multi stage approximated error cost computation with early exit at each stage

Due to sharp edges and high spatial frequencies exhibited by screen content sequences, most of the error might be concentrated around on few samples. Most of the times it might be possible to rule out a candidate based on approximated error computed from partial samples. In the proposed method, the total cost of block is approximated based on the partial samples of the block and this approximated cost is used to decide the best candidate. This total estimated cost is computed in different intermediate stages with each stage refining the amount of partial samples used. The intermediate cost at each stage is used to decide the need of further refinement stages thereby providing an early exit mechanism. The motion vector bit cost is added to error cost to obtain the total cost. It may be noted that this is possible as the approximated total cost represent the total cost of the block.

For a block size of BxB, the approximated total error cost based of ‘l’ pixels for the block can be represented by below equation,

where i(m), j(m)|m =0,1,…B2 -1 is the index set of all the pixels in the block.

The figure below illustrates an example implementation where the partial error is first computed based on partial samples and approximated to complete error.

Fig 1: An example implementation partial error approximation with early exit

## Modified initial search

Due to non-monotonic error pattern exhibited by non-natural sequences, an exponentially expanding diamond that is used in the current HM’s fast inter search may not be efficient as it might either miss one of the best candidates or falsely infer the best candidate to be within the best predictor. In the proposed method, a uniform moving diamond pattern search is proposed around a smaller initial search area to better capture the local minima. Uniformly moving fixed diamond along horizontal and vertical around the initial search area is proposed to better capture the local minima.

In the figure below, a search space of 64x64, an initial search space of 16x16 and a fixed diamond with step size of 4 that uniformly traverses initial search space is illustrated.

Fig 2:  Modified initial search using uniformly moving diamond

## Modified early skip detection

In the current HM’s fast inter search the early skip detection is based on either residual signal of merge MV or absolute sum of horizontal and vertical MV are zero. The decision based on second decision is too aggressive and not efficient for animation images that are closer to natural content. Early skip detection is modified to be performed based on the residual signal of the Merge MV only. When the residual signal of Merge MV is already zero, it is highly likely that the block may not have high frequencies and best suitable to be skipped.

# Results

In this section, the experimental results of the proposed modified inter search method are presented. The method is tested on the top of HM13.0_Rext6.0. The test conditions of the CfP on SCC [1] are used.

The results of table 1 are updated with simulations over RExt 6.0. For the lossy coding, Error: Reference source not found reports the coding gain of modified inter search under SCC CfP test conditions. For random access lossy configuration, the results show BD-rate gain of89.8%, 4.3% for 1080p text and graphic sequences and mixed content respectively. For low-delay lossy configuration results show BD-rate gain of 12.9%,56% for 1080p text and graphic sequences and mixed content respectively.

Table 1 Coding gains of modified inter search compared to anchor (SCC CfP lossy test conditions)

|  |  | Random Access |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y |  | U |  | V |  |
| RGB, text & graphics with motion, 1080p | -9.8% |  | -10.0% |  | -9.7% |  |
| RGB, text & graphics with motion,720p | -4.2% |  | -4.1% |  | -4.2% |  |
| RGB, mixed content, 1440p | -1.6% |  | -1.6% |  | -1.6% |  |
| RGB, mixed content, 1080p | -4.3% |  | -4.5% |  | -4.5% |  |
| RGB, Animation, 720p | 0.0% |  | 0.0% |  | 0.0% |  |
| YUV, text & graphics with motion, 1080p | -10.5% |  | -9.7% |  | -9.6% |  |
| YUV, text & graphics with motion,720p | -4.2% |  | -4.2% |  | -4.3% |  |
| YUV, mixed content, 1440p | -1.4% |  | -1.8% |  | -1.7% |  |
| YUV, mixed content, 1080p | -4.9% |  | -5.1% |  | -5.1% |  |
| YUV, Animation, 720p | 0.0% |  | -0.1% |  | -0.4% |  |
| Enc Time[%] | 119% |  |  |  |  |  |
| Dec Time[%] | 100% |  |  |  |  |  |
|  | Low delay B |  |  |  |  |  |
|  | Y |  | U |  | V |  |
| RGB, text & graphics with motion, 1080p | -12.2% |  | -12.5% |  | -12.2% |  |
| RGB, text & graphics with motion,720p | -8.7% |  | -8.5% |  | -8.6% |  |
| RGB, mixed content, 1440p | -1.9% |  | -1.8% |  | -1.8% |  |
| RGB, mixed content, 1080p | -6.0% |  | -6.2% |  | -6.0% |  |
| RGB, Animation, 720p | 0.0% |  | 0.0% |  | -0.1% |  |
| YUV, text & graphics with motion, 1080p | -12.5% |  | -11.8% |  | -11.7% |  |
| YUV, text & graphics with motion,720p | -8.7% |  | -9.4% |  | -9.5% |  |
| YUV, mixed content, 1440p | -1.7% |  | -2.2% |  | -2.2% |  |
| YUV, mixed content, 1080p | -6.5% |  | -7.4% |  | -7.3% |  |
| YUV, Animation, 720p | 0.2% |  | -0.4% |  | -0.2% |  |
| Enc Time[%] | 123% |  |  |  |  |  |
| Dec Time[%] | 100% |  |  |  |  |  |

For the lossy coding, Error: Reference source not found2 reports the coding gain of modified inter search under AHG8 common test conditions.

Table 2 Coding gains of modified inter search compared to anchor (AHG8 lossy test conditions)

|  | Random Access Main-tier |  |  | Random Access High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class F | -1.5% | -2.1% | -2.2% | -1.4% | -1.9% | -1.9% |
| Class B | 0.1% | -0.4% | -0.5% | 0.0% | -0.3% | -0.4% |
| RGB 4:4:4 SC | -4.3% | -4.3% | -4.3% | -4.1% | -4.1% | -4.1% |
| RGB 4:4:4 Animation | -0.3% | -0.4% | -0.3% | -0.3% | -0.3% | -0.3% |
| YCbCr 4:4:4 SC | -4.4% | -4.3% | -4.3% | -4.2% | -4.3% | -4.3% |
| YCbCr 4:4:4 Animation | -0.2% | -0.7% | -0.7% | -0.2% | -0.5% | -0.6% |
| RangeExt | 0.0% | -0.7% | -0.8% | 0.1% | -0.3% | -0.5% |
| RGB 4:4:4 SC (Optional) | -5.4% | -5.4% | -5.4% | -5.6% | -5.6% | -5.6% |
| YCbCr 4:4:4 SC (Optional) | -4.8% | -4.4% | -4.3% | -5.2% | -4.9% | -4.8% |
| Enc Time[%] | 111% |  |  | 117% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |
|  | Low delay B Main-tier |  |  | Low delay B High-tier |  |  |
|  | Y | U | V | Y | U | V |
| Class F | -2.8% | -3.2% | -3.2% | -2.5% | -2.8% | -2.8% |
| Class B | 0.2% | -0.5% | -0.5% | 0.1% | -0.4% | -0.4% |
| RGB 4:4:4 SC | -7.5% | -7.4% | -7.4% | -7.2% | -7.3% | -7.2% |
| RGB 4:4:4 Animation | -0.2% | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% |
| YCbCr 4:4:4 SC | -7.9% | -8.2% | -8.3% | -7.8% | -8.1% | -8.1% |
| YCbCr 4:4:4 Animation | 0.2% | -0.2% | -0.3% | 0.1% | -0.3% | -0.3% |
| RangeExt | 0.1% | -0.3% | -0.4% | 0.1% | -0.1% | -0.2% |
| RGB 4:4:4 SC (Optional) | -12.2% | -12.2% | -12.2% | -12.7% | -12.6% | -12.7% |
| YCbCr 4:4:4 SC (Optional) | -9.7% | -9.6% | -9.5% | -9.9% | -10.0% | -9.8% |
| Enc Time[%] | 124% |  |  | 130% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

# References

- ISO/IEC JTC1/SC29/WG11 (MPEG) and ITU-T SG16 WP3 (VCEG), “Joint Call for Proposals for coding of screen content,” Jan. 2014

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).