| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Valencia, ES, 27 March – 4 April 2014 | Document: JCTVC-Q0148 |
| --- | --- |

| Title: | Quantization rounding for RDPCM |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Feng Zou, Rajan Joshi Joel Sole Chao Pang Marta Karczewicz | Tel: Email: | +1-858-845-1115 fzou@qti.qualcomm.com |
| Source: | Qualcomm |  |  |

_____________________________

# Abstract

In this proposal, a uniform quantization is proposed for residual differential pulse code modulation (RDPCM) blocks. It is asserted that this is an encoder-only change. The proposed scheme is implemented on top of HM-13.0+RExt-6.0. It is reported that the proposed change results in 0.2%, 0.4%, and 0.8% BD-rate savings on average for AI, RA and LB main tiers respectively.

# Introduction and existing problems

In the 14th JCTVC meeting, several coding tools were adopted together for Range Extension of HEVC. In particular, the tool residue differential pulse-code modulation (RDPCM) [1, 2, 3] work under the scenario when transforms are bypassed, such as for lossless transform-quantization bypass blocks or for lossy transform-skip (TS) blocks. In lossy coding of HM-13.0+RExt-6.0, for RDPCM coded blocks, spatial residue signals are directly quantized, dequantized and predicted pixel by pixel, and no transform is applied. Note that during the quantization, a traditional dead-zone plus uniform threshold scalar quantization is used as shown in Figure 1, where Q is the quantization step size and α is the rounding offsets. In HM-13.0+RExt-6.0, α is 1/3 for intra blocks and 1/6 for inter blocks.

Figure 1. Traditional dead-zone plus uniform threshold quantization

The traditional dead-zone plus uniform threshold scalar quantization is designed for transform coefficient signals, which are in the frequency domain. However, for RDPCM, the quantization is applied to the spatial signals, which have quite different statistical characteristics. Therefore, the traditional dead-zone rounding may not be efficient in coding RDPCM signals

# Proposed uniform quantization

Instead of using dead-zone plus uniform threshold quantization, it is proposed to use the uniform quantization for RDPCM mode with α=Q/2, where Q is the quantization step size and α is the rounding offset.

# Simulation results

The proposed scheme is implemented on top of HM-13.0+RExt-6.0 using the common test condition defined in [4].

# Conclusion

The proposed uniform quantization provides 0.2%, 0.4%, 0.8% BD-rate savings on average for AI, RA and LB main tiers respectively without additional complexity increase.

# Patent rights declaration(s)

Qualcomm may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

[1] “RCE2: Experimental results on Test 3 and Test 4, ” JCTVC-M0056, Apr. 2013

[2] “RCE2 subtest C.2: Extension of residual DPCM to lossy coding,” JCTVC-N0052, Jul. 2013

[3] “RCE2: Experimental results for Test C.1,” JCTVC-N0074, Jul. 2013

[4] “Common test conditions and software reference configurations for HEVC range extensions”, JCTVC-P1006, Jan. 2014