| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Valencia, ES, 27 March – 4 April 2014 | Document: JCTVC-Q0149 |
| --- | --- |

| Title: | Pixel-based 1D Dictionary Coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Feng Zou, Ying Chen Chao Pang Joel Sole Marta Karczewicz | Tel: Email: | +1-858-845-1115 fzou@qti.qualcomm.com |
| Source: | Qualcomm |  |  |

_____________________________

# Abstract

In this proposal, a pixel based 1D dictionary method is proposed to improve the video coding efficiency. The matching in 1D dictionary is applied in unit of pixels. And the proposed 1D dictionary is implemented on LCU level and associated syntaxes are modified to further improve the coding efficiency of 1D dictionary. Simulation results show that the proposed method achieves 17.0%, 10.1% and 6.2% BD-rate savings for 1080p RGB sequences for AI RA and LB respectively.

# Introduction and existing problems

Block matching used in traditional hybrid coding, like Intra BC, is not always efficient to code similar or identical pattern within a picture. Traditional pattern-matching based algorithms use only 1-D pattern or 2-D pattern of a few fixed sizes. To improve the coding efficiency of such repeated pattern, 1D dictionary coding is by Tongji University in [1] and [2]. 1D dictionary coding is flexible in the sense that it allows string matching of variable sizes. For the matched strings, the relative distance from reference sample position to current sample position and matching length will be coded.

- In the prior art, the processing order of 1D in each CU is vertical scan. However, there are more cases that there is higher horizontal similarity or horizontal repeated patents in the screen content.

- In the prior art, the 1D string matching is applied on pixel samples. In this case, the matching string may include different pixels, a couple of which might not contain the three components in the matching string. This would result in cross pixel sample fetching for comparison (at the encoder) and compensation (at the decoder), which causes additional spectacular calculation and increased memory access.

- In the prior art, the unmatched pixel sample are predicted using the previous coded pixel sample of the same channel, and the prediction error is entropy coded. This requires accessing the previous coded pixels and prediction error calculation with prediction error sign and absolute value coded. For the matched string, the syntax matching string offset is coded using exponential-golomb like code word, which has redundancy in the prefix design given the current pixel location within the picture. The syntax element of matching string run is coded using region based fixed length code word, and the run is limited to 272. It is not efficient when the matching length is over 272.

# Proposed pixel based 1D dictionary

1D dictionary algorithm is proposed in this proposal providing an arbitrary shape matching scheme for screen content coding. In specific, a Coding Unit (CU) is split into multiple pixel strings, where each pixel contains three color component samples(Y, U or V).

For each CU, the pixels are raster scanned into 1D pixel array. When the string in the current CU has a matching string in the previously coded reconstructed area, as shown in Figure 1, two syntaxes are entropy coded, one of which is called matching string offset herein, denoting the relative distance between the current string and the reference string, and one of which is called matching string run herein, denotes the matching length.

Figure 1. 1D matching example

Note that in our proposed 1D dictionary scheme, the matching criterion is applied in unit of pixels instead of samples. In other words, if the three color components (samples) of the reference pixel are equal to the three samples of the current pixel respectively, it is asserted that the current pixel has a matching pixel; otherwise, it is asserted that the reference pixel is not matched with the current pixel. When the pixel in the current CU does not have the matching pixel in the previously coded reconstructed area, the original pixel sample value is directly coded. A typical pixel based matching example is shown in Figure 2.

Figure 2. Example of pixel matching in 1D dictionary

To enable fast matching at the encoder, Hash value is calculated in a way of a simple concatenation of the most significant bits (MSBs) from three samples of a pixel. The number of the bits (nBitHash) for the Hash value is configurable at the encoder. And the number of the MSBs of each component i is derived as follows: (nBitHash+2-i)/3. In this proposal, we use 7 MSBs from each component, and thus a 21bit hash value is used for each pixel. Note that the Hash value is calculated using the reconstructed pixels.

A linked list is established to store the reference pixel locations with the same hash value. Given the current pixel Hash value, the reference pixel with the same Hash value can be quickly found using the linked list implemented in the form of look-up table. After a match is identified between a current pixel and a reference pixel, the string run continues till the match cannot be identified to get a consecutive number of matched pixels In case that there are multiple reference pixels with the same hash value, the one that has the longest run is chosen at the encoder

1D dictionary algorithm is designed as an alternative coding mode competing with traditional HEVC coding modes. And RD criterion is used to select the best mode in terms of minimum RD cost for each CU. And in this proposal, 1D dictionary is applied on the largest CU (CTU).

To code matching string offset, 8 recently coded offsets are used to predict the current matching string offset. The offset predictor set is reset to 0 at the beginning of each slice or if the CU is coded using traditional HEVC mode. If the current matching string offset is equal to one the 8 predictors, a predictor flag being 1 and predictor index of 3 bit are coded in the bitstream; otherwise, a predictor flag being 0 and the current matching string offset are coded in the bitstream, where an exponential Golomb like codeword is used to code the current offset. There is no limitation on the range of offset, and in other words, the reference pixel can be anywhere within the casual area.

To code matching string run, the concatenation of Golomb rice and exponential Golomb is used with the rice parameter being 4. The run is in the range of 1 to 64x64.

# Simulation results

The proposed scheme is implemented on top of HM-13.0+RExt-6.0 using the common test condition defined in [3] except that the test sequences are screen content sequences defined in [4].

Table 1: Lossy test results                                                 Table 2: Lossless test results

# Patent rights declaration(s)

Qualcomm may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

[1]T. Lin, S. Wang, P. Zhang, K. Zhou, “AHG7: Full-chroma (YUV444) dictionary+hybrid dual-coder extension of HEVC”, JCT-VC Document, JCTVC-K0133, Shanghai, China, October 2012.

[2]T. Lin, K. Zhou, X. Chen, and S. Wang, “Arbitrary Shape Matching for Screen Content Coding”, Picture Coding Symposium (PCS), San Jose, 2013.

[3] “Common test conditions and software reference configurations for HEVC range extensions”, JCTVC-P1006, Jan. 2014

[4] “JCT-VC AHG report: Screen content coding (AHG8)”, JCTVC-P0008, Jan 2014

# Syntax and semantics change

## Semantics

### Sequence parameter set semantics

dictionary_1d_enable_flag equal to 1 specifies that dictionary coding may be invoked for coding units of the coded video sequence. dictionary_1d_enable_flag equal to 0 specifies that dictionary coding is not invoked for any coding units of the coded video sequence. When not present, the value of dictionary_1d_enable_flag is inferred to be equal to 0.

Alternatively, such a flag is put in picture parameter set.

Alternatively, an additional flag controlling 1D dictionary coding is put in picture parameter set when dictionary_1d_enable_flag is equal to 1.

Alternatively, a slice level flag may be introduced to disable or enable 1D dictionary coding.

Alternatively, dictionary_1d_enable_flag is equal to 1 only when lossless coding is enforced for the coded video sequence. In one example, when transquant_bypass_enabled_flag is equal to 0 for any coding unit, dictionary_1d_enable_flag shall be set equal to 0.

### Coding unit semantics

dictionary_coded_flag equal to 1 specifies that dictionary coding is used for the coding unit and all the any other syntax element for the current coding is not present. dictionary_coded_flag equal to 0 specifies that dictionary coding is not used for the coding unit. When not present, the value of dictionary_coded_flag is inferred to be equal to 0.

In one alternative, dictionary_coded_flag is only equal to 1 when the coding unit size is the same as the coding tree block size. That is: dictionary_coded_flag shall be equal to 0 when CtbLog2SizeY is larger than log2CbSize.

In another alternative, however, dictionary_coded_flag is present only if coding unit size is the same as the coding tree block size, as illustrated below.

| coding_unit( x0, y0, log2CbSize ) { | Descriptor |
| --- | --- |
| if ( dictionary_1d_enable_flag && log2CbSize = = CtbLog2SizeY) |  |
| dictionary_coded_flag | av(v) |
| if( dictionary_coded_flag ) |  |
| dictionary_syntax_table( ) |  |
| else { |  |
| … |  |
| } |  |
| } |  |

### 1D dictionary block table semantics

matching_string_flag equal to 1 indicates that the current pixel starts a matching string. matching_string_flag equal to 0 indicates the current does not start a matching string and its values are explicitly present.

matching_string_offset_use_recent_8_flag equal to 1 indicates the current matching string offset is equal to one of the eight previously decoded matching string offsets and the string offset is specified by matching_string_offset_recent_8_idx. matching_string_offset_use_recent_8_flag equal to 0 indicates the current matching string offset is explicitly present by syntax matching_string_offset_minus1.

matching_string_offset_recent_8_idx specifies the index to the eight previously coded matching string offsets. When not present, the value of matching_string_offset_recent_8_idx is inferred to be equal to 0.

matching_string_offset_minus1 plus 1 specifies the matching string offset between the current string and the reference string. When not present, the value of matching_string_offset_minus1is inferred to be equal to 0.

matching_string_length_minus1 plus 1 specifies the matching string run (the number of pixels that the current string match the reference string). When not present, the value of matching_string_length_minus1is inferred to be equal to 0.

unmatchable_sample_value_component0 is specifies the value of the 0-th sample of the current pixel.

unmatchable_sample_value_component1 is equal to pixel value of the 1-th sample the current pixel.

unmatchable_sample_value_component2 is equal to pixel value of 2-th sample of the current pixel.

## 1D dictionary block table syntax

| dictionary_syntax_table( ){ |  |
| --- | --- |
| for( decPixelCnt=0; decPixelCnt < (1<<(2* log2CbSize); ) { |  |
| matching_string_flag | ae(v) |
| if(matching_string_flag = = 1) { |  |
| matching_string_offset_use_recent_8_flag | ae(v) |
| if(matching_string_distance_use_recent_8_flag) |  |
| matching_string_offset_recent_8_idx | ae(v) |
| else |  |
| matching_string_offset_minus1 | ae(v) |
| matching_string_length_minus1 | ae(v) |
| decPixelCnt += (matching_string_length_minus1 + 1) |  |
| } |  |
| else { |  |
| unmatchable_sample_value_component0 | ae(v) |
| unmatchable_sample_value_component1 | ae(v) |
| unmatchable_sample_value_component2 |  |
| decPixelCnt ++ |  |
| } |  |
| } |  |
| } |  |